# 3GPP TSG-RAN WG4 Meeting # 107	                                  R4-2312473

# Incheon, KR, May 22 – May 26, 2023

Source:	ZTE Corporation

Title:	TP for TR38.718-03-01_3DL_2UL CA_n8A-n41A-n79A

Agenda Item:	7.11.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/2UL CA_n8A-n41A-n79A in TR38.718-03-01. Note that the 3DL/1UL CA_n8A-n41A-n79A have already been defined in the spec.

- Reference

- [1] TR38.718-03-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for3 bands DL with x bands UL (x=1,2),v0.6.0

- Text Proposal

----- Start of TP -----

  - 5.x			n8A-n41A-n79A

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| n8A-n41A-n79A | n8, n41, n79 |

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY+nZ

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n41A-n79A | CA_n8A-n41A CA_n8A-n79A CA_n41A-n79A | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

      - 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n8-n41-n79, the TIB,c and RIB,c values have already been defined in the spec.

    - 5.x.2	Specific for 2 bands UL CA

      - 5.x.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n8A-n41A, CA_n8A-n79A and CA_n41A-n79A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

IMD3 issue caused by n8+n41 fall into the its own band n79 Rx;

IMD3 and 5 issue caused by n41+n79 fall into the its own band n8 Rx;

IMD3 issue caused by n8+n79 fall into the its own band n41 Rx.

      - 5.x.2.2	REFSENS requirements

Based on co-existence studies additional MSD are needed to be defined, shown in table 5.x.2.2-1. where some MSD requirements are reused form DC_8_n41-n79.

Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n8-n41-n79 | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | N/A |
|  | n79 | 4470 | 40 | 216 | 4470 | 16.3 | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | 15.5 | IMD3 |
|  | n79 | 4470 | 40 | 216 | 4470 | N/A | N/A |
|  | n8 | 895 | 5 | 25 | 940 | 11.8 | IMD31 |
|  | n41 | 2680 | 10 | 50 | 2680 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

----- End of TP -----