# 3GPP TSG-RAN WG4 Meeting # 108	                              R4-2312474

# Toulouse, France, August 21 – August 25, 2023

Source:	ZTE Corporation

Title:	TP for TR38.718-03-01_3DL_2UL CA_n28A-n39A-n79A

Agenda Item:	7.11.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/2UL CA_n8A-n39A-n41A in TR38.718-03-01. Note that the 3DL/1UL CA_n8A-n39A-n41A have already been defined in the spec.

- Reference

- [1] TR38.718-03-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for3 bands DL with x bands UL (x=1,2),v0.6.0

- Text Proposal

----- Start of TP -----

  - 5.x			n28A-n39A-n79A

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| n28A-n39A-n79A | n28, n39, n79 |

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY+nZ

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n39A-n79A | CA_n28A-n39A CA_n28A-n79A CA_n39A-n79A | n28 | 5, 10, 15, 20, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

      - 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n8-n39-n79, the TIB,c and RIB,c values have already been defined in the spec.

    - 5.x.2	Specific for 2 bands UL CA

      - 5.x.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n8A-n39A, CA_n8A-n41A and CA_n39A-n41A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

IMD3, IMD4 and IMD5 issuess caused by n28-n39 fall into its own band n79 Rx;

IMD5 issue caused by n28+n79 fall into the its own band n39 Rx;

IMD3 and IMD4 issue caused by n39+n79 fall into the its own band n28 Rx;

      - 5.x.2.2	REFSENS requirements

Based on co-existence studies additional MSDs are needed to be defined, shown in table 5.x.2.2-1.:

Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28-n39-n79 | n28 | 715 | 5 | 25 | 770 | N/A | N/A |
|  | n39 | 1902.5 | 5 | 25 | 1902.5 | N/A | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | 6.7 | IMD3 |
|  | n28 | 727.5 | 5 | 25 | 782.5 | N/A | N/A |
|  | n39 | 1902.5 | 5 | 25 | 1902.5 | N/A | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | 4.0 | IMD41 |
|  | n28 | 715.5 | 5 | 25 | 770.5 | N/A | N/A |
|  | n39 | 1898 | 5 | 25 | 1898 | 5.7 | IMD5 |
|  | n79 | 4760 | 40 | 216 | 4760 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | 15.6 | IMD3 |
|  | n39 | 1887.5 | 5 | 25 | 1887.5 | N/A | N/A |
|  | n79 | 4560 | 40 | 216 | 4560 | N/A | N/A |
|  | n28 | 725 | 5 | 25 | 780 | 8.5 | IMD4 |
|  | n39 | 1900 | 5 | 25 | 1900 | N/A | N/A |
|  | n79 | 4920 | 40 | 216 | 4920 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

----- End of TP -----