3GPP TSG-RAN WG4 Meeting # 108                                   R4-2312542

# Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.9.2.3

Source:	China Telecom

Title:	Discussion on Case 2 requirements for NCSG and concurrent MG

Document for:	Discussion

- Introduction

In RAN4 #107 meeting, RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG were discussed further and the WF [1] was approved.

In this paper, we present our views about Case 2 RRM requirements for the combination of NCSG and concurrent MG issues captured in the WF [1].

- Discussion

Regarding Case 2 RRM requirements for the combination of NCSG and concurrent MG, there are issues about other Rel-17 rules being discussed in this paper.

Other Rel-17 rules to be revisited

Status in the WF [1]:

- Issue 4-4-1: [Case 2] Potential changes for NCSG upon SCell activation < Agreements from meeting RAN4#106-bis-e >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped. < Agreements from online session >: - Option 1: - UE measures the deactivated SCell outside of MG - Option 2: - When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG. - FFS whether a new indication shall be introduced enable support of NCSG for deactivated SCell only. < Tentative agreement >: - Align the understanding of Rel-17 UE behaviours - Only up to 1 NCSG can be configured. All activated Scell MOs are implicitly associated to the NCSG - In the dynamic UE capability signalling, there is no separate indication for activated/deactivated serving cells. This implies UE only indicate the capability if it supports all scenarios, including - deactivated Scell - activated Scell but SSB not in active BWP - Understanding to be clarified: - Will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG? - New in Rel-18 - When Type-2 MG and NCSG are both configured, some serving cell MOs may associated to the NCSG and some are not. - Question 1: What is the expected UE behaviour (assume SMTC partially overlapped with NCSG) - Option 1: skip gap association, all deactivated Scells are measured within NCSG. (This implies some new rule to override the existing gap association rule) - Option 2: Still follow the gap association, i.e., (This implies we follow Rel-17 gap association rule) - Deactivated Scell MO associated with NCSG is measured within NCSG - Deactivated Scell MO not associated with NCSG is measured outside NCSG - Question 2: Whether additional UE capability indication is needed

In our views, when the SCell is deactivated, the deactivated SCell’s MO being measured within NCSG can avoid interruption, it will be beneficial for performance. In our understanding, the expected UE behavior is that the deactivated SCell’s MO is measured within NCSG if the SMTC is partially or fully overlapped with the NCSG. In our views, in the premise of SMTC partially or fully overlapped with the NCSG, the association between deactivated SCell’s MO and NCSG can be implicitly indicated.

Proposal 1: The expected UE behavior is that the deactivated SCell’s MO is measured within NCSG if the SMTC is partially or fully overlapped with the NCSG.

Proposal 2: In the premise of SMTC partially or fully overlapped with the NCSG, the association between deactivated SCell’s MO and NCSG can be implicitly indicated.

- Conclusion

The contribution presents our views about Case 2 RRM requirements for the combination of NCSG and concurrent MG, with the following proposals:

Proposal 1: The expected UE behavior is that the deactivated SCell’s MO is measured within NCSG if the SMTC is partially or fully overlapped with the NCSG.

Proposal 2: In the premise of SMTC partially or fully overlapped with the NCSG, the association between deactivated SCell’s MO and NCSG can be implicitly indicated.

- References

- R4-2310051, WF on NR MG enhancements, MediaTek inc., RAN WG4 Meeting #107, May, 2023.