3GPP TSG-RAN WG4 Meeting #108                                                            R4-2312549

Toulouse, France, 21 – 25 Aug. 2023

Agenda Item:	8.13.5.2

Source:	Ericsson

Title:	On UE demodulation requirements for ATG network

Document for:	Discussion

# 1	Introduction

In last RAN4 #107 meeting, companies had several discussions on the scope and the general issues for UE demodulation requirements for ATG network. A Way Forward [1] was agreed and carried all the agreements and open issues. Main agreements are summarized as follows:

- For UE demodulation requirements, only PDSCH requirement was implicitly agreed to be introduced (Need confirmation in the next meeting). FFS on choosing some of legacy PDCCH test cases to apply. Legacy CSI reporting requirements under AWGN propagation condition will be reused.

- For PDSCH, companies agreed to introduce limited new test cases and pick some legacy cases as well. FFS on which test cases to be chosen.

- 10MHz and 40MHz were agreed for the bandwidth for FDD and TDD respectively. Meanwhile, 4R was also agreed to be tested based on the UE capability.

- 256QAM was agreed to be considered in UE demodulation requirements. FFS on the exact MCS values.

Except the agreements above, however, the test applicability rule about how to test the PDSCH, PDCCH and CSI reporting test cases needs more discussions.

In this contribution, we shared simulation results and observations for 256QAM, and we also discussed the test applicability rule for newly introduced test cases.

# 2	Discussion

## 2.1 TDD pattern

During the last RAN4 #107 meeting, it was agreed that the new TDD pattern with the feature ‘Increasing the number of HARQ processes’ and ‘K1 range extension’ can be considered as one of possible solution to mitigate the guard period impact for Rel-18 ATG scenario. Following agreements are made for UE demodulation:

- For UE demodulation: - New UE demodulation requirement can be specified for the static TDD pattern (if introduced) - FFS on exact test case(s) which may be configured with new TDD pattern. - FFS on test applicability rules. - FFS on the impact of UE demodulation for all candidate options

Comparing the simulation results for configuring new TDD pattern and the legacy pattern, there is no obvious performance difference observed [2]:

Table 2.1-1 Performance comparison for different TDD pattern

| Test num. | TDD pattern | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 7D1S2U | 40/30 | 13 (Table-1) | AWGN + 500Hz Doppler | 2x2 | 70 | 0.39 |
| 2-11 | 30D4S6U |  | 13 (Table-1) | AWGN + 500Hz Doppler | 2x2 | 70 | 0.41 |

Observation 1: There is no performance difference between the new and the legacy TDD pattern.

Regarding defining requirements for this new TDD pattern, it should be decided after receiving the reply and confirmation from RAN1 on the LS [3] for UE feature.  Since there is no performance difference observed, to avoid duplicate testing, it can be considered to only select 16QAM (MCS13) for defining requirement under the assumption of the new TDD pattern if it is agreed to be introduced based on the reply of the LS from RAN1 in following meetings.

Proposal 1: Select following test cases to configure new TDD pattern (if introduced):

| TDD 40MHz 30kHz SCS | FFS on new TDD pattern: 30D4S6U | 2T2R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- | --- |
|  |  | 2T4R | 16QAM (MCS [13] in table 1) |

## 2.2 Parameter assumptions for PDSCH requirement

MCS Selection for 256QAM

It was agreed in the last RAN4 107 meeting to cover 256QAM for ATG. As for the corresponding MCS value, however, it has not been decided yet:

Issue 2-1-4: MCS&Rank Agreement: - Cover 256QAM for ATG demod - Further discuss the MCS value based on simulation results from companies.

To compare the performance of different MCS for 256QAM in ATG scenario we performed followings simulations and summarized the results [2]:

Table 2.2-1 256QAM Performance of different MCS (FDD)

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS index and MCS Table | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-5 | 10 / 15 | 20 (Table-2) | AWGN + 220Hz Doppler | 2x2 | 70 | 12.49 |
| 1-6 |  | 20 (Table-2) | AWGN + 220Hz Doppler | 2x4 | 70 | 9.47 |
| 1-7 |  | 24 (Table-2) | AWGN + 220Hz Doppler | 2x2 | 70 | 16.40 |
| 1-8 |  | 24 (Table-2) | AWGN + 220Hz Doppler | 2x4 | 70 | 13.40 |
| 1-9 |  | 27 (Table-2) | AWGN + 220Hz Doppler | 2x2 | 70 | 19.51 |
| 1-10 |  | 27 (Table-2) | AWGN + 220Hz Doppler | 2x4 | 70 | 16.49 |

Table 2.2-2 256QAM Performance of different MCS (TDD)

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS index and MCS Table | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-5 | 40/30 | 20 (Table-2) | AWGN + 500Hz Doppler | 2x2 | 70 | 12.41 |
| 2-6 |  | 20 (Table-2) | AWGN + 500Hz Doppler | 2x4 | 70 | 9.41 |
| 2-7 |  | 24 (Table-2) | AWGN + 500Hz Doppler | 2x2 | 70 | 16.41 |
| 2-8 |  | 24 (Table-2) | AWGN + 500Hz Doppler | 2x4 | 70 | 13.41 |
| 2-9 |  | 27 (Table-2) | AWGN + 500Hz Doppler | 2x2 | 70 | 19.53 |
| 2-10 |  | 27 (Table-2) | AWGN + 500Hz Doppler | 2x4 | 70 | 16.47 |

Proposal 2: Consider MCS24 (Table-2) for defining requirements for 256QAM

Test cases for defining requirement for PDSCH

Considering the test coverage and to avoid duplicate testing, we propose the following test cases for defining PDSCH requirement:

Proposal 3: Propose following test cases for new PDSCH requirements:

| FDD 10 MHz 15kHz SCS | 2T2R, 2T4R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- |
|  |  | 64QAM (MCS [22] in table 1) |
|  |  | 256QAM (MCS [24] in table 2) |

| TDD 40MHz 30kHz SCS | New TDD pattern: 30D4S6U(if introduced) | 2T2R, 2T4R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- | --- |
|  | 7D1S2U |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |

Applicability rule

ATG is a new deployment scenario which will have newly introduced UE demodulation requirements. Therefore, we should define the corresponding applicability rule.

For PDSCH, newly introduced test cases shall be passed.

Here we provide an example as follows:

5.1.1.12 Applicability of requirements for ATG The performance requirements in Table 5.1.1.12-1 shall apply for UEs which support optional feature supportOfATG. UE capability; Test type; Test list. ATG with 2RX; FR1 FDD; PDSCH; All tests in Clause 5.2.2.1.22 (new). FR1 TDD; PDSCH; All tests in Clause 5.2.2.2.23 (new). ATG with 4RX; FR1 FDD; PDSCH; All tests in Clause 5.2.3.1.21 (new). FR1 TDD; PDSCH; All tests in Clause 5.2.3.2.22 (new).

For PDCCH, following legacy requirements for 2Rx and 4Rx shall be passed:

| UE capability | Test type |  | Test list |
| --- | --- | --- | --- |
| SupportofATG with 2RX | FR1 FDD | PDCCH | All tests in Clause 5.3.2.1.2 |
|  | FR1 TDD | PDCCH | All tests in Clause 5.3.2.2.2 |
| SupportofATG with 4RX | FR1 FDD | PDCCH | All tests in Clause 5.3.3.1.2 |
|  | FR1 TDD | PDCCH | All tests in Clause 5.3.3.2.2 |

Proposal 4: New UE feature for ATG scenario shall be introduced.

Proposal 5: Corresponding applicability rule for newly introduced PDSCH test cases shall be introduced.

Proposal 6: Following legacy PDCCH requirements shall be passed:

| UE capability | Test type |  | Test list |
| --- | --- | --- | --- |
| SupportofATG with 2RX | FR1 FDD | PDCCH | All tests in Clause 5.3.2.1.2 |
|  | FR1 TDD | PDCCH | All tests in Clause 5.3.2.2.2 |
| SupportofATG with 4RX | FR1 FDD | PDCCH | All tests in Clause 5.3.3.1.2 |
|  | FR1 TDD | PDCCH | All tests in Clause 5.3.3.2.2 |

# 3	Summary

In summary, we shared simulation results and observations for 256QAM, and we also discussed the test applicability rule for newly introduced test cases. Based on the analysis and the simulation results above we proposed the following:

Proposal 1: Select following test cases to configure new TDD pattern (if introduced):

| TDD 40MHz 30kHz SCS | FFS on new TDD pattern: 30D4S6U | 2T2R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- | --- |
|  |  | 2T4R | 16QAM (MCS [13] in table 1) |

Proposal 2: Consider MCS24 (Table-2) for defining requirements for 256QAM

Proposal 3: Propose following test cases for new PDSCH requirements:

| FDD 10 MHz 15kHz SCS | 2T2R, 2T4R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- |
|  |  | 64QAM (MCS [22] in table 1) |
|  |  | 256QAM (MCS [24] in table 2) |

| TDD 40MHz 30kHz SCS | New TDD pattern: 30D4S6U(if introduced) | 2T2R, 2T4R | 16QAM (MCS [13] in table 1) |
| --- | --- | --- | --- |
|  | 7D1S2U |  | 64QAM (MCS [22] in table 1) |
|  |  |  | 256QAM (MCS [24] in table 2) |

Proposal 4: New UE feature for ATG scenario shall be introduced.

Proposal 5: Corresponding applicability rule for newly introduced PDSCH test cases shall be introduced

Proposal 6: Following legacy PDCCH requirements shall be passed:

| UE capability | Test type |  | Test list |
| --- | --- | --- | --- |
| SupportofATG with 2RX | FR1 FDD | PDCCH | All tests in Clause 5.3.2.1.2 |
|  | FR1 TDD | PDCCH | All tests in Clause 5.3.2.2.2 |
| SupportofATG with 4RX | FR1 FDD | PDCCH | All tests in Clause 5.3.3.1.2 |
|  | FR1 TDD | PDCCH | All tests in Clause 5.3.3.2.2 |

# References

[1]	R4-2309796, WF for ATG demodulation requirements, CMCC, RAN4 #107, Incheon

[2]	R4-2312550, Simulation results for ATG PDSCH demodulation, Ericsson, RAN4 #108, Toulouse

[3]	R4-2309797, LS on UE features for NR ATG, RAN4, RAN4 #107, Incheon