3GPP TSG-RAN WG4 Meeting # 108	R4-2312555

# Toulouse, France, August 21th – 25th, 2023

Source: 	vivo

Title: 	Discussion on requirements for 4Rx at low frequency band

Agenda Item:	7.29.2

Document for:	Discussion

Introduction

In RAN4#107, the feasibility of low band 4Rx has been confirmed as documented in the WF [1]. The WID has also been updated according to the agreement in RAN#100 in [2].

However, how to specify the requirements is still pending, and a tentative draft has been written in [1]. The ΔRIB,4R value is also still in the range of [-2.7dB, -2.2dB]. A more comprehensive Topic summary can reference to [3].

In this paper, some more views are provided.

Discussion

During RAN4#107, the following agreements have been reached in [1]:

- The feasibility to support low band 4Rx in handheld UE is confirmed at least for some UE.

Based on the agreement, the following bullet has been updated in the WID in [2]:

  - Specify requirements for handheld UE with 4Rx antennas including delta RIB,4R and ∆TRxSRS

According to previous discussion, only ΔRIB,4R needs to be further defined for handheld low band 4Rx, and a proposal was provided in last meeting’s WF for evaluation:

Table 7.3.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n5, n8, n28, n71, n85, n105 | -2.71 |
| n1, n2, n3, n25, n30, n40, n7, n34, n38, n39, n41, n66, n70 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |
| NOTE 1: 4 Rx operation is targeted for FWA form factor, and for handheld UE ΔRIB,4R = TBD is applied |  |

The intention is to differentiate the FWA and handheld requirements. However, in our understanding, define separate and specific requirements in the form of Note is somewhat unusual. In addition, this may potentially bring some confusion or problem if some regional requirements when Note information has been omitted. So it is proposed to list the requirements somewhat in parallel with FWA in the table, and not in the form of a note.

Proposal 1: List the requirements for handheld low band 4Rx somewhat in parallel with FWA in the table, and not in the form of a note.

Regarding the detailed number, some differentiation of delta RIB,4R between handheld and FWA is reasonable and acceptable, but considering the reasonable implementation margin. Unified requirements can also be accepted.

Proposal 2: The requirements of delta RIB,4R can be acceptable as long as in the range of [-2.7dB, -2.2dB].

Conclusion

In this paper, some further analysis and proposals are provided for 4Rx for low band for handheld UE.

Proposal 1: List the requirements for handheld low band 4Rx somewhat in parallel with FWA in the table, and not in the form of a note.

Proposal 2: The requirements of delta RIB,4R can be acceptable as long as in the range of [-2.7dB, -2.2dB].

- References

[1] R4-2310409, WF on UE RF requirements for 3Tx and 4Rx, OPPO, RAN4#107

[2] RP-231013, Revised WID: Core part: 4Rx handheld UE for low NR bands (<1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC, vivo, RAN4#107

[3] R4-2309998, Topic summary for [107][115] NR_3Tx-4Rx_WI, Moderator (OPPO) , RAN4#107