3GPP TSG-RAN WG4 Meeting # 108	R4-2312560

# Toulouse, France, August 21th – 25th, 2023

Source: 	vivo

Title: 	Discussion of Delta_powerclass reporting

Agenda Item:	8.27.1

Document for:	Discussion

Introduction

In RAN4#107, the main progress Coverage enhancement part 1 has been agreed in [1]. A reply LS is also sent out in [2]. The detailed documents and discussion summary can reference to the topic summary in [3].

In the WF, many of the previously discussed topics are stopped for this WI. The only progress and agreement is for DeltaPpowerclass report and relating full power mode reporting.

In this paper, we provide a few views on this.

Discussion

In the WF, the following agreement has been reached:

<Agreement>:

-	Introduce such report the ΔPPowerClass report based on the duty cycle scheme:

- The occasion of the report should be limited to when configured duty cycle is exceeded.

- The full power transmission capability corresponding to the current reference power class should also be reported to guarantee more reasonable UL scheduling.

The full power transmission capability is closed related to the UE implementation, and also the UE power class. With the fall back of power class using ΔPPowerClass, the appropriate capability is indeed may be different. This was proposed in [4] and included in the WF and LS. In another word, this behaviour is reporting multiple sets of capabilities for one UE corresponding to multiple fallback conditions.

However, it should be noted that introducing this kind of multiple sets of capabilities for one UE corresponding to different power class should be cautious, since this may increase the complexities considerably. Anyway, UE is formally only has one set of nominal power class.

Observation: Reporting multiple sets of capabilities for one UE corresponding to multiple fallback conditions may increase the complexities considerably.

With this, although we already agreed the behaviour for fall power mode feature, the following proposal is provided:

Proposal: Any new reporting related to multiple sets of capabilities for one UE corresponding to different power class should be discussed case by case.

Conclusion

In this paper, the following observation and proposal are provided:

Observation: Reporting multiple sets of capabilities for one UE corresponding to multiple fallback conditions may increase the complexities considerably.

Proposal: Any new reporting related to multiple sets of capabilities for one UE corresponding to different power class should be discussed case by case.

- References

[1] R4-2310484, WF on coverage enhancement part1, Huawei, RAN4#107

[2] R4-2310500, Draft Reply LS on enhancements to realize increasing UE power high limit for CA and DC, Ericsson, RAN4#107

[3] R4-2310027, Topic summary for [107][144] NR_cov_enh2_part1, Moderator (Huawei), RAN4#107

[4] R4-2308949, On enhancements of increasing UE power high limit for CA and DC, Huawei, HiSilicon, RAN4#107