# 3GPP TSG-RAN WG4 Meeting #108	R4-2312586

# Toulouse, France, August 21 – August 25, 2023

Source: 	vivo

Title: 	Remaining UE Rx RF issues for SL-U

Agenda Item:	8.30.2.1.3

Document for:	Approval

Introduction

In the last meeting, we had very efficient discussions on SL-U UE Tx/Rx requirements and the agreements were captured in [1]. The TPs [2][3] for SL-U Tx/Rx requirements were approved. In this contribution, we discussed the remaining UE Rx RF issues for SL-U.

Discussion

The SL-U UE Rx requirements were discussed in the last meeting [1]. One remaining issue is RB configuration for NR SL-U reference sensitivity requirement. It is still FFS on RB numbers for 100MHz with 60kHz SCS.

Issue 4-1-1: RB configuration for REFSENS

- Proposals

  - Option 1:

- Table 3-1: Proposed RB configuration for REFSENS requirements for SL-U UE (Facebook)

| NR band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Band | SCS kHz | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 132 |  |
| n96 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 132 |  |
| n102 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 132 |  |

  - Option 2:  (OPPO. LGE)

| NR band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Band | SCS kHz | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 135 |  |
| n96 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 135 |  |
| n102 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 100 | 135 |  |

  - Option 3: To use the number of maximum transmission bandwidth configuration as defined in Table 5.3.2-1 for REFSENS calculation. (VIVO)

Agreement:

- Use Option 2 as baseline

  - FFS on RB numbers for 100MHz with 60kHz SCS

According to [3], the agreed NR SL-U REFSENS requirements were captured as Table 7.1.1-3. According to REFSENS formula, the REFSENS were calculated for 100MHz based on 132 RB and 135 RB for Band n46.

- REFSENS (132 RB)=[-89.6 + 10log10(132/24)]=-82.20 dBm

- REFSENS (135 RB)=[-89.6 + 10log10(135/24)]=-82.10 dBm

It can be observed that the REFSENS difference is 0.1 dB with 132 RB and 135 RB, which is negligible. Since these two numbers are within the maximum RB configuration, we are OK to use 135 RB as the RB configuration for 100MHz with 60kHz SCS.

Table 7.1.1-3 REFSENS for SL-U

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | [-89.2 + 10log10(NRB/105)] | HD |
|  | 30 | 20, 40, 60, 80, 100 | [-89.4 + 10log10(NRB/50)] |  |
|  | 60 | 20, 40, 60, 80, 100 | [-89.6 + 10log10(NRB/24)] |  |
| n96, n102 | 15 | 20, 40 | [-88.7 + 10log10(NRB/105)] | HD |
|  | 30 | 20, 40, 60, 80, 100 | [-88.9 + 10log10(NRB/50)] |  |
|  | 60 | 20, 40, 60, 80, 100 | [-89.1 + 10log10(NRB/24)] |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the RB configuration for NR SL-U REFSENS as shown in table 7.1.1-2 |  |  |  |  |

Proposal 1: To adopt 135 RB for 100MHz with 60kHz SCS for REFSENS calculation for SL-U bands n46, n96 and n102.

Conclusion

This contribution discusses remaining UE Rx requirements for SL-U single carrier operation. The following proposal is made:

Proposal 1: To adopt 135 RB for 100MHz with 60kHz SCS for REFSENS calculation for SL-U bands n46, n96 and n102.

- References

[1] R4-2310310, WF on NR_SL_enh2_UERF_part1, OPPO, RAN4#107.

[2] R4-2310308, on the SL-e TX requirement, OPPO, RAN4#107.

[3] R4-2310309, on the SL-e RX requirement, OPPO, RAN4#107.