# 3GPP TSG-RAN WG4 Meeting #108	R4-2312588

# Toulouse, France, August 21 – August 25, 2023

Source: 	vivo

Title: 	Further discussion on SL CA

Agenda Item:	8.30.2.3

Document for:	Approval

Introduction

In the last meeting, we had extensive discussion on SL CA and the agreements were captured in [1]. The TP [2] for NR SL CA operation was approved. For the remaining issues for SL CA, we provide our views in this contribution.

Discussion

In the RAN4#106bis-e, we agreed to specify reference sensitivity by considering the aggregated channel bandwidth for SL intra-band contiguous CA.

Topic 4: Rx RF Requirements for SL CA

< GTW Agreement>: Specify reference sensitivity by considering the aggregated channel bandwidth for SL intra-band contiguous CA

In the last meeting, the way forward for reference sensitivity for SL CA was captured as follows:

Sub-topic 2-3: Rx requirements for SL CA

<Way Forward>: Reference sensitivity

- FFS in the next meeting: For SL_n47B, the reference sensitivity defined for single carrier operation in n47 as shown in Table 7.3E.2-1 shall apply for each component carrier with two CCs are activated at the same time

However, this agreement in RAN4#106bis-e is confusing to us. The reference sensitivity for SL CA should be based on the Rx channel bandwidth for each CC, instead of the aggregated channel bandwidth. In NR CA, uplink transmission may interfere the DL reception and there is additional degradation for REFSENS value. Unlink NR CA, when UE receives SL CCs, it is not allowed to transmit since SL works in Half Duplex mode. There is no additional exception needed for REFSENS in SL CA case.

From our perspective, the reference sensitivity should be defined for each component carrier with all carriers active for SL CA. For single carrier in band n47, the reference sensitivity defined for each channel bandwidth is shown in Table 7.3E.2-1. And this REFSENS requirements for band n47 shall apply for each component carrier with two CCs are activated at the same time for SL_n47.

Table 7.3E.2-1: Reference sensitivity of NR V2X Bands (PC5) (Section 7.3E.2, TS38.101-1)

| NR V2X Band | SCS kHz | Channel bandwidth / PREFSENS_V2X(dBm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5MHz4 | 10 MHz | 20 MHz | 30 MHz | 40 MHz | Duplex Mode |
| n47 | 15 |  | -92.5 | -89.2 | -87.4 | -86.1 | HD |
|  | 30 |  | -92.1 | -89.4 | -87.7 | -86.2 |  |
|  | 60 |  | -92.9 | -89.1 | -87.9 | -86.4 |  |

Proposal 1: For SL_n47B, the reference sensitivity defined for single carrier operation in n47 as shown in Table 7.3E.2-1 shall apply for each component carrier with two CCs are activated at the same time.

- Conclusion

This contribution discusses remaining issues for SL CA operation on band n47. The following observations and proposals are made:

Proposal 1: For SL_n47B, the reference sensitivity defined for single carrier operation in n47 as shown in Table 7.3E.2-1 shall apply for each component carrier with two CCs are activated at the same time.

- References

[1] R4-2310314, WF on NR_SL_enh2_UERF_part3, Huawei, RAN4#107.

[2] R4-2310312, TP for TR 38.786 on the updated TR structure for NR SL CA operation, Facebook Japan K.K., RAN4#107.

[3] R4-2306637, WF on SL CA, Huawei, HiSilicon, RAN4#106bis-e.