# 3GPP TSG-RAN WG4 Meeting #108	R4-2312589

# Toulouse, France, August 21 – August 25, 2023

Source: 	vivo

Title: 	Discussion on RF architecture for inter-band SSB-less Scell operation

Agenda Item:	8.34.2

Document for:	Approval

Introduction

In the last meeting, work plan [1] on network energy savings for NR was approved. There was some initial discussion on SSB-less operation for FR1 inter-band CA. In this contribution, we give some preliminary thoughts on UE RF architecture aspects for SSB-less operation for FR1 inter-band CA.

Discussion

SSB-less operation for Scell was already supported for intra-band CA in Rel-15. Different CCs share the same RF chain for the intra-band operating band, thus Pcell and Scell go through the similar wireless propagation environment. The Scell can use the measurement results of Pcell for frequency and timing synchronisation.

In Rel-18 NES, it needs further study to enable SSB-less operation for Scell for inter-band CA. For inter-band CA, different CCs can use the different RF chains which is not likely with intra-band CA. Thus, Pcell and Scell can experience different wireless propagation environments. It is likely to happen that the frequency and timing offset can be large for different CCs for inter-band CA. In this case, feasibility study is needed for SSB-less Scell operation for inter-band CA. In RF part, we can give some assumptions on shared or separate RF chains for inter-band CA; in RRM part, further enhancement may be needed to enable this SSB-less operation for inter-band CA.

In Table 1, we give a summary of RF architecture assumptions for inter-band CA band combinations. It can be observed that CCs belong to different bands can assume separate RF chains for inter-band CA. It is suggested to consider separate RF architecture assumption for inter-band CA for SSB-less Scell Operation in RRM session.

Table 1. RF architecture assumption for example inter-band CA band combinations

| Inter-band operating bands | Frequency Range combination | Separate or Shared RF chains |
| --- | --- | --- |
| CA_n41-n79 (n41+n79) | 2.6GHz+4.7GHz | CCs belong to different bands assuming separate RF chains |
| CA_n8-n78 (n8+n78) | 700/800/900MHz + 3.5GHz | CCs belong to different bands assuming separate RF chains |
| CA_n28-n41 (n28+n78) | 700/800/900MHz + 2.6GHz | CCs belong to different bands assuming separate RF chains |
| CA_n1-n78 | 2GHz + 3.5GHz | CCs belong to different bands assuming separate RF chains |
| CA_n1-n8 | 700/800/900MHz + 2GHz | CCs belong to different bands assuming separate RF chains |
| CA_n3-n78 | 1.8GHz + 3.5GHz | CCs belong to different bands assuming separate RF chains |
| CA_n5-n78 | 700/800/900MHz + 3.5GHz | CCs belong to different bands assuming separate RF chains |
| CA_n1-n3 | 1.8GHz + 2GHz | To be confirmed |
| CA_n3-n8 | 700/800/900MHz + 1.8GHz | CCs belong to different bands assuming separate RF chains |
| CA_n3-n8-n78 | 700/800/900MHz + 1.8GHz + 3.5GHz | CCs belong to different bands assuming separate RF chains |
| CA_n1-n8-n78 | 700/800/900MHz + 2GHz + 3.5GHz | CCs belong to different bands assuming separate RF chains |
| CA_n1-n5 | 700/800/900MHz + 2GHz | CCs belong to different bands assuming separate RF chains |
| CA_n3-n5 | 700/800/900MHz + 1.8GHz | CCs belong to different bands assuming separate RF chains |
| CA_n5-n8 | 700/800/900MHz CA | To be confirmed |
| CA_n1-n3-n78 | 1.8GHz + 2GHz + 3.5GHz | To be confirmed |
| CA_n1-n3-n5 | 700/800/900MHz + 1.8GHz + 2GHz | To be confirmed |

Proposal 1: Separate UE RF chain for SSB-less Scell Operation under inter-band CA should be considered as a baseline assumption in RAN4.

Conclusion

This contribution discusses baseline RF architectures related to SSB-less operation for inter-band CA. The following proposal is made:

Proposal 1: Separate UE RF chain for SSB-less Scell Operation under inter-band CA should be considered as a baseline assumption in RAN4.

- References

[1] R4-2306640, Work plan on network energy savings for NR, Huawei, HiSilicon, RAN4#107.