3GPP TSG-RAN WG4 Meeting #108                                                               R4-2312611

# Toulouse, France, August 21 – 25, 2023

Source:	ZTE Corporation, CHTTL

Title:	Clarification on delta TIB and RIB values for band combination

Agenda Item:	8.1.3

Document for:	Approval

- 1	Introduction

In the current specifications, for the UE which supports EN-DC, CA/DC and SUL configurations, the allowed maximum configured output power relaxation has been set for the inter-band or SUL operation as ΔTIB,c. The allowed reference sensitivity relaxation has been set for the inter-band operation as ΔRIB,c. To optimize the tables of ΔTIB,c and ΔRIB,c, the new templates have been applied to the specification as shown in Table 1 and Table 2 respectively.

Table 1: New template for ΔTIB,c tables in Rel-18

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration |  |  |  |  |
| DC_1-3-7-40_n78 | 0.6 | 0.6 | 0.5 | 0.3 | 0.8 |
| DC_1-3-8-11_n28 | 0.3 | 0.8 | 0.6 | 0.9 | 0.6 |
| … | … | … | … | … | … |
| Note 1: “-” denotes ΔTIB,c = 0. Note 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-48_(n)5 the band order from left to right is 2, 5, 48 and n5. |  |  |  |  |  |

Table 2: New template for ΔRIB,c tables in Rel-18

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration |  |  |  |  |
| DC_1-3-7-40_n78 | 0.2 | 0.2 | - | 0.4 | 0.5 |
| DC_1-3-8-11_n28 | - | 0.3 | 0.2 | 0.5 | 0.2 |
| … | … | … | … | … | … |
| Note 1: “-” denotes ΔRIB,c = 0. Note 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-48_(n)5 the band order from left to right is 2, 5, 48 and n5. |  |  |  |  |  |

In the above tables, for the values of delta TIB and RIB, it is noted that unless otherwise stated, the ΔTIB,c / ΔRIB,c is set to zero which is denoted as “-”. However, for some band combinations with the component bands having special bands such as SDL bands, SUL bands and immediately close bands, the delta TIB and RIB tables are currently also denoted as “-” which are not reasonable. In this paper, we intend to clarify some special cases for delta TIB and RIB values for band combinations.

- 2	Discussions on delta TIB and RIB values

Based on the discussion above, we see that for some special cases in delta TIB and RIB tables, the values are denoted as “-” no matter the values are set to zero for the corresponding bands or the component bands are special bands whose values are unavailable. In our understanding, the note of ‘“-” denotes ΔRIB,c = 0’ or ‘“-” denotes ΔTIB,c = 0’  means the requirement is applicable and defined as 0dB. However, for some special band combinations discussed in the following, the requirement is not applicable.

- Band combinations with SDL component bands

For the band combination having the component SDL band, there is no delta TIB value defined due to no uplink transmission on the corresponding band. For example, in Table 6.2A.4.2.3-1 of ΔTIB,c in TS 38.101-1, the delta TIB value of band n75 and n76 with “-” which means zero should be “N/A” to be more accurately.

Table 3: ΔTIB,c due to NR CA (two bands) (Table 6.2A.4.2.3-1 in TS 38.101-1)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| … | … | … |
| CA_n75-n78 | - → N/A | 0.8 |
| CA_n76-n78 | - → N/A | 0.8 |
| … | … | … |

- Band combinations with SUL component bands

For the band combination having the component SUL band, there is no delta RIB value defined due to no downlink reception on the corresponding band. For example, in Table 7.3C.3.2.1-1 of ΔRIB,c in TS 38.101-1, the delta RIB value of the SUL bands n80, n84, n95, n98 and n99 with “-” which means zero should be “N/A” to be more accurately, although there is a “NOTE 2:	“-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s)” which is somehow confusing.

Table 4: ΔRIB,c due to SUL (two bands) (Table 7.3C.3.2.1-1 in TS 38.101-1)

| Band combination for SUL | ΔRIB,c for NR band (dB)2 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration3 |  |
| SUL_n41-n80 | 0.51 | - → N/A |
| SUL_n41-n95 | 0.2 | - → N/A |
| SUL_n41-n98 | 0.2 | - → N/A |
| SUL_n48-n99 | 0.5 | - → N/A |
| SUL_n77-n80 | 0.5 | - → N/A |
| SUL_n77-n84 | 0.5 | - → N/A |
| SUL_n77-n99 | 0.5 | - → N/A |
| … | … | … |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 2: ΔRIB,c is not applicable to SUL band(s). NOTE 3: The component band order in the configuration should be listed by the order of NR band, such as for SUL_n41-n80 the order of band is n41 and n80. |  |  |

Table 5: ΔRIB,c due to SUL (three bands) (Table 7.3C.3.2.2-1 in TS 38.101-1)

| Band combination for SUL | ΔRIB,c for NR bands (dB)2 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration3 |  |  |
| CA_n1_n78-n80 | 0.2 | 0.5 | → N/A |
| CA_n1_n78-n81 | - | 0.5 | → N/A |
| CA_n1_n78-n84 | 0.2 | 0.5 | → N/A |
| CA_n3_n41-n80 | - | 0.51 | → N/A |
| … | … | … | … |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 2: “-” denotes ΔRIB,c = 0 and ΔRIB,c is not applicable to SUL band(s). NOTE 3: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1_n78-n80 the order of band is n1, n78 and n80. |  |  |  |

- Band combinations with Immediately close bands

For some band combinations having the component immediately close bands, such as band 7/n7 and 38/n38, the UL and DL cannot appear simultaneously. There is no delta TIB value defined due to no uplink transmission on the corresponding immediately close bands. For example, in Table 6.2A.4.2.4-1 of ΔTIB,c in TS 38.101-1, the delta TIB value of the immediately close bands n7 and n38 should be “N/A” instead of “-” which will cause somehow confusion.

Table 6: ΔTIB,c due to NR CA (three bands) (Table 6.2A.4.2.4-1 in TS 38.101-1)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| … | … | … | … |
| CA_n1-n7-n38 | 0.5 | - → N/A | - → N/A |
| … | … | … | … |
| CA_n3-n7-n38 | 0.5 | - → N/A | - → N/A |
| … | … | … | … |

- Band combinations with overlapping bands

For the band combinations having the component overlapping bands, such as band 48 and n77, the uplink EN-DC configurations are all marked as “N/A”. In NOTE 9 of Table 5.5B.4.1-1 for inter-band EN-DC configuration table, the combination is not used alone as fallback mode of other band combinations in which UL in Band 42 or Band 48 is not used. Thus take DC_2A-48A_n77A as an example, for the higher order configurations having the component overlapping bands, the delta TIB value for the overlapping bands such as Band 48 should be denoted as “N/A” instead of “-”, while the delta TIB value for Band n77 has non-zero value.

| 48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| --- | --- | --- | --- |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_48A_n77A3. 4. 9, 11 DC_48C_n77A3. 4. 9, 11 DC_48A_n77C3. 4. 9, 11 DC_48C_n77C3. 4. 9, 11 DC_48D_n77A3. 4. 9, 11 DC_48D_n77C3. 4. 9, 11 DC_48E_n77A3. 4. 9, 11 | N/A | N/A |  |
| NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 or Band 48 is not used. |  |  |  |

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-48A_n77A14,15,16 | DC_2A_n77A14 |

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_2-48_n77 DC_2-48-48_n77 DC_2-48-48-48_n77 | 0.3 | 0.6 → N/A | 0.5 |

- EN-DC combination with LTE LAA band

For EN-DC combination with LTE LAA band such as B46, since the LAA band could only be used as Scell and no uplink for B46 in the related EN-DC combination. Thus, for EN-DC combination with LTE LAA band, there is no delta TIB value for the component LTE LAA band. For example, in Table 6.2B.4.2.3.2-1 of ΔTIB,c in TS 38.101-3, the delta TIB value for the LTE LAA band should be “N/A” instead of “-”.

Table 7: ΔTIB,c due to EN-DC (three bands) (Table 6.2B.4.2.3.2-1 in TS 38.101-3)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_2-46_n5 DC_2-2-46_n5 | 0.3 | - → N/A | 0.3 |
| DC_2-46_n41 | 0.5 | - → N/A | 0.41 / 0.92 |
| DC_2-46_n66 | 0.5 | - → N/A | 0.5 |
| DC_2-46_n77 DC_2-46-46_n77 | 0.6 | - → N/A | 0.8 |

Proposal 1:	 It is suggested to differentiate the value of “-” and “N/A” for delta TIB and RIB in NR CA and EN-DC configurations.

- For the band which has the value of zero, for the sake of simplicity the value in the delta TIB/RIB table will be denoted as “-”, as what it is now in the specification.

- For the component bands which are SDL bands, SUL bands, immediately close bands, band combination with overlapping bands, or EN-DC combination with LTE LAA band, etc.

    - If uplink is not supported on a constituted band of the DC/CA band combination, "N/A" is used when deriving the delta T requirements for that constituted band of the band combination.

    - If downlink is not supported on a constituted band of the DC/CA band combination, "N/A" is used when deriving the delta R requirements for that constituted band of the band combination.

- 3	Conclusion

In this paper, we clarify the value of delta TIB and RIB for some special cases in NR CA and EN-DC configurations. The value of delta TIB and RIB for the component bands having SDL bands, SUL bands and immediately close bands has been discussed. Based on the discussion, the following proposal has been proposed.

Proposal 1:	 It is suggested to differentiate the value of “-” and “N/A” for delta TIB and RIB in NR CA and EN-DC configurations.

- For the band which has the value of zero, for the sake of simplicity the value in the delta TIB/RIB table will be denoted as “-”, as what it is now in the specification.

- For the component bands which are SDL bands, SUL bands, immediately close bands, band combination with overlapping bands, or EN-DC combination with LTE LAA band, etc.

    - If uplink is not supported on a constituted band of the DC/CA band combination, "N/A" is used when deriving the delta T requirements for that constituted band of the band combination.

    - If downlink is not supported on a constituted band of the DC/CA band combination, "N/A" is used when deriving the delta R requirements for that constituted band of the band combination.

- 4	References

[1] 	TS 38.101-1, NR UE radio transmission and reception; Part 1: Range 1 Standalone.

[2] 	TS 38.101-3, NR UE radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios.