3GPP TSG-RAN WG4 # 108	R4-2312613

Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.4.3.1.4

Source:	MediaTek Inc.

Title:	Discussion on CQI requirements for 8Rx UE

Document for:	Discussion

- Introduction

In RAN4#107 meeting, a WF related to 8Rx UE CQI requirements was agreed [1]. In this contribution, we propose SNR points for CQI test under static channels according to simulation results.

- Discussion

In the last meeting, RAN4 discussed the SNR points for CQI requirements and there was no conclusion.

Issue 4-2: SNR points - Proposals - Option 1: [4,5] dB and [10,11] dB - Option 2: [1, 2] dB and [7,8] dB

We provide the simulation results based on Annex A.3 in [1].

Figure 1. Simulation results for median CQI

According to results in Figure 1, we propose to use “SNR [4,5] dB for 16QAM” and “SNR [10,11] dB for 64QAM” for 8Rx CQI testing.

Proposal 1: For 8Rx UE CQI requirements under static channels, RAN4 can use “SNR [4,5] dB for 16QAM” and “SNR [10,11] dB for 64QAM”.

- Conclusion

In this contribution, we propose SNR points for CQI test under static channels based on our simulation results.

Proposal 1: For 8Rx UE CQI requirements under static channels, RAN4 can use “SNR [4,5] dB for 16QAM” and “SNR [10,11] dB for 64QAM”.

- Reference

- R4-2309806, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon