3GPP TSG-RAN WG4 Meeting #108	R4-2312646

# Toulouse, FR, August 21st – 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR |  | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-2 to add new CA configurations for the NR intra-band CA combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R18_intra |  |  |  |  |  | Date: |  |  | 2023-07-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New configurations for intra-band CA combinations for FR2 only are introduced according to the request |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the following new CA configurations: CA_n260(3G/4G) CA_n260(2O/3O/4O/2P) CA_n260(2O-4O/P/Q) CA_n260(A-O/P/Q/2H/2O/2P/3G/3O) CA_n260(3A-H/O/P/Q/2O/2G) CA_n260(2A-H/O/P/Q/2H/2O/2G/2P/3O/3G) CA_n260(4A-O/P/Q/G/2G/2O) CA_n260(5A-O/P/G) CA_n260(O-P/Q/2P) CA_n260(P-Q) CA_n260(A-G-H) CA_n260(A-O-P/Q) CA_n260(2A-O-P) CA_n260(2A-G-H) CA_n260(G-O/2O/3O) CA_n260(2G-O/2O) CA_n260(A-G-O) CA_n260(2A-G-O/2O) CA_n260(A-2G-O) CA_n260(2A-2G-O) CA_n260(3G-O) CA_n260(H/2H-O) CA_n260(3A/4A-G-O) CA_n260(A-G-2O) CA_n260(A-H-O) CA_n260(G-H-O) CA_n260(2A-O-Q) CA_n260(2Q) CA_n260(6A-O) CA_n260(A-2O-P) CA_n261(A-2H/3G) CA_n261(A-H-I) CA_n261(2A-2G-O) CA_n261(2A-2G/3G) CA_n261(2A-G-2O) CA_n261(2A-G-O) CA_n261(2G-O/2O) CA_n261(3A-2G) CA_n261(3A-G-O) CA_n261(3G-O) CA_n261(3G) CA_n261(4A-G) CA_n261(4G) CA_n261(A-2G-O) CA_n261(A-G-O/2O) CA_n261(G-O/2O) CA_n261(A-J/K/L) CA_n261(G-J) The fallbacks are either proposed in this CR or specified in latest spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requested band combinations will not exist in current TS 38.101-2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of change >>

5.5	Configurations

5.5A	Configurations for CA

5.5A.2	Configurations for intra-band specific CA

Configurations listed in this clause apply to downlink carrier aggregation only.

NOTE:	Sub-blocks belonging to a CA configuration can be in any order. In other words certain CA configuration acronym includes all sub-block arrangements which have exactly the same sub-block set. As an example, CA_n260(2G-3O) denotes CA_n260(2O-2G-O), CA_n260(G-3O-G) etc. but these are not listed in tables separately.

Table 5.5A.2-1: NR CA configurations with single CA bandwidth class defined for intra-band non-contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |
| --- | --- | --- | --- |
| NR configuration | Uplink CA configurations | (BWChannel,block) (MHz) | BCS |
| CA_n257(2A) | - | 800 | 0 |
| CA_n257(2G) | CA_n257(2G) | 400 | 0 |
| CA_n258(2A) | - | 800 | 0 |
| CA_n258(3A) | - | 1200 | 0 |
| CA_n258(4A) | - | 1600 | 0 |
| CA_n258(5A) | - | 2000 | 0 |
| CA_n258(2G) | CA_n258G | 400 | 0 |
| CA_n260(2A) | CA_n260(2A) | 800 | 0 |
| CA_n260(3A) | CA_n260(3A) | 1200 | 0 |
| CA_n260(4A) | - | 1600 | 0 |
| CA_n260(5A) | - | 2000 | 0 |
| CA_n260(6A) | - | 2400 | 0 |
| CA_n260(7A) | - | 2800 | 0 |
| CA_n260(8A) | - | 2900 | 0 |
| CA_n260(9A) | - | 2950 | 0 |
| CA_n260(10A) | - | 2950 | 0 |
| CA_n260(2D) | - | 800 | 0 |
| CA_n260(2G) | CA_n260G | 400 | 0 |
| CA_n260(3G) | CA_n260G | 600 | 0 |
| CA_n260(4G) | CA_n260G | 800 | 0 |
| CA_n260(2H) | CA_n260G/H | 600 | 0 |
| CA_n260(2O) | CA_n260O | 400 | 0 |
| CA_n260(3O) | CA_n260O | 600 | 0 |
| CA_n260(4O) | CA_n260O | 800 | 0 |
| CA_n260(2P) | CA_n260O/P | 600 | 0 |
| CA_n260(3P) | - | 900 | 0 |
| CA_n260(4P) | - | 1200 | 0 |
| CA_n260(2Q) | CA_n260O/P/Q | 800 | 0 |
| CA_n261(2A) | - | 800 | 0 |
| CA_n261(3A) | - | 800 | 0 |
| CA_n261(4A) | - | 800 | 0 |
| CA_n261(5A) | - | 800 | 0 |
| CA_n261(6A) | - | 800 | 0 |
| CA_n261(7A) | - | 800 | 0 |
| CA_n261(8A) | - | 800 | 0 |
| CA_n261(2D) | - | 800 | 0 |
| CA_n261(2G) | CA_n261G | 400 | 0 |
| CA_n261(3G) | CA_n261G | 600 | 0 |
| CA_n261(4G) | CA_n261G | 800 | 0 |
| CA_n261(2H) | CA_n261G/H | 600 | 0 |
| CA_n261(2I) | CA_n261G/H/I | 800 | 0 |
| CA_n261(2O) | - | 400 | 0 |
| CA_n261(3O) | - | 600 | 0 |
| CA_n261(4O) | - | 800 | 0 |
| CA_n261(5O) | - | 800 | 0 |
| CA_n261(6O) | - | 800 | 0 |
| CA_n261(7O) | - | 800 | 0 |
| CA_n261(2P) | - | 600 | 0 |
| CA_n261(2Q) | - | 800 | 0 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Void NOTE 4: Void NOTE 5: Channel bandwidth per operating band defined in Table 5.3.5-1 NOTE 6: Unless otherwise stated, BCS0 is referred in each constituent CA configuration NOTE 7: (BWChannel,block) denotes the maximum total bandwidth from the summation of the sub-block bandwidths and shall be less than the bandwidth of the operating band. NOTE 8: The delimiter “/” will only be used in the uplink configurations for the sake of simplicity. For example, CA_nyA/B/C denotes CA_nyA, CA_nyB and CA_nyC, where ny is a FR2 NR band and A, B and C are the corresponding bandwidth classes respectively. |  |  |  |

Table 5.5A.2-2: NR CA configurations with multiple CA bandwidth classes defined for intra-band non-contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |
| --- | --- | --- | --- |
| CA configuration | Uplink CA configurations | (BWChannel,block) (MHz) | BCS |
| CA_n257(A-G) | CA_n257G | 600 | 0 |
| CA_n257(A-D) | - | 800 | 0 |
| CA_n257(A-H) | - | 700 | 0 |
| CA_n257(A-I) | - | 800 | 0 |
| CA_n257(G-H) | - | 500 | 0 |
| CA_n257(G-I) | - | 600 | 0 |
| CA_n258(A-D) | - | 800 | 0 |
| CA_n258(A-G) | CA_n258G | 600 | 0 |
| CA_n258(A-H) | CA_n258G/H | 700 | 0 |
| CA_n258(A-I) | - | 800 | 0 |
| CA_n258(G-H) | CA_n258G/H | 500 | 0 |
| CA_n258(G-I) | - | 600 | 0 |
| CA_n260(A-D) | - | 800 | 0 |
| CA_n260(A-E) | - | 1000 | 0 |
| CA_n260(2A-D) | - | 1200 | 0 |
| CA_n260(3A-D) | - | 1600 | 0 |
| CA_n260(4A-D) | - | 2000 | 0 |
| CA_n260(A-2D) | - | 1200 | 0 |
| CA_n260(2A-2D) | - | 1600 | 0 |
| CA_n260(A-D-O) | - | 1000 | 0 |
| CA_n260(A-D-G) | - | 1000 | 0 |
| CA_n260(2A-D-O) | - | 1400 | 0 |
| CA_n260(3A-D-O) | - | 1800 | 0 |
| CA_n260(A-D-2O) | - | 1200 | 0 |
| CA_n260(2A-D-2O) | CA_n260G/H | 1600 | 0 |
| CA_n260(A-G) | CA_n260G | 600 | 0 |
| CA_n260(A-G-H) | CA_n260G/H | 900 | 0 |
| CA_n260(2A-G) | CA_n260G | 1000 | 0 |
| CA_n260(A-2G) | CA_n260G | 800 | 0 |
| CA_n260(A-3G) | CA_n260G | 1000 | 0 |
| CA_n260(A-4G) | - | 1200 | 0 |
| CA_n260(2A-2G) | CA_n260G | 1200 | 0 |
| CA_n260(2A-3G) | CA_n260G | 1400 | 0 |
| CA_n260(2A-2G-O) | CA_n260G/O | 1400 | 0 |
| CA_n260(2A-2G-2O) | - | 1600 | 0 |
| CA_n260(3A-2G) | CA_n260G | 1600 | 0 |
| CA_n260(3A-2G-O) | - | 1800 | 0 |
| CA_n260(4A-G) | CA_n260G | 1800 | 0 |
| CA_n260(4A-G-O) | CA_n260G/O | 2000 | 0 |
| CA_n260(4A-2G) | CA_n260G | 2000 | 0 |
| CA_n260(5A-G) | CA_n260G | 2200 | 0 |
| CA_n260(A-2G-2O) | - | 1200 | 0 |
| CA_n260(A-2G-3O) | - | 1400 | 0 |
| CA_n260(2A-G-H) | CA_n260G/H | 1300 | 0 |
| CA_n260(2A-G-2O) | CA_n260G/O | 1400 | 0 |
| CA_n260(2A-G-3O) | - | 1600 | 0 |
| CA_n260(3A-G) | CA_n260G | 1400 | 0 |
| CA_n260(3A-G-O) | CA_n260G/O | 1600 | 0 |
| CA_n260(3A-G-2O) | - | 1800 | 0 |
| CA_n260(A-2H) | CA_n260G/H | 1000 | 0 |
| CA_n260(2A-H) | CA_n260G/H | 1100 | 0 |
| CA_n260(3A-H) | CA_n260G/H | 1500 | 0 |
| CA_n260(2A-2H) | CA_n260G/H | 1400 | 0 |
| CA_n260(A-H) | CA_n260G/H | 700 | 0 |
| CA_n260(A-H-O) | CA_n260G/H/O | 900 | 0 |
| CA_n260(A-O) | CA_n260O | 600 | 0 |
| CA_n260(A-O-P) | CA_n260O/P | 900 | 0 |
| CA_n260(A-O-P-Q) | - | 1300 | 0 |
| CA_n260(A-O-2P) | - | 1200 | 0 |
| CA_n260(A-O-3P) | - | 1500 | 0 |
| CA_n260(2A-O-P) | CA_n260O/P | 1300 | 0 |
| CA_n260(2A-O-P-Q) | - | 1700 | 0 |
| CA_n260(2A-O-2P) | - | 1600 | 0 |
| CA_n260(2A-O-3P) | - | 1900 | 0 |
| CA_n260(2A-2O-P) | - | 1500 | 0 |
| CA_n260(2A-2O-P-Q) | - | 1900 | 0 |
| CA_n260(A-O-Q) | CA_n260O/P/Q | 1000 | 0 |
| CA_n260(A-O-2Q) | - | 1400 | 0 |
| CA_n260(2A-O-Q) | CA_n260O/P/Q | 1400 | 0 |
| CA_n260(2A-O-2Q) | - | 1800 | 0 |
| CA_n260(2A-2O-Q) | - | 1600 | 0 |
| CA_n260(2A-O) | CA_n260O | 1000 | 0 |
| CA_n260(A-2O) | CA_n260O | 800 | 0 |
| CA_n260(A-2O-P) | CA_n260O/P | 1100 | 0 |
| CA_n260(A-2O-P-Q) | - | 1500 | 0 |
| CA_n260(A-2O-2P) | - | 1400 | 0 |
| CA_n260(A-2O-Q) | - | 1200 | 0 |
| CA_n260(A-2O-2Q) | - | 1600 | 0 |
| CA_n260(A-3O-P) | - | 1300 | 0 |
| CA_n260(A-3O-Q) |  | 1400 | 0 |
| CA_n260(2A-2O) | CA_n260O | 1200 | 0 |
| CA_n260(2A-2O-2P) | - | 1800 | 0 |
| CA_n260(2A-2O-2Q) | - | 2000 | 0 |
| CA_n260(2A-3O) | CA_n260O | 1400 | 0 |
| CA_n260(2A-3O-P) | - | 1700 | 0 |
| CA_n260(2A-3O-Q) | - | 1800 | 0 |
| CA_n260(3A-2O) | CA_n260O | 1600 | 0 |
| CA_n260(3A-2O-P) | - | 1900 | 0 |
| CA_n260(3A-2O-Q) | - | 2000 | 0 |
| CA_n260(4A-O) | CA_n260O | 1800 | 0 |
| CA_n260(4A-O-P) | - | 2100 | 0 |
| CA_n260(4A-O-Q) | - | 2200 | 0 |
| CA_n260(5A-O-P) | - | 2500 | 0 |
| CA_n260(6A-O-P) | - | 2900 | 0 |
| CA_n260(4A-3O) | - | 2200 | 0 |
| CA_n260(5A-O) | CA_n260O | 2200 | 0 |
| CA_n260(6A-O) | CA_n260O | 2600 | 0 |
| CA_n260(7A-O) | - | 2950 | 0 |
| CA_n260(8A-O) | - | 2950 | 0 |
| CA_n260(4A-2O) | CA_n260O | 2000 | 0 |
| CA_n260(4A-2Q) | - | 2400 | 0 |
| CA_n260(3A-3O) | - | 1800 | 0 |
| CA_n260(A-G-O) | CA_n260G/O | 800 | 0 |
| CA_n260(A-G-2O) | CA_n260G/O | 1000 | 0 |
| CA_n260(A-G-3O) | - | 1200 | 0 |
| CA_n260(A-G-4O) | - | 1400 | 0 |
| CA_n260(2A-G-O) | CA_n260G/O | 1200 | 0 |
| CA_n260(A-2G-O) | CA_n260G/O | 1000 | 0 |
| CA_n260(A-3G-O) | - | 1200 | 0 |
| CA_n260(A-3O) | CA_n260O | 1000 | 0 |
| CA_n260(3A-O) | CA_n260O | 1400 | 0 |
| CA_n260(3A-O-P) | CA_n260O/P | 1700 | 0 |
| CA_n260(3A-O-P-Q) | - | 2100 | 0 |
| CA_n260(3A-O-2P) | - | 2000 | 0 |
| CA_n260(3A-O-Q) | - | 1800 | 0 |
| CA_n260(3A-O-2Q) | - | 2200 | 0 |
| CA_n260(A-4O) | - | 1200 | 0 |
| CA_n260(2A-4O) | - | 1600 | 0 |
| CA_n260(3A-4O) | - | 2000 | 0 |
| CA_n260(4A-4O) | - | 2400 | 0 |
| CA_n260(5A-4O) | - | 2800 | 0 |
| CA_n260(A-P) | CA_n260O/P | 700 | 0 |
| CA_n260(A-3P) | - | 1300 | 0 |
| CA_n260(A-4P) | - | 1600 | 0 |
| CA_n260(A-P-Q) | CA_n260P/Q | 1100 | 0 |
| CA_n260(2A-P) | CA_n260O/P | 1100 | 0 |
| CA_n260(2A-P-Q) | - | 1500 | 0 |
| CA_n260(3A-P) | CA_n260O/P | 1500 | 0 |
| CA_n260(3A-P-Q) | - | 1900 | 0 |
| CA_n260(4A-P) | CA_n260O/P | 1900 | 0 |
| CA_n260(4A-P-Q) | - | 2300 | 0 |
| CA_n260(5A-P) | CA_n260O/P | 2300 | 0 |
| CA_n260(6A-P) | - | 2700 | 0 |
| CA_n260(7A-P) | - | 3000 | 0 |
| CA_n260(A-2P) | CA_n260O/P | 1000 | 0 |
| CA_n260(2A-2P) | CA_n260O/P | 1400 | 0 |
| CA_n260(2A-3P) | - | 1700 | 0 |
| CA_n260(2A-4P) | - | 2000 | 0 |
| CA_n260(3A-2P) | - | 1800 | 0 |
| CA_n260(3A-3P) | - | 2100 | 0 |
| CA_n260(4A-2P) | - | 2200 | 0 |
| CA_n260(5A-2P) | - | 2600 | 0 |
| CA_n260(5A-2O) | - | 2400 | 0 |
| CA_n260(6A-2O) | - | 2800 | 0 |
| CA_n260(5A-3O) | - | 2600 | 0 |
| CA_n260(6A-3O) | - | 2950 | 0 |
| CA_n260(7A-2O) | - | 2950 | 0 |
| CA_n260(7A-3O) | - | 2950 | 0 |
| CA_n260(6A-2P) | - | 2950 | 0 |
| CA_n260(8A-2O) | - | 2550 | 0 |
| CA_n260(9A-O) | - | 3000 | 0 |
| CA_n260(A-Q) | CA_n260O/P/Q | 800 | 0 |
| CA_n260(A-2Q) | - | 1200 | 0 |
| CA_n260(2A-Q) | CA_n260O/P/Q | 1200 | 0 |
| CA_n260(2A-2Q) | - | 1600 | 0 |
| CA_n260(3A-Q) | CA_n260O/P/Q | 1600 | 0 |
| CA_n260(3A-2Q) | - | 2000 | 0 |
| CA_n260(4A-Q) | CA_n260O/P/Q | 2000 | 0 |
| CA_n260(5A-Q) | - | 2400 | 0 |
| CA_n260(D-2G) | - | 800 | 0 |
| CA_n260(2D-O) | - | 1000 | 0 |
| CA_n260(D-2O) | - | 800 | 0 |
| CA_n260(A-I) | CA_n260I | 800 | 0 |
| CA_n260(D-G) | CA_n260D/G | 600 | 0 |
| CA_n260(D-H) | CA_n260D/H | 700 | 0 |
| CA_n260(D-I) | CA_n260D/I | 800 | 0 |
| CA_n260(D-O) | CA_n260D/O | 600 | 0 |
| CA_n260(D-P) | CA_n260D/P | 700 | 0 |
| CA_n260(D-Q) | CA_n260D/Q | 800 | 0 |
| CA_n260(E-O) | CA_n260E/O | 800 | 0 |
| CA_n260(E-P) | CA_n260E/P | 800 | 0 |
| CA_n260(E-Q) | CA_n260E/Q | 1000 | 0 |
| CA_n260(G-H) | CA_n260G/H | 500 | 0 |
| CA_n260(G-H-O) | CA_n260G/H/O | 700 | 0 |
| CA_n260(G-I) | CA_n260G/I | 600 | 0 |
| CA_n260(G-O) | CA_n260G/O | 400 | 0 |
| CA_n260(G-2O) | CA_n260G/O | 600 | 0 |
| CA_n260(2G-O) | CA_n260G/O | 600 | 0 |
| CA_n260(2G-2O) | CA_n260G/O | 800 | 0 |
| CA_n260(G-3O) | CA_n260G/O | 800 | 0 |
| CA_n260(3G-O) | CA_n260G/O | 800 | 0 |
| CA_n260(2G-3O) | - | 1000 | 0 |
| CA_n260(G-4O) | - | 1000 | 0 |
| CA_n260(2G-4O) | - | 1200 | 0 |
| CA_n260(4G-O) | - | 1000 | 0 |
| CA_n260(H-O) | CA_n260G/H/O | 500 | 0 |
| CA_n260(2H-O) | CA_n260G/H/O | 800 | 0 |
| CA_n260(O-2P) | CA_n260O/P | 800 | 0 |
| CA_n260(O-3P) | - | 1100 | 0 |
| CA_n260(O-2Q) | - | 1000 | 0 |
| CA_n260(O-P) | CA_n260O/P | 500 | 0 |
| CA_n260(O-P-Q) | - | 900 | 0 |
| CA_n260(2O-P) | CA_n260O/P | 700 | 0 |
| CA_n260(2O-P-Q) | - | 1100 | 0 |
| CA_n260(2O-2P) | - | 1000 | 0 |
| CA_n260(3O-P) |  | 900 | 0 |
| CA_n260(3O-Q) |  | 1000 | 0 |
| CA_n260(O-Q) | CA_n260O/P/Q | 600 | 0 |
| CA_n260(2O-Q) | CA_n260O/P/Q | 800 | 0 |
| CA_n260(2O-2Q) | - | 1200 | 0 |
| CA_n260(P-Q) | CA_n260O/P/Q | 700 | 0 |
| CA_n261(A-D) | - | 800 | 0 |
| CA_n261(A-E) | - | 800 | 0 |
| CA_n261(2A-D) | - | 800 | 0 |
| CA_n261(2A-D-O) | - | 800 | 0 |
| CA_n261(A-2D) | - | 800 | 0 |
| CA_n261(A-D-G) | - | 800 | 0 |
| CA_n261(A-D-H) | - | 800 | 0 |
| CA_n261(A-D-O) | - | 800 | 0 |
| CA_n261(A-D-2O) | - | 800 | 0 |
| CA_n261(A-G) | CA_n261G | 600 | 0 |
| CA_n261(A-G-H) | CA_n261G/H | 800 | 0 |
| CA_n261(A-G-I) | CA_n261G/H/I | 800 | 0 |
| CA_n261(A-G-O) | CA_n261G | 800 | 0 |
| CA_n261(2A-G-O) | CA_n261G | 800 | 0 |
| CA_n261(A-G-2O) | CA_n261G | 800 | 0 |
| CA_n261(2A-G-2O) | CA_n261G | 800 | 0 |
| CA_n261(A-2G-O) | CA_n261G | 800 | 0 |
| CA_n261(A-2G-2O) | - | 800 | 0 |
| CA_n261(A-3G) | CA_n261G | 800 | 0 |
| CA_n261(A-3G-O) | - | 800 | 0 |
| CA_n261(A-2G) | CA_n261G | 800 | 0 |
| CA_n261(A-4G) | - | 800 | 0 |
| CA_n261(A-H) | CA_n261G/H | 700 | 0 |
| CA_n261(A-2H) | CA_n261G/H | 800 | 0 |
| CA_n261(A-H-I) | CA_n261G/H/I | 800 | 0 |
| CA_n261(A-I) | CA_n261G/H/I | 800 | 0 |
| CA_n261(A-2I) | - | 800 | 0 |
| CA_n261(A-J) | CA_n261G/H/I/J | 700 | 0 |
| CA_n261(A-K) | CA_n261G/H/I/J/K | 800 | 0 |
| CA_n261(A-L) | CA_n261A/G/H/I/J/K/L | 800 | 0 |
| CA_n261(A-O) | - | 600 | 0 |
| CA_n261(A-O-P) | - | 800 | 0 |
| CA_n261(A-O-Q) | - | 800 | 0 |
| CA_n261(2A-O) | - | 800 | 0 |
| CA_n261(A-2O) | - | 800 | 0 |
| CA_n261(A-3O) | - | 800 | 0 |
| CA_n261(A-4O) | - | 800 | 0 |
| CA_n261(A-5O) | - | 800 | 0 |
| CA_n261(A-6O) | - | 800 | 0 |
| CA_n261(A-7O) | - | 800 | 0 |
| CA_n261(A-P) | - | 700 | 0 |
| CA_n261(A-P-Q) | - | 800 | 0 |
| CA_n261(2A-P) | - | 800 | 0 |
| CA_n261(A-2P) | - | 800 | 0 |
| CA_n261(A-Q) | - | 800 | 0 |
| CA_n261(2A-Q) | - | 800 | 0 |
| CA_n261(A-2Q) | - | 800 | 0 |
| CA_n261(2A-G) | CA_n261G | 800 | 0 |
| CA_n261(2A-2G) | CA_n261G | 800 | 0 |
| CA_n261(2A-2G-O) | CA_n261G | 800 | 0 |
| CA_n261(2A-3G) | CA_n261G | 800 | 0 |
| CA_n261(2A-2O) | - | 800 | 0 |
| CA_n261(2A-3O) | - | 800 | 0 |
| CA_n261(2A-4O) | - | 800 | 0 |
| CA_n261(2A-5O) | - | 800 | 0 |
| CA_n261(2A-6O) | - | 800 | 0 |
| CA_n261(2A-H) | CA_n261G/H | 800 | 0 |
| CA_n261(2A-I) | CA_n261G/H/I | 800 | 0 |
| CA_n261(3A-G) | CA_n261G | 800 | 0 |
| CA_n261(3A-G-O) | CA_n261G | 800 | 0 |
| CA_n261(3A-2G) | CA_n261G | 800 | 0 |
| CA_n261(3A-D) | - | 800 | 0 |
| CA_n261(3A-O) | - | 800 | 0 |
| CA_n261(3A-2O) | - | 800 | 0 |
| CA_n261(3A-3O) | - | 800 | 0 |
| CA_n261(3A-4O) | - | 800 | 0 |
| CA_n261(3A-5O) | - | 800 | 0 |
| CA_n261(4A-G) | CA_n261G | 800 | 0 |
| CA_n261(4A-O) | - | 800 | 0 |
| CA_n261(4A-2O) | - | 800 | 0 |
| CA_n261(4A-3O) | - | 800 | 0 |
| CA_n261(4A-4O) | - | 800 | 0 |
| CA_n261(5A-O) | - | 800 | 0 |
| CA_n261(5A-2O) | - | 800 | 0 |
| CA_n261(5A-3O) | - | 800 | 0 |
| CA_n261(6A-O) | - | 800 | 0 |
| CA_n261(6A-2O) | - | 800 | 0 |
| CA_n261(7A-O) | - | 800 | 0 |
| CA_n261(D-G) | CA_n261D/G | 600 | 0 |
| CA_n261(D-H) | CA_n261D/H | 700 | 0 |
| CA_n261(D-I) | CA_n261D/I | 800 | 0 |
| CA_n261(D-O) | CA_n261D/O | 600 | 0 |
| CA_n261(D-2O) | - | 800 | 0 |
| CA_n261(D-P) | CA_n261D/P | 700 | 0 |
| CA_n261(D-Q) | CA_n261D/Q | 800 | 0 |
| CA_n261(E-O) | CA_n261E/O | 800 | 0 |
| CA_n261(E-P) | CA_n261E/P | 800 | 0 |
| CA_n261(E-Q) | CA_n261E/Q | 800 | 0 |
| CA_n261(G-I) | CA_n261G/H/I | 600 | 0 |
| CA_n261(G-H) | CA_n261G/H | 500 | 0 |
| CA_n261(G-J) | CA_n261A/G/H/I/J | 700 | 0 |
| CA_n261(2G-2O) | CA_n261G | 800 | 0 |
| CA_n261(G-O) | CA_n261G | 400 | 0 |
| CA_n261(G-2O) | CA_n261G | 600 | 0 |
| CA_n261(2G-O) | CA_n261G | 600 | 0 |
| CA_n261(3G-O) | CA_n261G | 800 | 0 |
| CA_n261(H-I) | CA_n261G/H/I | 700 | 0 |
| CA_n261(O-P) | - | 500 | 0 |
| CA_n261(O-Q) | - | 600 | 0 |
| CA_n261(P-Q) | - | 700 | 0 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Channel bandwidth per operating band defined in Table 5.3.5-1 NOTE 4: Configurations for intra-band contiguous CA defined in Table 5.5A.1-1 NOTE 5: Configurations for intra-band non-contiguous CA defined in Table 5.5A.2-1 NOTE 6: Void NOTE 7: Unless otherwise stated, BCS0 is referred in each constituent CA configuration. NOTE 8: (BWChannel,block) denotes the maximum total bandwidth from the summation of the sub-block bandwidths and shall be less than the bandwidth of the operating band. NOTE 9: The delimiter “/” is only used in the uplink configurations for the sake of simplicity. For example, CA_nyA/B/C denotes CA_nyA, CA_nyB and CA_nyC, where ny is a FR2 NR band and A, B and C are the corresponding bandwidth classes respectively. |  |  |  |

## << End of change >>