3GPP TSG-RAN WG4 Meeting # 108	R4-2312680

# Toulouse, France, August 21 – August 25, 2023

Agenda Item:	8.31.3.3

Source: 	OPPO

Title: 	On RRM requirements for co-channel coexistence for LTE SL and NR SL

Document for:	Approval

1	Introduction

As shown below, RAN4 reached the following agreements on co-channel coexistence between LTE sidelink and NR sidelink [1]. This contribution will provide our views on the next step about this topic.

#### Issue 2-1: Requirements for solution of TDM-based semi-static resource pool partitioning - Agreements - the existing RRM requirements can be applicable, including - interruptions to WAN in section 12.7.3 and - scheduling availability in section 12.9.1. #### Issue 2-2: Requirements for the dynamic resource sharing co-channel coexistence solution - Agreements - RAN4 to discuss whether/how to reuse or update RRM requirements for dynamic co-channel coexistence when RAN1 has completed their work. #### Issue 2-3: Requirements for solution of FDM based resource pool partitioning FFS: - Option 1 (Xiaomi)： - The discussion on potential RRM impact for FDM based resource pool partitioning solution need to be postponed until RAN1 agree to support it. #### Issue 2-4: Others FFS: - Option 1 (Nokia): - No enhancements to the in-device coexistence framework from Release 16 under the objective of sidelink co-channel coexistence.

2	Discussion

For TDM-based semi-static resource pool partitioning, it was agreed to reuse the existing RRM requirements including interruptions to WAN in section 12.7.3 and scheduling availability in section 12.9.1. Then we think no specific RRM requirements need to be introduced for this scenario.

If needed, we can discuss whether or how to capture some applicable conditions in section 12.7.3 and section 12.9.1, such as “The requirements in this section is applicable when TDM-based semi-static resource pool partitioning solution is used for co-channel coexistence between LTE sidelink and NR sidelink”

Proposal 1: No specific specification impact for solution of TDM-based semi-static resource pool partitioning. Consider whether or how to update the requirements applicability during CR discussion stage.

RAN1’s Agreement For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module may include one or more of the following parameters, to be down-selected: - Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs - SL RSRP measurement results - Resource reservation periods based on decoded SCI and for own LTE SL transmissions - Priority based on decoded SCI and for own LTE SL transmissions - Time and frequency location of resources used for own LTE SL transmissions - Candidate resource set SA or SB - SL RSSI measurements - LTE logical subframe related information - Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE Agreement For dynamic resource pool sharing, the NR SL module is expected to use the information shared by the LTE SL module to the NR SL module which is known by NR SL module at the latest T ms prior to slot n (as defined in clause 8.1.4 of TS 38.214), to determine a set of resources for its own (re)transmission. - T is defined using - T≤Tmax ms, and is based on UE implementation, according to the Rel-16 NR SL timeline for in-device coexistence. - FFS: Value of Tmax Agreement For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where - The NR SL module excludes resources based on the shared information from its own candidate resource set when performing the resource (re)selection procedure in the PHY layer. - FFS how to exclude resources at least based on the time and frequency locations of LTE SL transmissions that have been indicated in the shared candidate information. - FFS how the exclusion is performed according to clause 8.1.4 of TS 38.214. - FFS: whether/how NR SL module excludes resources not belonging to the generated LTE SL’s candidate resource set SB from its own candidate resource set. Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - FFS whether/how LTE SL RSRP is applied Agreement In NR SL resource (re)selection procedure, for dynamic resource pool sharing, the list of initial SL RSRP thresholds is separately (pre)configured (i.e., Alt 3 in the agreement of RAN1#112 meeting) for the PHY layer of NR SL module to exclude NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE. - FFS: whether a different initial SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH. Agreement In NR SL resource (re)selection procedure, for determining the LTE SL periodic reserved resources by other LTE SL UE, the formula of Q in Section 8.1.4 in TS 38.214 is used. - FFS: whether the formula of Q in Section 14.1.1.6 in TS 36.213 is used additionally to derive the largest value between the two formulas Note: The reservation period and the location of the LTE SL periodic reserved resources are determined based on the set of LTE SL logical subframe that may belong to LTE SL resource pool as specified in TS 36.213. Agreement In NR SL resource (re)selection procedure for dynamic resource pool sharing, the PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR SL module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes all NR SL candidate resources in a NR SL slot where NR SL periodic resources are in the NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR SL module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - Alt 1: The above procedure is applied at least when the priority of LTE SL transmission is higher than the priority of NR SL transmission - It is up to UE implementation whether or not to apply the above procedure when the priority of LTE SL transmission is not higher than the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module Agreement For NR SL transmissions of 30kHz SCS with dynamic resource pool sharing, the power level of the NR PSCCH/PSSCH transmission in the first of NR SL slots overlapping with an LTE SL subframe is always larger than or equal to the power level(s) of the NR PSCCH/PSSCH transmission in the subsequent NR SL slot overlapping with the LTE SL subframe. - Note: How to ensure the above condition is up to UE implementation - FFS on whether same or different frequency allocation may be used in the second overlapping slot Agreement When the same TB is transmitted on the NR SL slots overlapping with the LTE SL subframe, it is up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with an LTE SL subframe according to RAN#99’s agreement for NR PSCCH/PSSCH transmissions of 30kHz SCS with dynamic resource pool sharing Agreement When different TBs are transmitted on the NR SL slots overlapping with an LTE SL subframe and the NR SL transmission in the first overlapping NR SL slot is dropped or reselected, - Option 1: It is up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with the LTE SL subframe according to RAN#99’s agreement for NR PSCCH/PSSCH transmissions of 30kHz SCS with dynamic resource pool sharing Conclusion RAN1’s understanding is that for NR SL with dynamic resource pool sharing, overlapping in time domain between LTE SLSS/PSBCH and NR SL resource pool and overlapping in time domain between NR S-SSB and LTE SL resource pool could lead to performance degradation. Agreement RAN1 does not pursue further enhancements except for the following to handle NR SL candidate resources overlapping with LTE PSCCH for non-adjacent LTE SL resource pool in NR SL resource (re)selection procedure for dynamic resource pool sharing in Rel-18. - NR SL module uses LTE SL RSRP measurement results (as specified in TS 36.213) shared from LTE SL module to determine excluding NR SL candidate resources overlapping with LTE PSCCH and/or LTE PSSCH resources reserved by other LTE SL UE in NR SL resource (re)selection procedure for dynamic resource pool sharing in Rel-18.

For dynamic resource sharing co-channel coexistence solution or FDM-based resource pool portioning solution, RAN1 agreed that when the same or different TBs are transmitted on the NR SL slots overlapping with the LTE SL subframe, it is up to UE implementation how to avoid transmitting NR PSCCH/PSSCH only in the subsequent NR SL slot overlapping with an LTE SL subframe, with certain performance degradation.

RAN1 does not pursue further enhancements except for handling NR SL candidate resources overlapping with LTE PSCCH for non-adjacent LTE SL resource pool in NR SL resource (re)selection procedure for dynamic resource pool sharing in Rel-18.

And UE capability is agreed to introduce in R18 as below.

| Transmission using dynamic resource pool sharing | 1) Avoidance of NR PSCCH/PSSCH/PSFCH overlapping with EUTRA SL resources in dynamic resource pool sharing using LTE sidelink resource reservation information in NR mode2 resource (re)selection 2) UE supports NR PSCCH/PSSCH/ [PSFCH] TXs in 15kHz and/or 30kHz SCSs. Candidate value sets: {[15KHz, 30kHz, both]} FFS Rx capability including supported SCS |
| --- | --- |

Thus, no specific impact on RAN4 RRM requirements is identified for co-channel coexistence solution of dynamic resource sharing. Before any specific issue is identified based on RAN1 conclusion, RAN4 could postpone the discussion.

Proposal-2: No specific impact on RRM requirements is identified for co-channel coexistence solution of dynamic resource sharing.

3	Conclusion

This contribution gave our views on core requirements for co-channel coexistence between LTE sidelink and NR sidelink and the following proposals:

Proposal 1: No specific specification impact for solution of TDM-based semi-static resource pool partitioning. Consider whether or how to update the requirements applicability during CR discussion stage.

Proposal-2: No specific impact on RRM requirements is identified for co-channel coexistence solution of dynamic resource sharing.

4	Reference

- R4-2310061, WF on SL evolution RRM requirements – SL CA and co-channel co-existence, OPPO