3GPP TSG-RAN WG4 Meeting #108                                                       R4-2312718
Toulouse, France, Aug 21 – Aug 25, 2023

Agenda item:	7.32.2

Source:	Vodafone

# Title:	System parameters for NR_FDD_ULn28_DLn75

Document for:	Approval

# Introduction

The new WID on FDD band using the uplink from n28 and the downlink of n75 was approved in RAN#99 in [1] to specify the following new NR FDD frequency band:

a)	Band nA: 703 – 748 MHz UL / 1432 – 1517 MHz DL

In the last RAN4#107 meeting, the work of this new band was discussed and the WF [2] was approved. In this contribution we continue discussion on RAN4 requirements for these new NR FDD frequency band.

# Discussion

In this section we discuss channel bandwidth, subcarrier spacings, asymmetric channel BW and assigning a NR band number for the new FDD band to be specified under the new work item in [1].

## 2.1 Channel Bandwidth

The DL frequencies of the new FDD band are same as the SDL bands n75. The UL frequencies of the new FDD bands are same as the UL frequencies of the existing FDD band n28. Therefore, possible channel bandwidths and subcarrier spacings for the DL signals on these new FDD bands could be assumed to be the same as the ones for the existing NR SDL band n75 or their subsets. Similarly, for the UL signals the channel bandwidth and subcarrier options could be assumed be the same as n28. Table 1 below shows the existing channel bandwidths (CBW) and subcarrier spacing (SCS) for the NR bands n75 and n28.

Table 1: UE CBW and SCS for n75 and n28

|  | NR band / SCS / UE CBW |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n75 (1432 MHz – 1517 MHz) | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
| n28 (UL: 703 MHz – 748 MHz) | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |

From the table above we can see that channel bandwidth and subcarrier spacing options do not fully match between the DL and UL frequencies.

In the following table we have combined what channel bandwidth and subcarrier spacings would be supported for the DL or UL of these new bands if the current CBW and SCS were directly copied from the existing bands.  ‘YES’ in the table indicates that both DL and UL support the given CBW and SCS. ‘DL’ or ‘UL’ is used when only DL or UL support the given CBW and SCS.

Table 2: Combined UE CBW and SCSs for the new FDD bands based on the n75 and n28 CBW and SCS support.

|  | NR band / SCS / UE CBW |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Band nA: 1432 – 1517 MHz DL / 703 – 748 MHz UL | 15 | Yes | Yes | Yes | Yes | Yes | Yes | DL | DL |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | DL | DL |  |  |  |  |
|  | 60 |  | DL | DL | DL | DL | DL | DL | DL |  |  |  |  |

After excluding different SCS cases, the CBW and SCS table for the new FDD bands looks as follows:

Table 3: Available options for CBWs and SCSs for the new FDD band

|  | NR band / SCS / UE CBW |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Band nA: 1432 – 1517 MHz DL / 703 – 748 MHz UL | 15 | Yes | Yes | Yes | Yes | Yes | Yes | DL | DL |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | DL | DL |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |

The remaining open item is to decide if UL and DL CBW options in Table 3 should all be supported for the new FDD bands.

Proposal 1: To confirm CBW and SCSs for the new FDD bands according to the following table (‘YES’ is for UL & DL / DL is only for downlink scenario):

Table 4: CBWs and SCSs for the new FDD band

|  | NR band / SCS / UE CBW |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Band nA: 1432 – 1517 MHz DL / 703 – 748 MHz UL | 15 | Yes | Yes | Yes | Yes | Yes | Yes | DL |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | DL |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |

## Asymmetric Channel BW

The DL BW for n75 is 85 MHz. This could be sufficient to avoid DL transmission on the UL 2nd harmonic interference.

Proposal 2: In addition to the symmetric channel BW, the following asymmetric channel BW need to be supported:

| NR Operating Band | Channel BWs for UL (MHz) | Channel BWs for DL (MHz) |
| --- | --- | --- |
| Band nA: 1432 – 1517 MHz DL / 703 – 733 MHz UL | 5 | 10, 15, 20, 25, 30, 40 |
|  | 10 | 15, 20, 25, 30, 40 |
|  | 15 | 20, 25, 30, 40 |
|  | 20 | 25, 30, 40 |
|  | 25 | 30, 40 |
|  | 30 | 40 |

## 2.3 Operating band

Proposal 3: Assigning NR operating band number n106 as follows

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n106 | 703 MHz – 733 MHz | 1432 MHz – 1517 MHz | FDD |

# Conclusions

In this contribution we have discussed channel bandwidth, subcarrier spacings, asymmetric channel BW and assigning a NR band number for the following new NR FDD band:

a)	Band nA: 703 – 748 MHz UL / 1432 – 1517 MHz DL

Based on our analyses we make the following proposal:

Proposal 1: : To confirm CBW and SCSs for the new FDD bands according to the following table (‘YES’ is for UL & DL / DL is only for downlink scenario):

Table 4: CBWs and SCSs for the new FDD band

|  | NR band / SCS / UE CBW |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Band nA: 1432 – 1517 MHz DL / 703 – 748 MHz UL | 15 | Yes | Yes | Yes | Yes | Yes | Yes | DL |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | DL |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: In addition to the symmetric channel BW, the following asymmetric channel BW need to be supported:

| NR Operating Band | Channel BWs for UL (MHz) | Channel BWs for DL (MHz) |
| --- | --- | --- |
| Band nA: 1432 – 1517 MHz DL / 703 – 733 MHz UL | 5 | 10, 15, 20, 25, 30, 40 |
|  | 10 | 15, 20, 25, 30, 40 |
|  | 15 | 20, 25, 30, 40 |
|  | 20 | 25, 30, 40 |
|  | 25 | 30, 40 |
|  | 30 | 40 |

Proposal 3: Assigning NR operating band number n106 as follows:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n106 | 703 MHz – 733 MHz | 1432 MHz – 1517 MHz | FDD |

# References

- RP-230791, new FDD Bands using the uplink from n28 and the downlink of n75 and n76, RAN#99 meeting, March 2023.

- R4-2310262, “WF on RF requirements NR_FDD_ULn28_DLn75_n76”, Vodafone, 3GPP TSG RAN WG4 Meeting #107, Incheon, Korea, May 22nd – 26th, 2023.