3GPP TSG-RAN WG4 Meeting #108 	R4-2312727

Toulouse, France, August 21 – August 25, 2023

Agenda Item:	8.7.3.3

Source: 	Ericsson

Title:	On RLM and beam management

Document for:	Discussion

# Introduction

The agreements from RAN4#107 are summarized in [16] and [17] for the 1st and 2nd threads, respectively. And the discussion summaries are available in [18] and [19] for the 1st and 2nd threads, respectively.

The following was agreed for RLM and BM in RAN4#107:

- Issue 2-1-1: Beam sweeping factor for cell specific RLM and BFD/CBD for multi-Rx

  - Faster beam sweeping is not applicable for CSI-RS based RLM and BFD measurements, due to N=1 in the existing requirements.

RAN4#106bis-e:

- Cell specific RLM and BFD/CBD

  - No further discussion on TRP specific RLM as there is no such procedure in existing specification..

- Issue 2-1-2: Number of samples

  - For RLM with multi-Rx chain operation, the same number of samples Mout and Min are used as in Rel-17.

  - For link recovery with multi-Rx chain operation, the same number of samples MBFD and MCBD are used as in Rel-17.

- Issue 2-1-3: PDCCH configuration

  - For RLM with multi-Rx chain operation, the same PDCCH transmission configuration is used as in Rel-17.

  - For link recovery with multi-Rx chain operation, the same PDCCH transmission configuration is used as in Rel-17.

RAN4#106:

- Baseline L1 measurement requirements for potential multi-Rx enhancement

  - For cell specific RLM: section 8.1

  - For cell specific link recovery: section 8.5

  - For TRP specific link recovery: section 8.18

  - For L1-RSRP measurement: section 9.5.

RAN4#105:

- Overall handling of L1 measurements

  - Discussion on Rx beam sweeping scaling factor, simultaneous reception of DL data and RS, and other multi-Rx related topics can be done jointly for L1-RSRP, RLM, L1-SINR and CBD/BFD.

  - Note: issues specific to each of the above measurements can be discussed separately.

# Discussion

A draft CR on RLM for multi-rx UE is provided in [20], based on the below proposals.

## RLM

Given that N=1 for CSI-RS based RLM, no faster beam sweeping can be specified for multi-rx UEs in RLM requirements for CSI-RS.

So, RAN4 can focus on measurement restrictions enhancements for RLM in sections 8.1.2.3 and 8.1.3.3 and on and scheduling availability in section 8.1.7.3, limited to FR2-1 as earlier agreed by RAN4.

- Proposal 1: RLM enhancements for measurement restrictions in FR2-1 are captured in sections 8.1.2.3 and 8.1.3.3.

- Proposal 2: RLM enhancements for scheduling availability in FR2-1 are captured in section 8.1.7.3.

## BFD

Given that N=1 for CSI-RS based RLM, no faster beam sweeping can be specified for multi-rx UEs in BFD requirements for CSI-RS.

For cell-specific BFD, RAN4 can focus on measurement restrictions enhancements for BFD in sections 8.5.2.3 and 8.5.3.3 and on scheduling availability enhancements for BFD in section 8.5.7.3 (Scheduling availability of UE performing beam failure detection on FR2).

- Proposal 3: Cell-specific BFD enhancements for measurement restrictions in FR2-1 are captured in sections 8.5.2.3 and 8.5.3.3.

- Proposal 4: Cell-specific BFD enhancements for scheduling availability in FR2-1 are captured in section 8.5.7.3.

For TRP-specific BFD, the evaluation period can be enhanced by setting PTRP=1 for UEs capable of multi-rx operation, in addition to measurement restrictions and scheduling availability enhancements similar to cell-specific BFD.

- Proposal 5: For TRP-specific BFD, the evaluation period is enhanced by setting PTRP=1.

- Proposal 6: TRP-specific BFD enhancements for measurement restrictions in FR2-1 are captured in sections 8.18.2.3 and 8.18.3.3.

- Proposal 7: TRP-specific BFD enhancements for scheduling availability in FR2-1 are captured in section 8.18.8.3.

## CBD

For cell-specific CBD, RAN4 can focus on measurement restrictions enhancements for CBD in sections 8.5.5.3 and 8.5.6.3 and on scheduling availability enhancements for CBD in section 8.5.8.3.

- Proposal 8: Cell-specific BFD enhancements for measurement restrictions in FR2-1 are captured in sections 8.5.5.3 and 8.5.6.3.

- Proposal 9: Cell-specific BFD enhancements for scheduling availability in FR2-1 are captured in section 8.5.8.3.

For TRP-specific CBD, the evaluation period can be enhanced by setting PTRP=1 for UEs capable of multi-rx operation, in addition to measurement restrictions and scheduling availability enhancements similar to cell-specific CBD.

- Proposal 10: For TRP-specific CBD, the evaluation period is enhanced by setting PTRP=1.

- Proposal 11: TRP-specific CBD enhancements for measurement restrictions in FR2-1 are captured in sections 8.18.5.3 and 8.18.6.3.

- Proposal 12: TRP-specific CBD enhancements for scheduling availability in FR2-1 are captured in section 8.18.9.3.

# Summary

The following have been proposed in the current contribution:

- Proposal 1: RLM enhancements for measurement restrictions in FR2-1 are captured in sections 8.1.2.3 and 8.1.3.3.

- Proposal 2: RLM enhancements for scheduling availability in FR2-1 are captured in section 8.1.7.3.

- Proposal 3: Cell-specific BFD enhancements for measurement restrictions in FR2-1 are captured in sections 8.5.2.3 and 8.5.3.3.

- Proposal 4: Cell-specific BFD enhancements for scheduling availability in FR2-1 are captured in section 8.5.7.3.

- Proposal 5: For TRP-specific BFD, the evaluation period is enhanced by setting PTRP=1.

- Proposal 6: TRP-specific BFD enhancements for measurement restrictions in FR2-1 are captured in sections 8.18.2.3 and 8.18.3.3.

- Proposal 7: TRP-specific BFD enhancements for scheduling availability in FR2-1 are captured in section 8.18.8.3.

- Proposal 8: Cell-specific BFD enhancements for measurement restrictions in FR2-1 are captured in sections 8.5.5.3 and 8.5.6.3.

- Proposal 9: Cell-specific BFD enhancements for scheduling availability in FR2-1 are captured in section 8.5.8.3.

- Proposal 10: For TRP-specific CBD, the evaluation period is enhanced by setting PTRP=1.

- Proposal 11: TRP-specific CBD enhancements for measurement restrictions in FR2-1 are captured in sections 8.18.5.3 and 8.18.6.3.

- Proposal 12: TRP-specific CBD enhancements for scheduling availability in FR2-1 are captured in section 8.18.9.3.

Based on the proposals, a draft CR on RLM for multi-rx UE is provided in [20].

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] R4-2214344, WF on FR2 multi-Rx RRM, vivo, Aug. 2022.

[3] R4-2217246, WF on RRM impacts and general aspects for multi-Rx, vivo, Oct. 2022.

[4] R4-2217587, WF on L1 measurements, beam sweeping factors and simultaneous reception, Qualcomm, Oct. 2022.

[5] R4-2217248, WF on TCI state switching for multi-RX chain DL reception, Ericsson, Oct. 2022.

[6] R4-2220742, WF on NR FR2 multi-Rx chain DL reception – Part 1, Nov. 2022.

[7] R4-2220743, WF on NR FR2 multi-Rx chain DL reception – Part 2, Nov. 2022.

[8] R4-2220402, WF on NR FR2 multi-Rx chain DL reception – Part 3, Nov. 2022.

[9] R4-2303193, Ad-hoc minutes for NR FR2 multi-Rx chain DL reception RRM, vivo, March 2023.

[10] R4-2303302, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, March 2023.

[11] R4-2303317, WF on NR FR2 multi-Rx chain DL reception RRM requirements(part2), Qualcomm, March 2023.

[12] R4-2306318, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, Apr. 2023.

[13] R4- 2306319, WF on FR2_multiRx_part2, Ericsson, Apr. 2023.

[14] R4-2306392, Topic summary for [106bis-e][202] FR2_multiRx_part1, vivo, Apr. 2023.

[15] R4-2306234, Topic summary for [106bis-e][203] FR2_multiRx_part2, Ericsson, Apr. 2023.

[16] R4-2310046, WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), vivo, May 2023.

[17] R4-2310047, WF on FR2_multiRx_part2, Ericsson, May 2023.

[18] R4-2309952, Topic summary for [106][209] FR2_multiRx _part1, vivo, May 2023.

[19] R4-2309953, Topic summary for [107][208] FR2_multiRx_part2, Ericsson, May 2023.

[20] R4-2312728, RLM requirements for UE with multi-rx chain in FR2, draft CR to TS 38.133, Ericsson, Aug. 2023.