3GPP TSG-RAN WG4 Meeting # 108 	    R4-2312738

Toulouse, France, 21st – 25th August 2023

Agenda Item:	8.22.3.1

Source: 	Ericsson

Title:	Response to LS on LPHAP

Document for:	Approval

# Introduction

RAN2#122 agreed on LS on LPHAP [1]. The LS discusses agreement in eRedCap WI on eDRX cycle values in RRC_INACTIVE state and seeks confirmation from RAN1/RAN4 on whether the agreed eDRX cycle length values meet LPHAP use case requirement defined in TS 22.104 [2]. In this contribution we share our view on eRedCap WI agreed eDRX cycle values and propose a draft of RAN4 response to RAN2 LS in annex.

# Discussion

LPHAP feature mainly focuses on meeting the requirements of use case 6 as defined in TS 22.104. Table A.7.2-1 in TS 22.104 highlights the requirements for use case 6 (copied below for reference).

Table A.7.2-1: Low power high accuracy positioning use cases

| Use Case # | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- |
| 1 | 10 m | Service Level 1 | on request | 24 months |
| 2 | 2 m to 3 m | Service Level 2 | < 4 seconds | > 6 months |
| 3 | < 1 m | Service Level 3 | no indication | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |
| 4 | < 1 m | Service Level 3 | 1 second | 6 - 8 years |
| 5 | < 1 m | Service Level 3 | 5 seconds - 15 minutes | 18 months |
| 6 | < 1 m | Service Level 3 | 15 s to 30 s | 6 - 12 months |
| 7 | 30 cm | Service Level 5 | 250 ms | 18 months |
| 8 | 30 cm | Service Level 5 | 1 second | 6 - 8 years (no strong limitation in battery size) |
| 9 | 30 cm | Service Level 1 | 20 minutes | 12 years (@20mJ/position fix) |

From the table above following observations can be made:

Observation 1: positioning interval/duty cycle for use case 6 is 15 seconds to 30 seconds.

Observation 2: Battery lifetime at the device performing positioning measurement should be between 6 months to 12 months.

To meet these requirements, rel. 18 WI identifies LPHAP as one of the positioning features. Length of eDRX cycle to meet the target device battery lifetime was evaluated during the rel. 18 SI. It was then concluded that eDRX cycle longer than 10.24 seconds in RRC_INACTIVE state is required to meet the battery lifetime target.

eRedCap WI agreed to introduce eDRX cycle lengths of {2, 4, 8, 16, 32, 64, 128, 256, 512, and 1024} hyperframes. Duration of 1 hyperframe is 10.24 seconds. Based on the agreed eDRX cycle length UE in RRC_INACTIVE state can now be configured with eDRX cycles longer than 10.24 seconds, which is fundamental to meet the target device battery lifetime to support LPHAP use case in TS 22.104.

Observation 3: Agreed eDRX cycle lengths allow eDRX cycle longer than 10.24 seconds to be configured to UE in RRC_INACTIVE state.

The agreed eDRX cycle values are also suitable to meet positioning interval requirement of LPHAP use case. The LPHAP use case in Table A.7.2-1 of TS 22.104 require UE to perform positioning measurements every 15 seconds to 30 seconds. This requirement can also be met when UE is configured with an eDRX cycle of 2 hyperframes that results into positioning interval of 20.48 seconds.

Observation 4: Agreed eDRX cycle lengths are suitable to meet positioning interval requirement of LPHAP use case.

In our view eRedCap WI agreed eDRX cycles values are well suited to meet LPHAP requirements and RAN4 should respond to RAN2 confirming the suitability of eDRX cycles for LPHAP use cases. To facilitate discussion of RAN4 response to RAN2 LS a draft of reply is provided in the annex of this contribution.

Proposal 1: RAN4 to confirm suitability of eRedCap WI agreed eDRX values to meet LPHAP use case 6 requirements.

# Summary

In this contribution we presented our view on eRedCap WI agreed eDRX cycle values for RRC_INACTIVE state. Following are the observations and proposals:

Observation 1: positioning interval/duty cycle for use case 6 is 15 seconds to 30 seconds.

Observation 2: Battery lifetime at the device performing positioning measurement should be between 6 months to 12 months.

Observation 3: Agreed eDRX cycle lengths allow eDRX cycle longer than 10.24 seconds to be configured to UE in RRC_INACTIVE state.

Observation 4: Agreed eDRX cycle lengths are suitable to meet positioning interval requirement of LPHAP use case.

Proposal 1: RAN4 to confirm suitability of eRedCap WI agreed eDRX values to meet LPHAP use case 6 requirements.

# References

- R2-2306841, LS on LPHAP, Huawei.

- Rel. 19, Service requirements for cyber-physical control applications in vertical domains, 3GPP.

# Draft of reply LS

3GPP TSG-RAN WG4 Meeting #108		R4-23xxxxx

Toulouse, France, 21 – 25 August 2023

Title:	Response to LS on LPHAP

Response to:	R2-2306841

Release:	Rel-18

Work Item:	NR_pos_enh2-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact Person:

Name:	Deep Shrestha

E-mail Address:	deep.shrestha@ericsson.com

Attachments:	N/A

1. Overall Description:

RAN4 thanks RAN2 for LS on LPHAP R2-2306841.

RAN4 discussed the suitability of eRedCap WI agreed eDRX values for meeting the LPHAP requirements. RAN4 would like to confirm that the agreed eDRX cycle values for RRC_INACTIVE state are sufficient to meet LPHAP use case 6 requirements (UE target battery lifetime and positioning interval) in RRC_INACTIVE state.

2. Actions:

To RAN WG2 group.

ACTION: 	RAN4 kindly requests RAN2 to take the above confirmation into consideration.

3. References:

- R2-2306841, LS on LPHAP, Huawei.

- Rel. 19, Service requirements for cyber-physical control applications in vertical domains, 3GPP.

4. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #108bis			09th – 13th October 2023				Xiamen, CN

TSG RAN WG4 Meeting #109			13th – 17th November 2023				Chicago, US