3GPP TSG-RAN WG4 Meeting #108                                           R4-2312753

Toulouse, France, August 21 – August 25, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on Sidelink on a single unlicensed spectrum

Agenda Item:	8.30.2.1

Document for:	Approval

Introduction

The WID revision [1] was approved in RAN#99e with the updated scope for the objective of sidelink CA operation in NR sidelink evolution.

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] - Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, power control for simultaneous sidelink TX, packet duplication) - The work is limited to intra-band CA for the ITS band in FR1 (Band n47). - No specific enhancements of Rel-17 sidelink features with sidelink CA support. - This feature is backwards compatible in the following regards - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI) - Only Mode 2 operation - Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues - Time resources for PSFCH are aligned among the carriers for CA - No enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation) - SL resource indication remains to be per-resource pool and per-carrier basis (no cross-carrier scheduling in SCI) - UE transmits SL HARQ feedback on the same carrier on which it receives the associated PSSCH - No consideration for limited transmission and reception capability - No primary/secondary carrier differentiation - Reuse the LTE sidelink CA design for the following aspects: - Sidelink carrier (re-)selection, synchronization of aggregated carriers, Tx power split for simultaneous sidelink transmissions, packet duplication - The CA band combination work in RAN4 is limited to intra-band contiguous CA in Rel-18. - Note: The SL CA work in Rel-18 mainly targets some V2X use cases

The TP on the MPR/A-MPR simulation assumptions for SL-U was approved in [2]. In this contribution, we provide our initial simulation evaluations on MPR for SL-U based on the agreed assumptions.

Discussion

Simulation assumptions

Table 1 Evaluation assumptions on SL-U

| Frequency band | n46 |
| --- | --- |
| Bandwidth | 20/40/60/80/100MHz |
| Maximum output power | Power class 5: 20dBm |
| numerology | 15 kHz/30kHz |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM for SL-U |
| PSCCH/PSSCH multiplexing |  |
| PSCCH size | 10RB*3 Symbols |
| CIM3 | 60dBc |
| PSD | 10 dBm/MHz |
| Resource allocation | Full allocation/ Interlace-based allocation/ wideband operation |
| PA calibration | PA calibrated to deliver -30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Analysis and proposal

Referring to the simulation cases listed in Table 2, the MPR value are shown in Table 3 and Table 4 for below.

Table 2 Simulation cases for SL-U

|  | case | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 15 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 4 | CP-OFDM | 60 | 160RB0 | 30 |
|  | 5 | CP-OFDM | 80 | 216RB0 | 30 |
|  | 6 | CP-OFDM | 100 | 270RB0 | 30 |
| Interlaced Allocation Single CC | 7 | CP-OFDM | 20 | 1 RB0 every 10RBs | 15 |
|  | 8 | CP-OFDM | 20 | 1RB0 every 5RBs | 30 |
|  | 9 | CP-OFDM | 40 | 1RB0 every 10RBs | 15 |
|  | 10 | CP-OFDM | 60 | 1RB0 every 5RBs | 30 |
|  | 11 | CP-OFDM | 80 | 1RB0 every 5RBs | 30 |
|  | 12 | CP-OFDM | 100 | 1RB0 every 5RBs | 30 |
| Wide band operation | 13 | CP-OFDM | 40 | Bitmap 10 | 15 |
|  | 14 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 15 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 16 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 17 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 18 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 19 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 20 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 21 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 22 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 23 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 24 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 25 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 26 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 27 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 28 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Table 3 Maximum power reduction (MPR) for SL-U with Full Allocation

| Modulation | Case: Bandwidth/SCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz/15k | 20MHz/30k | 40MHz/15k | 60MHz/30k | 80MHz/30k | 100MHz/30k |
| QPSK | 4.0 | 3.8 | 4.1 | 4.0 | 4.0 | 4.1 |
| 16 QAM | 4.1 | 3.9 | 4.1 | 4.1 | 3.8 | 4.2 |
| 64 QAM | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 |
| 256 QAM | 7.5 | 7.7 | 7.6 | 7.8 | 7.5 | 7.9 |

Table 4 Maximum power reduction (MPR) for SL-U with Interlaced Allocation

| Modulation | Case: Bandwidth/SCS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz/15k | 20MHz/30k | 40MHz/15k | 60MHz/30k | 80MHz/30k | 100MHz/30k |
| QPSK | 4.0 | 4.1 | 4.0 | 4.1 | 4.1 | 4.1 |
| 16 QAM | 4.1 | 4.2 | 4.2 | 4.1 | 4.2 | 4.2 |
| 64 QAM | 5.8 | 5.9 | 5.7 | 5.6 | 5.9 | 6.1 |
| 256 QAM | 7.0 | 7.3 | 7.1 | 7.4 | 7.7 | 7.4 |

Table 4 Maximum power reduction (MPR) for SL-U with Interlaced Allocation

| Modulation | Case: Bandwidth/Bitmap pattern |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 40MHz | 60MHz |  | 80MHz |  |  | 100MHz |  |  |  |  |  |
|  | 10 | 100 | 110 | 1000 | 1110 | 0110 | 10000 | 11110 | 01000 | 01100 | 01110 | 00100 |
| QPSK | 4.0 | 3.7 | 3.9 | 3.8 | 4.0 | 3.8 | 3.9 | 4.1 | 3.0 | 3.2 | 3.9 | 3.4 |
| 16 QAM | 4.0 | 3.9 | 4.1 | 3.9 | 4.1 | 3.9 | 3.9 | 4.2 | 3.7 | 3.8 | 4.0 | 3.7 |
| 64 QAM | 5.1 | 5.1 | 5.5 | 5.2 | 5.7 | 5.7 | 5.1 | 5.8 | 5.4 | 6.0 | 5.9 | 5.5 |
| 256 QAM | 6.4 | 6.8 | 6.9 | 6.3 | 7.0 | 7.1 | 6.5 | 7.3 | 6.7 | 7.4 | 7.3 | 7.0 |

Proposal 1: The proposed MPR values for SL-U with single CC is as below

Table 3: Proposed MPR for SL-U with single CC

| Precoding | Modulation | Full | Partial |
| --- | --- | --- | --- |
| CP-OFDM | QPSK | ≤ 4.1 | ≤ 4.1 |
|  | 16 QAM | ≤ 4.2 | ≤ 4.2 |
|  | 64 QAM | ≤ 6.1 | ≤ 6.1 |
|  | 256 QAM | ≤ 8.0 | ≤ 7.7 |

Conclusion

This contribution provides our initial evaluation results on MPR for SL-U with singl CC:

Proposal 1: The proposed MPR values for SL-U with single CC is as below

Table 3: Proposed MPR for SL-U with single CC

| Precoding | Modulation | Full | Partial |
| --- | --- | --- | --- |
| CP-OFDM | QPSK | ≤ 4.1 | ≤ 4.1 |
|  | 16 QAM | ≤ 4.2 | ≤ 4.2 |
|  | 64 QAM | ≤ 6.1 | ≤ 6.1 |
|  | 256 QAM | ≤ 8.0 | ≤ 7.7 |

- References

[1] RP-230077 WID revision: NR sidelink evolution, OPPO, TSG RAN Meeting #99-e, March. 20-23, 2023

[2] R4-2310378 TP for TR 38.786 on the MPR/A-MPR simulation assumptions, Facebook Japan K.K. TSG RAN4 Meeting #107, May. 22-27, 2023