3GPP TSG-RAN WG4 Meeting #108                                           R4-2312754

Toulouse, France, August 21 – August 25, 2023

Source: 	Huawei, HiSilicon

Title: 	On sidelink CA

Agenda Item:	8.30.2.3

Document for:	Approval

Introduction

The WID revision [1] was approved in RAN#99e with the updated scope for the objective of sidelink CA operation in NR sidelink evolution.

- Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] - Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, power control for simultaneous sidelink TX, packet duplication) - The work is limited to intra-band CA for the ITS band in FR1 (Band n47). - No specific enhancements of Rel-17 sidelink features with sidelink CA support. - This feature is backwards compatible in the following regards - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI) - Only Mode 2 operation - Same subcarrier spacing (SCS) among CA carriers to avoid resource selection enhancements and AGC issues - Time resources for PSFCH are aligned among the carriers for CA - No enhancement related to SCI transmissions on PSCCH/PSSCH, PSFCH transmission, RSRP feedback, CSI feedback and congestion control compared to Rel-16 (i.e., per-carrier operation) - SL resource indication remains to be per-resource pool and per-carrier basis (no cross-carrier scheduling in SCI) - UE transmits SL HARQ feedback on the same carrier on which it receives the associated PSSCH - No consideration for limited transmission and reception capability - No primary/secondary carrier differentiation - Reuse the LTE sidelink CA design for the following aspects: - Sidelink carrier (re-)selection, synchronization of aggregated carriers, Tx power split for simultaneous sidelink transmissions, packet duplication - The CA band combination work in RAN4 is limited to intra-band contiguous CA in Rel-18. - Note: The SL CA work in Rel-18 mainly targets some V2X use cases

In this contribution, we provide our consideration and initial analysis on RAN4 work for NR sidelink CA operation.

Discussion

Channel raster for SL CA

For the channel raster in system parameters for NR-V, the reference frequency can be shifted by configurations to be orthogonal with LTE-V.

FREF_V2X = FREF + Δshift + N * 5 kHz

where

Δshift = 0 kHz or 7.5 kHz indicated in IE (frequencyShift7p5khz), and

N can be set as one of following values {-1, 0, 1}, which are signalled by the network in higher layer parameters or configured by pre-configuration parameters.

If both the two carriers in SL CA are to be orthogonal with LTE-V, more restriction on the channel spacing is required based on 15kHz raster in n47.

Proposal 1: The restrictions on the channel spacing is required for SL CA, to maintain the orthogonality for the intra-band two carriers with LTE-V.

Tx Requirements for sidelink intra-band CA

Given that Tx and Rx requirements have been introduced for LTE V2X multi-carrier in Rel-15, some requirements can be reused in Rel-18 sidelink CA, but some of them are different due to NR design.

In the approved WF R4-2310314 in RAN4#107, the Tx requirements for SL CA were captured

## Sub-topic 2-2: Tx requirements for SL CA < GTW Agreement>: UE maximum output power for SL CA - Only support PC3 in Rel-18 < Way Forward>: FFS in the next meeting on methodology of Tx requirements for SL intra-band contiguous CA Candidate options: - Option 1: The requirements for NR intra-band contiguous CA as baseline. - Option 2: The requirements for LTE intra-band contiguous CA as baseline. - Option 3: Study the Tx requirements case by case < Way Forward>: Transmit ON/OFF time mask for SL con-current operation with CA - RAN4 can consider the switching between SL CA and LTE-V in Rel-18 - FFS: The optimization for symbol-level switching time similar to that of Tx switching for SL CA and uplink transmission. < Way Forward>: PSFCH for SL CA - Non-contiguous RB allocation of PSFCH for intra-band contiguous SL CA and the restriction on the number of PSFCH is for further study.

Configured transmitted power for sidelink intra-band CA

For the configured transmitted power, the standardization for SL CA can refer to that in NR or LTE intra-band contiguous CA as baseline. The main difference between LTE and NR lies in PEMAX,CA was introduced in NR.

The total configured maximum output power PCMAX shall be set within the following bounds for bothe LTE and NR:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

In LTE,

For uplink intra-band contiguous and non-contiguous carrier aggregation when same TTI pattern is used in all aggregated serving cells, PCMAX_L = MIN{10 log10 ∑ pEMAX,c - TC , (PPowerClass – ΔPPowerClass) – MAX(MPR + A-MPR + ΔTIB,c + TC + TProSe, P-MPR ) } PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PPowerClass}

For NR,

For uplink intra-band contiguous carrier aggregation when same slot pattern is used in all aggregated serving cells, PCMAX_L = MIN{10 log10 ∑ pEMAX,c - TC , PEMAX,CA,(PPowerClass,CA– ΔPPowerClass,CA) – MAX(MAX(MPR, A-MPR) + ΔTIB,c + TC +TRxSRS, P-MPRc ) } PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA ,PPowerClass,CA– ΔPPowerClass,CA }

NR also specified the configured transmitted power for intra-band contiguous CA when one different numerology/slot pattern is used, which is not applied to SL CA considering the SCS between the cells is same as agreed in previous meeting. The configured maximum output power defined for NR is a little complicated than that for LTE. According to the objective for SL CA in the WID, support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, power control for simultaneous sidelink TX, packet duplication). And we see no other different power control scheme for SL CA from LTE. Thus, to simplify the standardization of SL CA, the configured transmitted power for LTE intra-band contiguous CA can be taken as baseline.

Observation 1: According to the objective for SL CA in the WID, support only LTE sidelink CA features for NR (…, power control for simultaneous sidelink TX).

Proposal 2: To simplify the standardization of SL CA, the configured transmitted power for LTE intra-band contiguous CA can be taken as baseline.

Transmit ON/OFF time mask for SL con-current operation with CA

Rel-17 specifies the time mask for switching between Uu and SL for TDM operation in same carrier case with same bandwidth in Figure 1, and the time mask for switching between Uu and SL for different carrier case. Both the time masks are considered for single carrier operation in Figure 2. In Rel-18, the switching between SL CA and LTE-V should be considered.

Figure 1 Time mask for switching between Uu and SL for same carrier case with same bandwidth

Figure 2 Time mask for switching between Uu and SL for different carrier case

Proposal 3: The switching between SL CA and LTE-V should be considered in Rel-18.

For the switching time for switching between Uu and SL for different carrier in RRM specification TS38.133, it is scheduling restriction with the granularity of slot. In other words, if the switching between V2X sidelink transmission and uplink transmission is executed in a slot, the resource in the whole slot cannot be used for any transmission.

- 12.9.2 Scheduling availability of UE switching between uplink transmission and V2X sidelink transmission This clause contains the restrictions on the scheduling availability for V2X sidelink due to switching between uplink transmission and V2X sidelink transmission. For NR V2X sidelink, the assumed number of configured symbols in a slot is 14. When switch from uplink transmission to V2X sidelink transmission occurs in sidelink slot ‘n’, - UE is not expected to transmit or receive on V2X sidelink on the sidelink slot ‘n’. When switch from V2X sidelink transmission to uplink transmission occurs in sidelink slot ‘n-1’, - UE is not expected to transmit or receive on V2X sidelink on the sidelink slot ‘n-1’. When switch from V2X sidelink transmission to uplink transmission occurs in Uu slot ‘n’, - UE is not expected to transmit uplink or receive downlink on the Uu slot ‘n’. When switch from uplink transmission to V2X sidelink transmission occurs in Uu slot ‘n-1’, - UE is not expected to transmit uplink or receive downlink on the Uu slot ‘n-1’.

In the scope of Rel-18 WID, the SL CA is limited to only Mode 2 operation. Thus, it may be required to specify the switching time under the restriction. In addition, the optimization for symbol-level switching time similar to that of Tx switching for SL CA and uplink transmission can be considered. Whether the enhancement on the granularity for switching time has performance gain is up to RAN1’s evaluation.

Proposal 4: The optimization for symbol-level switching time similar to that of Tx switching for SL CA and uplink transmission can be considered.

PSFCH for SL CA

PSFCH with single carrier supports non-contiguous RB allocation. The non-contiguous RB can also be considered in SL CA, and it may result in power fallback. And the restriction on the number of PSFCH due to MPR is up to the RF simulation results.

Proposal 5: Non-contiguous RB allocation of PSFCH for intra-band contiguous SL CA and the restriction on the number of PSFCH can be considered.

Rx Requirements for sidelink intra-band CA

For SL CA operation, requirements for LTE V2X multi-carrier operation should be referred. For SL_n47B, the reference sensitivity defined for single carrier operation in n47 as shown in Table 7.3E.2-1 shall apply for each component carrier with two CCs are activated at the same time

Table 1 UE Rx RF requirement for Rel-15 V2X multi-carrier operation

Receiver characteristics

REFSENS

Maximum input level

Adjacent Channel Selectivity (ACS)

In-band blocking

Out-of-band blocking

Spurious response

Intermodulation characteristics

RX spurious emissions

Receiver image

Conclusion

This contribution provides our consideration and initial analysis on RAN4 work for Rel-18 SL evolution with the following observation and proposals:

Observation 1: According to the objective for SL CA in the WID, support only LTE sidelink CA features for NR (…, power control for simultaneous sidelink TX).

Proposal 1: The restrictions on the channel spacing is required for SL CA, to maintain the orthogonality for the intra-band two carriers with LTE-V.

Proposal 2: To simplify the standardization of SL CA, the configured transmitted power for LTE intra-band contiguous CA can be taken as baseline.

Proposal 3: The switching between SL CA and LTE-V should be considered in Rel-18.

Proposal 4: The optimization for symbol-level switching time similar to that of Tx switching for SL CA and uplink transmission can be considered.

Proposal 5: Non-contiguous RB allocation of PSFCH for intra-band contiguous SL CA and the restriction on the number of PSFCH can be considered.

- References

[1] RP-230077 WID revision: NR sidelink evolution, OPPO, TSG RAN Meeting #99-e, March. 20-23, 2023