3GPP TSG-RAN WG4 Meeting #108	R4-2312788

Toulouse, France, 21 August – 25 August 2023

Agenda Item:	8.11.4

Source:	Ericsson

Title:	UE demodulation requirements for non-colocated NR-CA deployment scenario

Document for:	Discussion

# 1	Introduction

RAN4#107 agreed with the way forward on the UE demodulation requirements for intra-band non-colocated EN-DC/NR-CA deployments [1], where RAN4 agreed to define new Type 2 UE demodulation requirements with NR-CA assuming received power difference of 25dB and received time difference of 33us between two carriers.

This contribution discusses the detailed test setup for PDSCH demodulation requirements for Type 2 UE.

# 2	Discussion

RAN4#107 agreed to define PDSCH demodulation requirements to verify the feature of Type 2 UE as follows [1].

Test setup for Type 2 UE demodulation requirements Agreement: - Assume the power difference of 25dB and received time difference of 33us under the assumption that requirements introduced under static channel. Test method for Type 2 UE demodulation requirements Way forward: - For NR-CA measure both PCell and SCell at same time. - Channel model: AWGN - FFS for Test criteria: Reuse X% of the maximum throughput - FFS for How to select MCS for each carrier - Interested companies are encouraged to propose test methods of UE demodulation requirements for NR-CA to verify Type 2 UE features.

PDSCH demodulation requirements in TS38.101-4 are basically defined by SNR to achieve X% of the theoretical throughput with the fixed reference channel (FRC) model under the fading channel condition, where RAN4 usually sets X=70% considering the alignments of the contributing companies simulation results.

On the other hand, PDSCH demodulation requirements for Type 2 UE with non-colocated deployment scenario needs to consider the following purposes according to WF [1]:

- Verify the PDSCH reception performance (i.e., throughputs) from two carriers simultaneously.

- Apply the receive power difference of 25dB between two carriers.

- Apply the receive time difference of 33us between two carriers.

For these purposes, we need to set different MCS indexes for each carrier so that the required SNR test point difference between two carriers is around 25dB. To find out the feasible MCS indexes and test metrics, we ran the PDSCH simulations based on the parameters shown in Table 1, where we investigated 64QAM MCS table (MCS index table 1) and 256QAM MCS table (MCS index table 2). Since the channel model is static according to WF, we do not enable HARQ retransmission.

Table 1	PDSCH simulation parameters

| Parameters | Values |
| --- | --- |
| Carrier frequency | 3.5 GHz (Bands n77/n78) |
| SCS/CBW | 30kHz / 40MHz |
| Antenna configuration | 2x2 |
| Channel model | Static condition (TS38.101-4 B.1.1) |
| TDD Pattern | 7DS2U, S=6D4G4U No PDSCH schedule in special slots |
| Rank | 2 |
| Number of additional DMRS | 1 |
| Maximum number of HARQ transmission | 1 |
| MCS tables | Case 1: 64QAM (MCS index table 1) Case 2: 256QAM (MCS index table 2) |

| (a) MCS index table 1 (64QAM) | (b) MCS index table 2 (256QAM) |
| --- | --- |

Figure 1	PDSCH simulation results.

Figure 1 shows the PDSCH simulation results per MCS index for each MCS index table. From the simulation results, it is observed 64QAM MCS table does not have enough dynamic range covering the received power difference of 25dB. For example, even if we choose MCS0 and MCS26, the SNR difference with 70% of peak rate is about 23dB. Considering the REFSENS is set around SNR=-1dB, we would like to avoid setting lower MCS indexes such as 0, 1, or 2 because the required SNR is less than -3dB.

Observation 1: 64QAM MCS table (MCS index table 1) is not suitable for verifying the receive power difference of 25dB.

On the other hand, we observe 256QAM MCS table has enough dynamic range to cover the received power difference of 25dB. For example, if we consider the SNR test points to achieve 70% of peak rate, SNR=-1.1dB for MCS2 (QPSK CR=0.30) and SNR=+23.4dB for MCS24 (256QAM CR=0.82). Since we don’t enable HARQ retransmission and use static channel model, we don’t see significant SNR difference if we set 80% or 90% of peak rate.

Proposal 1: Use 256QAM MCS table (MCS index table 2) for Type 2 UE PDSCH demodulation requirements for non-colocated NR-CA scenario.

Proposal 2: Define the PDSCH demodulation requirements for non-colocated NR-CA deployment scenario as follows.

- For Type 2 UE, the throughput shall be ≥ [70] % of the maximum throughput with the fixed reference channels at the specified SNR test points.

- Decide MCS indexes for both carriers according to the simulation results, e.g., set [MCS 24] for PCell and [MCS 2] for SCell.

| Cell type | CBW/SCS | MCS, rank (MCS index table 2) | Propagation channel and antenna correlation | Fraction of maximum throughput | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| SCell | 40MHz/30kHz | [MCS 2, rank 2] | 2x2 static condition (TS38.101-4 B.1.1) | [70%] | TBD |
| PCell | 40MHz/30kHz | [MCS 24, rank 2] | 2x2 static condition (TS38.101-4 B.1.1) | [70%] | TBD + 25dB |
| Note 1 33 µsec timing difference between two CCs is applied. Note 2 No HARQ retransmission |  |  |  |  |  |

# 3	Summary

Observation 1: 64QAM MCS table (MCS index table 1) is not suitable for verifying the receive power difference of 25dB.

Proposal 1: Use 256QAM MCS table (MCS index table 2) for Type 2 UE PDSCH demodulation requirements for non-colocated NR-CA scenario.

Proposal 2: Define the PDSCH demodulation requirements for non-colocated NR-CA deployment scenario as follows.

- For Type 2 UE, the throughput shall be ≥ [70] % of the maximum throughput with the fixed reference channels at the specified SNR test points.

- Decide MCS indexes for both carriers according to the simulation results, e.g., set [MCS 24] for PCell and [MCS 2] for SCell.

| Cell type | CBW/SCS | MCS, rank (MCS index table 2) | Propagation channel and antenna correlation | Fraction of maximum throughput | SNR (dB) |
| --- | --- | --- | --- | --- | --- |
| SCell | 40MHz/30kHz | [MCS 2, rank 2] | 2x2 static condition (TS38.101-4 B.1.1) | [70%] | TBD |
| PCell | 40MHz/30kHz | [MCS 24, rank 2] | 2x2 static condition (TS38.101-4 B.1.1) | [70%] | TBD + 25dB |
| Note 1 33 µsec timing difference between two CCs is applied. Note 2 No HARQ retransmission |  |  |  |  |  |

We have presented the possible PDSCH demodulation requirements to be specified in TS38.101-4 for information.

# References

- R4-2309807, “WF for NonCol_intraB_ENDC_NR_CA_Demod”, Ericsson.

# Appendix – Text proposal for UE demodulation requirements in TS38.101-4 (For information)

##### 5.1.1.7.X	Applicability and test rules for power imbalance and reception time difference parameters for intra-band non-contiguous CA

The performance requirements in Table 5.1.1.7.X-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.7.X-1: Applicability and test rules for intra-band non-contiguous CA UE demodulation tests

| UE feature/capability [14] | CA combinations where the tests apply | Test list | PCell CC configuration | Applicability notes |
| --- | --- | --- | --- | --- |
| Type 2 UE (intraBandNonColocatedCA-r18] | CA_n77(2A) CA_n78(2A) | Clause 5.2A.2.X | Any of CCs |  |

#### 5.2A.2.X	Minimum requirements for carrier aggregation with power imbalance and reception time difference

The performance requirements are specified in Table 5.2A.2.X-3, with the addition of test parameters in Table 5.2A.2.X-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2A.2.X-1.

Table 5.2A.2.X-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the ability of NR-CA Type 2 UE ([intraBandNonColocatedCA-r18]) to demodulate the signal transmitted by the PCell and SCell in the presence of a stronger PCell signal on an adjacent frequency, and reception time difference between two cells. Throughput is measured on both the PCell and SCell simultaneously. | 1 |

Table 5.2A.2.X-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Length (L) |  | 12 for DL slot, 4 for special slot |
|  | PRB bundling size |  | Wideband |
| Number of HARQ Processes |  |  | 8 |
| Maximum number of HARQ transmission |  |  | 1 |
| Redundancy version coding sequence |  |  | {0} |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table A.1.2-2 for FR1.30-1 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 |
| Overhead for TBS determination |  |  | 0 |
| SSB transmission |  |  | Slot#0 with periodicity 20ms |
| Timing offset of the SCell from the PCell |  | µs | 33 |

Table 5.2A.2.X-3: Minimum performance for TDD CA with 30 kHz SCS

| Test number | Cell type | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation channel and antenna correlation | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SCell | R.PDSCH.X.1 TDD | 40MHz/30kHz | [QPSK 0.3] | FR1.30-1 | 2x2 static condition (TS38.101-4 B.1.1) | [70%] | TBD |
|  | PCell | R.PDSCH.X.2 TDD | 40MHz/30kHz | [256QAM 0.82] | FR1.30-1 | 2x2 static condition (TS38.101-4 B.1.1) | [70%] | TBD + 25dB |

Table A.3.2.2.2-X: PDSCH Reference Channel for TDD UL-DL pattern FR1.30-1 (CA power imbalance)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.2-X.1 TDD | R.PDSCH.2-X.2 TDD |  |  |  |
| Channel bandwidth | MHz | 40 | 40 |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 |  |  |  |
| Allocated resource blocks | PRBs | 106 | 106 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 |  |  |  |
| Allocated slots per 2 frames |  | 31 | 31 |  |  |  |
| MCS table |  | [256QAM] | [256QAM] |  |  |  |
| MCS index |  | [2] | [24] |  |  |  |
| Modulation |  | [QPSK] | [256QAM] |  |  |  |
| Target Coding Rate |  | [0.30] | [0.82] |  |  |  |
| Number of MIMO layers |  | [2] | [2] |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 6 | 6 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 0 | 0 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | [5376] | [58384] |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | [16896] | [184424] |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | [1] | [7] |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | CBs | [3] | [22] |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A |  |  |  |
| For Slots i = 20, 21 | Bits | 53424 | 213696 |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 17808 | 71232 |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} | Bits | 55968 | 223872 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | [23.885] | [260.649] |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |