3GPP TSG-RAN WG4 Meeting #108	R4-2312789

Toulouse, France, 21 August – 25 August 2023

Agenda Item:	6.8.5.2

Source:	Ericsson

Title:	Simulation results of SAN demodulation requirements for IoT-NTN

Document for:	Discussion

# 1	Introduction

RAN4#107 completed the SAN demodulation performance part of Rel-18 NB-IoT/eMTC core & performance requirements for NTN, however as the way forward mentioned, it was observed the big simulation result spans [1].

- RAN4 sets the SAN demodulation requirements in TS36.108/TS36.181 according to the simulation result summary R4-2308428 in RAN4#107. In RAN4#108, companies are encouraged to submit the updated simulation results, and RAN4 will discuss how to define the requirements if the span of the results are still large among companies.

We provide our updated simulation results to complete the requirements in TS 36.108 and TS 36.181.

# 2	SAN demodulation requirements for eMTC

## 2.1	PUSCH demodulation requirements

Table 1	Simulation parameters for PUSCH.

| Parameters | CE Mode A | CE Mode B |
| --- | --- | --- |
| Max number of HARQ transmissions | 4 | 2 |
| RV sequences | 0, 2, 3, 1, 0, 2, 3, 1 | 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 |
| Number of PUSCH repetitions | 8 | 256 |
| Frequency hopping | OFF | OFF |
| Duplex mode | FDD only | FDD only |
| Number of Tx antennas | 1 | 1 |
| Number of Rx antennas | 1 and 2 | 1 and 2 |
| Propagation channel | Test 1: NTN-TDLC5-5 Test 2: NTN-TDLA100-5 | Test 1: NTN-TDLA100-5 Test 2: NTN-TDLC5-5 |
| FRC | TS36.104 A3-2 | TS36.104 A3-1 |
| System BW | 1.4MHz only | 1.4MHz only |
| Fraction of maximum throughput | 70% | 70% |

Table 2	Simulation results for PUSCH CE Mode A.

| Rx antenna | Repetitions | Propagation condition | SNR for 70% of peak rate |
| --- | --- | --- | --- |
| 1 | 8 | NTN-TDLA100-5 | -6.1 |
| 1 | 8 | NTN-TDLC5-5 | -6.1 |
| 2 | 8 | NTN-TDLA100-5 | -10.5 |
| 2 | 8 | NTN-TDLC5-5 | -9.2 |

Table 3	Simulation results for PUSCH CE Mode B.

| Rx antenna | Repetitions | Propagation condition | SNR for 70% of peak rate |
| --- | --- | --- | --- |
| 1 | 256 | NTN-TDLA100-5 | -15.1 |
| 1 | 256 | NTN-TDLC5-5 | -16.3 |
| 2 | 256 | NTN-TDLA100-5 | -17.4 |
| 2 | 256 | NTN-TDLC5-5 | -18.1 |

## 2.2	PUCCH demodulation requirements

### 2.2.1	ACK missed detection requirements for single user PUCCH format 1a

Test metric:

- The ACK missed detection probability shall not exceed 1%.

Table 4	Simulation parameters for PUCCH format 1a.

| Parameters | Values |
| --- | --- |
| Number of PUCCH repetitions | 8 |
| Frequency hopping | OFF |
| Duplex mode | FDD only |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 1 and 2 |
| Propagation channel | NTN-TDLA100-5 |
| System BW | 1.4MHz |

Table 5	Simulation results for PUCCH format 1a.

| Rx antenna | Repetitions | Propagation condition | SNR for 1% of ACK missed detection |
| --- | --- | --- | --- |
| 1 | 8 | NTN-TDLA100-5 | -5.4 |
| 2 | 8 | NTN-TDLA100-5 | -11.1 |

## 2.3	PRACH demodulation requirements

### 2.3.1	PRACH detection requirements

Test metric:

- The false alarm probability shall be less than or equal to 0.1%.

- The probability of detection shall be equal to or exceed 99%.

Table 6	Simulation parameters for PRACH preamble detection requirements.

| Number of Tx antennas | Number of Rx antennas | Propagation channel | Frequency offset | Number of repetitions | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | NTN-TDLA100-5 | 270Hz | 8 | YES | YES |  |  |
|  |  |  |  | 16 |  |  | YES | YES |

Table 7	Simulation results for PRACH preamble detection requirements.

| Rx antenna | Repetitions | Propagation condition | Burst format | SNR for 99% of detection success rate |
| --- | --- | --- | --- | --- |
| 1 | 8 | NTN-TDLA100-5 | BF0 | -13.3 |
| 1 | 8 | NTN-TDLA100-5 | BF1 | -13.3 |
| 1 | 16 | NTN-TDLA100-5 | BF2 | -19.5 |
| 1 | 16 | NTN-TDLA100-5 | BF3 | -19.5 |
| 2 | 8 | NTN-TDLA100-5 | BF0 | -18.8 |
| 2 | 8 | NTN-TDLA100-5 | BF1 | -18.9 |
| 2 | 16 | NTN-TDLA100-5 | BF2 | -24.1 |
| 2 | 16 | NTN-TDLA100-5 | BF3 | -24.2 |

# 3	SAN demodulation requirements for NB-IoT

## 3.1	NPUSCH format 1 demodulation requirements

Table 8	Simulation parameters for NPUSCH format 1.

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions | FRC | Repetition number | Fraction of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 3.75KHz | 1 | Test 1: NTN-TDLA100-1 Test 2: NTN-TDLC5-1 | TS36.104 A16-1 | 4 | 70% |
| 1 | 1 and 2 | 15KHz | 12 | Test 1: NTN-TDLC5-1 Test 2: NTN-TDLA100-1 | TS36.104 A16-5 | 16 | 70% |

Table 9	Simulation results for NPUSCH format 1.

| Rx antenna | Subcarrier spacing | Number of tones | Repetitions | Propagation condition | SNR to achieve 70% of peak rate |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.75kHz | 1 | 4 | NTN-TDLA100-1 | -3.5 |
| 1 | 3.75kHz | 1 | 4 | NTN-TDLC5-1 | -6.1 |
| 1 | 15kHz | 12 | 16 | NTN-TDLA100-1 | -7.2 |
| 1 | 15kHz | 12 | 16 | NTN-TDLC5-1 | -9.8 |
| 2 | 3.75kHz | 1 | 4 | NTN-TDLA100-1 | -7.1 |
| 2 | 3.75kHz | 1 | 4 | NTN-TDLC5-1 | -8.7 |
| 2 | 15kHz | 12 | 16 | NTN-TDLA100-1 | -11.7 |
| 2 | 15kHz | 12 | 16 | NTN-TDLC5-1 | -12.8 |

## 3.2	NPUSCH format 2 demodulation requirements

### 3.2.1	ACK missed detection requirements

Test metric:

- The DTX to ACK probability, i.e., the probability that ACK is detected when nothing is sent per NPUSCH format 2 transmission, shall not exceed 1% per NPUSCH format 2 transmission.

- The ACK missed detection probability shall not exceed 1%.

Table 10	Simulation parameters for NPUCCH format 2.

| Number of TX antennas | Number of RX antennas | Propagation conditions | Number of allocated subcarriers | Subcarrier spacing | Repetition number |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | NTN-TDLA100-1 | 1 | 3.75KHz | 16 |
| 1 | 1 and 2 | NTN-TDLA100-1 | 1 | 15KHz | 16 |

Table 11	Simulation results for NPUSCH format 2.

| Rx antenna | Subcarrier spacing | Repetitions | Propagation condition | SNR for 1% of ACK missed detection rate |
| --- | --- | --- | --- | --- |
| 1 | 3.75kHz | 16 | NTN-TDLA100-1 |  |
| 1 | 15kHz | 16 | NTN-TDLA100-1 | 3.8 |
| 2 | 3.75kHz | 16 | NTN-TDLA100-1 |  |
| 2 | 15kHz | 16 | NTN-TDLA100-1 | -5.1 |

## 3.3	NPRACH demodulation requirements

### 3.3.1	NPRACH detection requirements

Test metric:

- The false alarm probability shall be less than or equal to 0.1%.

- The probability of detection shall be equal to or exceed 99%.

Table 12	Simulation parameters for NPRACH missed detection.

| Number of Tx | Number of Rx | Repetition number | Propagation channel and correlation | Frequency offset | PF0 | PF1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 and 2 | 8 | NTN-TDLA100-1 | 200Hz | YES | YES |
| 1 | 1 and 2 | 16 | NTN-TDLA100-1 | 200Hz | YES | YES |

Table 13	Simulation results for NPRACH missed detection.

| Rx antenna | Repetitions | Propagation condition | Preamble format | SNR for 99% of detection success rate |
| --- | --- | --- | --- | --- |
| 1 | 8 | NTN-TDLA100-1 | PF0 | 10.5 |
| 1 | 8 | NTN-TDLA100-1 | PF1 | 10.5 |
| 1 | 16 | NTN-TDLA100-1 | PF0 | 5.9 |
| 1 | 16 | NTN-TDLA100-1 | PF1 | 5.8 |
| 2 | 8 | NTN-TDLA100-1 | PF0 | 0.7 |
| 2 | 8 | NTN-TDLA100-1 | PF1 | 0.7 |
| 2 | 16 | NTN-TDLA100-1 | PF0 | -1.9 |
| 2 | 16 | NTN-TDLA100-1 | PF1 | -1.7 |

# References

- R4-2309827, “WF for IoT over NTN SAN demodulation requirements”, Ericsson.