3GPP TSG-RAN WG4 Meeting #108 	R4-2312803

Toulouse, France, August 21 – 25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results on NCR PDSCH and PDCCH Demodulation Requirements

Agenda item:	8.28.6

Document for:	Information

Introduction

In [1], the way forward (WF) from the discussions on NCR (Network-Controlled Repeaters) demodulation in RAN4#107 is documented. Some of the agreements in the WF related to NCR-Mobile Termination (MT) on the PDSCH requirements are summarized below:

Issue 1-1-1: FR1 requirements Agreement: - 10MHz for 15kHz SCS and 40MHz for 30kHz SCS for NCR-MT demodulation requirements. - FFS whether need to consider 5MHz/15kHz Issue 1-2-1: Test metric for FR1 and FR2 Agreement: - Both 1% BLER(1st transmission) and 70% maximum throughput using the same simulation configurations with maximum number of HARQ transmission=4 Issue 1-2-2: MCS Agreement: - MCS 4 (QPSK) for FR1 - QPSK is the same as legacy UE for FR2

The agreement on Issue 1-2-1 above requires simulation to be conducted by interested companies for the already agreed performance metrics, which results then will be used to define the requirements. The yellow highlighted FFS on Issue 1-1-1, will also need simulation results from interested companies to support their consideration on having PDSCH requirements for 5MHz/15KHz in FR1.

Some of the agreements in the WF related to NCR-Mobile Termination (MT) on the PDCCH requirements are summarized below:

Issue 1-3-1: How to define PDCCH requirements Agreements: - Reuse UE requirements for FDD and TDD - FFS for adapt PDCCH requirement for testing of DCI type 5_0 used for access link beam change indication. Issue 1-3-1: PDCCH requirements Agreement: - Reuse IAB-MT and legacy UE requirements as much as possible as baseline. - FFS for adapt PDCCH requirement for testing of DCI type 2_8 used for access link beam change indication. - Interesting companies can bring simulation results for DCI type 2_8.

As agreed on Issue 1-3-1, there is a need to have simulation for PDCCH requirements for testing DCI type 2_8. In addition to PDSCH simulation results, in this paper we also provide our simulation results on PDCCH.

Simulation Results on PDSCH Requirements

In this paper, we provide simulation results on PDSCH requirements for the test metrics agreed in RAN4#107. Furthermore, we provide simulation results on PDSCH for 5MHz/15KHz to support the proposal on having requirements for NCR-MT 5MHz/15KHz in FR1. Our observations and proposals in details on the remaining open issues on NCR-MT demodulation can be found in [2].

As there are no details on the simulation parameters proposed by any companies in RAN4#107, we are using related simulation parameters of UE and IAB-MT PDSCH requirements as in [3] and [4]. They are provided in the Appendix below for references.

Even though there are some requirements that can be reused from [3] and [4] for 70% throughput, as there is a new performance metric (1% BLER) being agreed in RAN4#107, there is a need to run simulation for both performance metrics. As agreed on Issue 1-2-1, to reduce the load of simulation campaign, both performance metrics to be evaluated are expected to come from the same simulation run. Hence, for 1% BLER we consider it from the 1st transmission without HARQ retransmission, while for 70% Throughput from the HARQ retransmission with number of retransmissions 4.

Furthermore, as discussed and agreed by the companies during online and offline discussions, companies will refer to current specifications of IAB-MT and UE and reuse as many as possible the requirements of IAB-MT and UE. As 3GPP specification in [4] provides only baseline requirements for TDD/30KHz case, for FDD/15 KHz the references are from [3], with some adaptation on the parameters as agreed for NCR-MT.

General Simulation Assumptions

The WF from RAN4#106bis and RAN4#107 meeting in [5] and [1], respectively, have documented several assumptions that need to be considered in the simulation. The summary of the agreed simulation assumptions for NCR-MT from [1] and [5] is given below:

- TDD for both NR FR1 and FR2, FDD for NR FR1.

- FDD/15 KHz and TDD/30 KHz for 1 layer (reusing/adapting requirements for IAB-MT).

  - 10M/15kHz; 40M/30kHz

  - FFS whether need to consider additional CHBW 5MHz/15kHz

- FR2: (Reuse IAB-MT) Rank 1 with 100MHz/120kHz

- Single-slot FRCs (similar as IAB-MT).

- TDD and FDD pattern: Default pattern can be reused from IAB-MT.

- Additional requirements which are not in IAB-MT: Based on Rel-15 UE performance requirements.

- QPSK and 16QAM

- Rank 1

- Antenna configuration:

  - 2x2 and 2x4 for FR1

  - 2x2 for FR2

- Propagation conditions:

  - TDLA30-10 FR1

  - TDLA30-75 FR2

- Mapping type A

- PRB bundling size 2

- Receiver type: MMSE-IRC

- PDSCH requirements

  - Test metric for FR1 and FR2

  - Both 1% BLER(1st transmission) and 70% maximum throughput using the same simulation configurations with maximum number of HARQ transmission=4

  - MCS

    - MCS 4 (QPSK) for FR1

    - QPSK is the same as legacy UE for FR2

- PDCCH requirements

  - FFS for adapt PDCCH requirement for testing of DCI type 2_8 used for access link beam change indication.

PDSCH Simulation in FR1

In the following, we provide the results of our simulation on NCR PDSCH requirements in FR1, i.e., using FR1 FRCs. We will first provide the simulation results on TDD, followed by FDD. In the caption of each table of simulation results, we also provide the corresponding table of parameters used in the simulation. The tables of parameters are provided in Appendix A, which can be used as proposals for parameters in the final specification for NCR PDSCH specifications in FR1.

PDSCH FR1 TDD

Simulation results on NCR-MT PDSCH FR1 TDD are tabulated below:

Table 1. Minimum requirements for PDSCH Type A with Rank 1 2x4 (Use Table A-1)

| FRC (Table A- 2) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M-FR1-A.3.1-1 | 40/30 | TDLA30-10 | 2x4, ULA Low | 70 | 1.72 | 1 | 4.09 |

Table 2. Minimum requirements for PDSCH with Rank 1, 40 MHz Channel Bandwidth, 30 kHz SCS 2x2 (Use Table A-3)

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | FRC (Table A-2) | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 2 | TDLA30-10, ULA Low | M-FR1-A.3.1-1 | 70 | 4.09 | 1 | 7.19 |

Table 3. Minimum performance for Rank 1 2x2 (Use Table A-8)

| Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| R.PDSCH.2-1.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-1A | TDLA30-10 | 2x2, ULA Low | 70 | -2.79 | 1 | -0.13 |
| R.PDSCH.2-1.2 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low | 70 | -2.06 | 1 | 5.35 |
| R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 2x2, ULA Low | 70 | 4.39 | 1 | 8.06 |
| R.PDSCH.2-5.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-2 | TDLA30-10 | 2x2, ULA Low | 70 | -2.87 | 1 | -0.22 |
| R.PDSCH.2-6.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-3 | TDLA30-10 | 2x2, ULA Low | 70 | -2.84 | 1 | -0.31 |
| R.PDSCH.2-11.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-5 | TDLA30-10 | 2x2, ULA Low | 70 | -2.79 | 1 | -0.33 |
| R.PDSCH.2-12.1 TDD | 40 / 30 | QPSK, 0.30 | FR1.30-6 | TDLA30-10 | 2x2, ULA Low | 70 | -2.79 | 1 | -0.36 |

PDSCH FR1 FDD

Simulation results on NCR-MT PDSCH FR1 FDD are tabulated below:

Table 4. Minimum performance for Rank 1 2x2 (Use Table A-6)

| Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 2x2, ULA Low | 70 | -2.39 | 1 | 0.6 |
| R.PDSCH.1-1.2 FDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 2x2, ULA Low | 70 | -1.38 | 1 | 6.88 |

Table 5. Minimum performance for Rank 1 2x4 (Use Table A-7)

| Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 2x4, ULA Low | 70 | -4.77 | 1 | -2.09 |
| R.PDSCH.1-1.2 FDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 2x4, ULA Low | 70 | -4.25 | 1 | 0.65 |
| R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 1.66 | 1 | 4.98 |

PDSCH Simulation in FR2

In the following, we provide our simulation results of NCR PDSCH requirements in FR2, i.e., using FR2 FRCs. They are tabulated below, and the tables of parameters are provided in Appendix A which can be used as proposals for parameters in the final specification for NCR PDSCH specifications for FR2.

Table 6. Minimum requirements for PDSCH with Rank 1, 100 MHz Channel Bandwidth, 120 kHz SCS 2x2 (Table A-4)

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | FRC (Table A-5) | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 2 | TDLA30-75, ULA Low | M-FR2-A.3.1-2 | 70 | 3.86 | 1 | 5.27 |

Table 7. Minimum performance for Rank 1 (FRC) 2x2 (Use Table A-9)

| Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) | BLER (%) | SNR (dB) |  |
| R.PDSCH.5-1.1 TDD | 100 / 120 | QPSK, 0.30 | FR2.120-1A | TDLA30-75, ULA Low | 2x2 ULA Low | 70 | -3.24 | 1 | -2.31 |  |
| R.PDSCH.5-2.1 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75, ULA Low | 2x2 ULA Low | 70 | 4.24 | 1 | 5.3 |  |

PDSCH Simulation for 5 MHz/15 KHz Case

Additionally, as 5MHz/15KHz has been agreed in [1] as FFS, we need to run simulation for 5MHz/15KHz. As 15KHz SCS is agreed to be used for requirements in FDD, the simulation for 5MHz needs to take test parameters for FDD in 2RX and 4RX cases. In our simulation, we are using the same simulation parameters as for 10 MHz/15KHz, but with halving the number of the resource blocks. Table 8 and Table 9 shows the simulation results for 5MHz/15KHz case.

Table 8. Minimum performance for Rank 1 2x2 (Use Table A-6)

| Reference channel/Number of RBs | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| R.PDSCH.1-1.1 FDD/25 | 5 / 15 | QPSK, 0.30 | TDLA30-10 | 2x2, ULA Low | 70 | -1.44 | 1 | 4.12 |
| R.PDSCH.1-1.2 FDD/6 | 5 / 15 | QPSK, 0.30 | TDLA30-10 | 2x2, ULA Low | 70 | -1.07 | 1 | 6.76 |

Table 9. Minimum performance for Rank 1 2x4 (Use Table A-7)

| Reference channel/Number of RBs | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | BLER (%) | SNR (dB) |
| R.PDSCH.1-1.1 FDD/25 | 5 / 15 | QPSK, 0.30 | TDLA30-10 | 2x4, ULA Low | 70 | -4.04 | 1 | -0.55 |
| R.PDSCH.1-1.2 FDD/6 | 5 / 15 | QPSK, 0.30 | TDLA30-10 | 2x4, ULA Low | 70 | -3.84 | 1 | 0.16 |
| R.PDSCH.1-2.1 FDD/25 | 5 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x4, ULA Low | 70 | 1.80 | 1 | 5.74 |

Simulation Results on PDCCH Requirements

As stated in the WF [1], companies are encouraged to provide simulation results on PDCCH for DCI Type 2_8. In [2], we have provided our observations and proposals related to the need of having new PDCCH requirements for NCR-MT due to the use of DCI Type 5_0/2_8, which has higher payload than DCI Type 1_0 and 1_1 that are currently available in the specifications. In this paper, we provide our simulation results of PDCCH for DCI Type 5_0/2_8, in which we adapt the DCI payload size according to the maximum value for DCI Type 5_0/2_8, which is 128 bits.

PDCCH Simulation in FR1

We will first provide the simulation results on TDD, followed by FDD. In the caption of each table of simulation results, we also provide the corresponding table of parameters used in the simulation. The tables of parameters are provided in Appendix B, which can be used as proposals for parameters in the final specification for NCR PDCCH specifications in FR1.

PDCCH FR1 TDD

Simulation results on NCR-MT PDCCH FR1 TDD are tabulated below:

Table 10. Minimum performance for PDCCH with 30 kHz SCS 1x2 Low (Use Table A-10)

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 40 | 102 | 1 | 2 | R.PDCCH. x-y.1 TDD | TDLA30-10 | 1x2 Low | 1 | 10.6 |
| 40 | 102 | 1 | 4 | R.PDCCH. x-y.2 TDD | TDLA30-10 | 1x2 Low | 1 | 4.5 |

Table 11. Minimum performance for PDCCH with 30 kHz SCS 2x2 Low (Use Table A-10)

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Table A-11) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 40 | 90 | 1 | 8 | R.PDCCH. x-y.3 TDD | TDLA30-10 | 2x2 Low | 1 | 1.4 |

Table 12. Minimum performance for PDCCH with 30 kHz SCS 1x4 Low (Use Table A-10)

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Table A-11) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 40 | 102 | 1 | 2 | R.PDCCH. x-y.1 TDD | TDLA30-10 | 1x4 Low | 1 | 3.2 |
| 40 | 102 | 1 | 4 | R.PDCCH. x-y.2 TDD | TDLA30-10 | 1x4 Low | 1 | -0.8 |

Table 13. Minimum performance for PDCCH with 30 kHz SCS 2x4 Low (Use Table A-10)

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Table A-11) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 40 | 90 | 1 | 8 | R.PDCCH. x-y.3 TDD | TDLA30-10 | 2x4 Low | 1 | -3.5 |

PDCCH FR1 FDD

Simulation results on NCR-MT PDSCH FR1 FDD are tabulated below:

Table 14. Minimum performance for PDCCH with 15 kHz SCS 1x2 Low (Use Table A-10)

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Table A- 13) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 10 | 24 | 2 | 2 | R.PDCCH. x-2.1 FDD | TDLA30-10 | 1x2 Low | 1 | 12.2 |
| 10 | 48 | 2 | 4 | R.PDCCH. x-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | 7.7 |

Table 15. Minimum performance for PDCCH with 15 kHz SCS 2x2 Low (Use Table A-10)

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Table A-12) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 10 | 48 | 1 | 8 | R.PDCCH.x-1.3 FDD | TDLA30-10 | 2x2 Low | 1 | 3.4 |

Table 16. Minimum performance for PDCCH with 15 kHz SCS 1x4 Low (Use Table A-10)

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Table A- 13) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 10 | 24 | 2 | 2 | R.PDCCH. x-2.1 FDD | TDLA30-10 | 1x4 Low | 1 | 4.4 |
| 10 | 48 | 2 | 4 | R.PDCCH. x-2.4 FDD | TDLA30-10 | 1x4 Low | 1 | 0.7 |

Table 17. Minimum performance for PDCCH with 15 kHz SCS 2x4 Low (Use Table A-10)

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Table A-12) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 10 | 48 | 1 | 4 8 | R.PDCCH.x-1.2 FDD | TDLA30-10 | 2x4 Low | 1 | -2.0 |

PDCCH Simulation in FR2

In the caption of each table of simulation results, we also provide the corresponding table of parameters used in the simulation. The tables of parameters are provided in the Appendix B which can be used as proposals for parameters in the final specification for NCR PDCCH specifications in FR2.

Table 18. Minimum performance requirements with 120 kHz SCS Table A-14

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Table A-15) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 100 | 60 | 1 | 2 | R.PDCCH. z-1.1 TDD | TDLA30-75 | 1x2 Low | 1 | 9.5 |
| 100 | 60 | 1 | 4 | R.PDCCH. z-1.2 TDD | TDLA30-75 | 1x2 Low | 1 | 3.7 |

Table 19. Minimum performance requirements with 120 kHz SCS Table A-14

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel (Table A-15) | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Pm-dsg (%) | SNRBB (dB) |
| 100 | 60 | 1 | 8 | R.PDCCH. z-1.3 TDD | TDLA30-75 | 2x2 Low | 1 | 0.3 |

Conclusion

In this paper, we provide our simulation results for NCR-MT PDSCH and PDCCH demodulation requirements. The PDSCH simulation results take into account the new agreed metrics, namely, BLER 1%. The PDCCH simulation results are using the payload size of DCI Type 5_0/2_8. It can be seen that with the higher number of payload size of DCI 5_0/2_8, higher SNR is required as compared to DCI Type 1_0 and 1_1 specified in IAB and UE specifications. More on our observations and proposal can be found in [2].

- References

- R4-2309832, WF on NCR_MT demodulation requirements, ZTE, RAN4#107, Incheon, KR, May 22 – May 26, 2023

- R4-2312802, NCR Demodulation Performance Requirements, Nokia, Nokia Shanghai Bell, Toulouse, FR, August 21 – 25, 2023

- 3GPP TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements, V17.7.0, 12-2022

- 3GPP TS 38.174, Integrated access and backhaul radio transmission and reception, V17.2.0, 12-2022

- R4-2305934, WF on NCR-MT demodulation requirements, ZTE, RAN4#106bis-e, E-meeting, April 17–26, 2023.

- Appendix A. Simulation Parameters for PDSCH

In this Appendix A, we provide the simulation parameters used in our simulation for PDSCH. The tabulated parameters below are mainly from [3] and [4], and we propose RAN4 to use them for simulation alignment between companies to define NCR-MT PDSCH requirements.

Table A-1. Test parameters for testing PDSCH (Conducted)

| Parameter |  | Value |
| --- | --- | --- |
| Cyclic prefix |  | Normal |
| Default TDD UL-DL pattern (Note 1) |  | 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests |
|  | DM-RS port(s) | {1000} for Rank 1 tests {1000-1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PDSCH mapping type | A |
|  | Start symbol | 2 |
|  | Allocation length | 12 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
| PT-RS configuration |  | Not configured |
| PRB bundling size |  | 2 |
| VRB-to-PRB mapping type |  | Not interleaved |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. |  |  |

Table A-2. FRC parameters for FR1 PDSCH performance requirements, 1 transmission layers, 16QAM

| Parameter | Unit |
| --- | --- |
| Reference channel |  | M-FR1-A.3.1-1 |
| Channel bandwidth | MHz | 40 |
| Subcarrier spacing | kHz | 30 |
| Allocated resource blocks | PRBs | 106 |
| Number of consecutive PDSCH symbols |  | 12 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  | 26632 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 4 |
| Binary Channel Bits Per Slot |  | 55968 |

Table A-3. Test parameters for PDSCH testing (Radiated 1-O)

| Parameter |  | Value |
| --- | --- | --- |
| Cyclic prefix |  | Normal |
| Default TDD UL-DL pattern (Note 1) |  | 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests |
|  | DM-RS port(s) | {1000} for Rank 1 tests {1000-1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PDSCH mapping type | A |
|  | Start symbol | 2 |
|  | Allocation length | 12 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
| PT-RS configuration |  | Not configured |
| PRB bundling size |  | 2 |
| VRB-to-PRB mapping type |  | Not interleaved |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. Note 2: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 [28] are left up to test implementation, if transmitted or needed. |  |  |

Table A-4. Test parameters for PDSCH testing (Radiated Type 2-O)

| Parameter |  | Value |
| --- | --- | --- |
| Cyclic prefix |  | Normal |
| Default TDD UL-DL pattern (Note 1) |  | 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 1 |
|  | DM-RS port(s) | {1000} for Rank 1 tests {1000-1001} for Rank 2 tests |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PDSCH mapping type | A |
|  | Start symbol | 1 |
|  | Allocation length | 13 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
| PT-RS configuration | Frequency density (KPT-RS) | 2 |
|  | Time density (LPT-RS) | 1 |
| PRB bundling size |  | 2 |
| VRB-to-PRB mapping type |  | Not interleaved |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. Note 2: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 [28] are left up to test implementation, if transmitted or needed. |  |  |

Table A-5. FRC parameters for FR2 PDSCH performance requirements, 1 transmission layers, 16QAM

| Parameter | Unit |
| --- | --- |
| Reference channel |  | M-FR2-A.3.1-2 |
| Channel bandwidth | MHz | 100 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  | 13 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 6 |
| Information Bit Payload per Slot |  | 17424 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 3 |
| Binary Channel Bits Per Slot |  | 36564 |

Table A-6. Test parameters FDD 2RX

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Test 1-1 2 for other tests |
|  | Resource allocation type |  | Test 1-2: Type 1 with start RB = 23, LRBs = 6 Other tests: Type 0 |
|  | RBG size |  | Test 1-2: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Tests 1-1, 1-5 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | Test 1-5: 10 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-5: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 8 for Test 1-4 4 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table A-7. Test parameters 4RX

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Test 1-1 wideband for Test 3-1 2 for other tests |
|  | Resource allocation type |  | Test 1-2: Type 1 with start RB = 23, LRBs = 6 Other test: Type 0 |
|  | RBG size |  | Test 1-2: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Test 1-1, 1-5 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | Test 1-5: 10 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-5: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 8 for Test 1-4 4 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table A-8. Test parameters TDD

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 4 for Tests 1-1, 1-8, 1-9 2 for other tests |
|  | Resource allocation type |  | Test 1-2: Type 1 with start RB = 50, LRBs = 6 Other tests: Type 0 |
|  | RBG size |  | Test 1-2: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 2 for Tests 1-1, 1-7, 1-8, 1-9 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | Tests 1-8, 1-9: l0 = 4 for CSI-RS resource 1 and 3 l0 = 8 for CSI-RS resource 2 and 4 Other tests; Table 5.2-1. |
|  | CSI-RS periodicity | Slots | Test 1-7: 20 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-7: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
|  | Frequency Occupation |  | Test 1-7: Start PRB 0 Number of PRB = 52 Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 16 for Test 1-4 10 for Test 1-9 8 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table A-9. Test Parameters FR2

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS (l0) |  | For Test 1-1 and 1-2: 3 for CSI-RS resource 1 and 3 7 for CSI-RS resource 2 and 4 |
|  | CSI-RS offset | Slots | For Test 1-2: 82 for CSI-RS resource 1 and 2 83 for CSI-RS resource 3 and 4 |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 1/AL4 for Test 2-3 1/AL8 for other tests |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | Specific to each Reference channel as defined in A.3.2.2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband for Test 1-1, 2 for other tests |
|  | Resource allocation type |  | Test 2-1: Type 1 with start RB = 30, LRBs = 6 Other tests: Type 0 |
|  | RBG size |  | Test 2-1: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 for Test 1-1, 1-3, 2-2, 2-4 10 for Test 2-1, 2-3, 2-5, 2-6, 3-1 16 for Test 1-2 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 |

- Appendix B. Simulation Parameters for PDCCH

In this Appendix B, we provide the simulation parameters used in our simulation for PDCCH. The tabulated parameters below are mainly from [3] and [4], and we propose RAN4 to use them for simulation alignment between companies to define NCR-MT PDCCH requirements.

Table A-10. Common test Parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) |  |  | 0 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Number of PDCCH candidates |  |  | 1 |
|  | Frequency domain resource allocation for CORESET |  |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  |  | TCI state #1 |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | CSI-RS resource 1: 4 CSI-RS resource 2: 8 CSI-RS resource 3: 4 CSI-RS resource 4: 8 |
|  | Number of CSI-RS ports (X) |  |  | 1 |
|  | CDM Type |  |  | No CDM |
|  | Density (ρ) |  |  | 3 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  | QCL info |  |  | TCI state #0 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type D |
| PDCCH Precoding configuration |  |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 for FDD. For TDD, specific to each TDD UL-DL pattern and as defined in Annex A.1.2. |

Table A-11. PDCCH Reference Channels TDD (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.2-1.1 TDD | R.PDCCH.2-1.2 TDD | R.PDCCH.2-1.3 TDD | R.PDCCH.x-y.1 TDD | R.PDCCH.x-y.2 TDD | R.PDCCH.x-y.3 TDD |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 | 30 |
| CORESET frequency domain allocation |  | 102 | 102 | 90 | 102 | 102 | 90 |
| CORESET time domain allocation |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 5_0/2_8 | 5_0/2_8 | 5_0/2_8 |
| Payload (without CRC) | Bits | 41 | 53 | 53 | 128 | 128 | 128 |

Table A-12. PDCCH Reference Channels FDD (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-1.1 FDD | R.PDCCH.1-1.2 FDD | R.PDCCH.1-1.3 FDD | R.PDCCH.x-1.1 FDD | R.PDCCH.x-1.2 FDD | R.PDCCH.x-1.3 FDD |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 48 | 48 | 48 | 48 | 48 | 48 |
| CORESET time domain allocation |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Aggregation level |  | 4 | 4 | 8 | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 5_0/2_8 | 5_0/2_8 | 5_0/2_8 |
| Payload (without CRC) | Bits | 39 | 52 | 52 | 128 | 128 | 128 |

Table A- 13. PDCCH Reference Channel FDD (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-2.1 FDD | R.PDCCH.1-2.2 FDD | R.PDCCH.1-2.3 FDD | R.PDCCH.1-2.4 FDD | R.PDCCH.1-2.5 FDD | R.PDCCH.1-2.6 FDD | R.PDCCH.x-2.1 FDD | R.PDCCH.x-2.2 FDD | R.PDCCH.x-2.3 FDD | R.PDCCH.x-2.4 FDD | R.PDCCH.x-2.5 FDD |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 24 | 24 | 24 | 48 | 48 | 48 | 24 | 24 | 24 | 48 | 48 |
| CORESET time domain allocation |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Aggregation level |  | 2 | 4 | 2 | 4 | 8 | 16 | 2 | 4 | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_0 | 1_1 | 1_1 | 1_1 | 1_0 | 5_0/2_8 | 5_0/2_8 | 5_0/2_8 | 5_0/2_8 | 5_0/2_8 |
| Payload (without CRC) | Bits | 39 | 39 | 52 | 52 | 52 | 39 | 128 | 128 | 128 | 128 | 128 |

Table A-14. Common test Parameters FR2

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) |  |  | 0 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Number of PDCCH candidates |  |  | 1 |
|  | Frequency domain resource allocation for CORESET |  |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  |  | TCI state #1 |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | CSI-RS resource 1: 4 CSI-RS resource 2: 8 CSI-RS resource 3: 4 CSI-RS resource 4: 8 |
|  | Number of CSI-RS ports (X) |  |  | 1 |
|  | CDM Type |  |  | No CDM |
|  | Density (ρ) |  |  | 3 |
|  | CSI-RS periodicity |  | Slots | 160 |
|  | CSI-RS offset |  | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for beam refinement | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | CSI-RS resource 1: 8 CSI-RS resource 2: 9 |
|  | Number of CSI-RS ports (X) |  |  | 1 |
|  | CDM Type |  |  | No CDM |
|  | Density (ρ) |  |  | 3 |
|  | CSI-RS periodicity |  | Slots | 120 kHz SCS: 160 for CSI-RS resource 1,2 |
|  | CSI-RS offset |  | Slots | 0 for CSI-RS resource 1,2 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4) *4 |
|  | Repetition |  |  | ON |
|  | QCL info |  |  | TCI state #1 |
| PDCCH & PDCCH DMRS Precoding configuration |  |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type D |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.3. |

Table A-15. PDCCH Reference Channels FR2 (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.5-1.1 TDD | R.PDCCH.5-1.2 TDD | R.PDCCH.5-1.3 TDD | R.PDCCH.z-1.1 TDD | R.PDCCH.z-1.2 TDD | R.PDCCH.z-1.3 TDD |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 | 120 | 120 |
| CORESET frequency domain allocation |  | 60 | 60 | 60 | 60 | 60 | 60 |
| CORESET time domain allocation |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 5_0/2_8 | 5_0/2_8 | 5_0/2_8 |
| Payload (without CRC) | Bits | 40 | 56 | 56 | 128 | 128 | 128 |