3GPP TSG-RAN4 Meeting #108	R4-2312822

# Toulouse, France, 21 – 25 August, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3531 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_pos_enh-Perf] CR on positioning measurement reporting R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Perf |  |  |  |  |  | Date: |  |  | 2023-06-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 37.355, the differential reporting for PRS-RSRPP (nr-DL-PRS-FirstPathRSRP-ResultDiff) is always with INTEGER (0..61), no matter it is reported for DL-AoD or for DL-TDOA/m-RTT. Technically, the first resource may not have largest PRS-RSRPP, so the reporting range should include both positive and negative values. However, the mapping table in 38.133 only has positive values. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update mapping table for differential PRS-RSRPP to include both positive and negative values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mapping table in 38.133 is not aligned with the signaling in 37.355 and not aligned with the technical intention. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.38.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

##### 10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement

The reporting range of differential PRS-RSRPP is defined from -30 dB to 30 dB with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.38.3.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

For differential reporting, PRS-RSRPP is reported as the difference in dB with respect to the first reported PRS-RSRPP.

Table 10.1.38.3.2-1: Measurement report mapping for differential PRS-RSRPP

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| DIFFRSRPP_0 | -30≥ΔRSRPP | dB |
| DIFFRSRPP_1 | -29≥ΔRSRPP>-30 | dB |
| DIFFRSRPP_2 | -28≥ΔRSRPP>-29 | dB |
| DIFFRSRPP_3 | -27≥ΔRSRPP>-28 | dB |
| DIFFRSRPP_4 | -26≥ΔRSRPP>-27 | dB |
| DIFFRSRPP_5 | -25≥ΔRSRPP>-26 | dB |
| DIFFRSRPP_6 | -24≥ΔRSRPP>-25 | dB |
| DIFFRSRPP_7 | -23≥ΔRSRPP>-24 | dB |
| DIFFRSRPP_8 | -22≥ΔRSRPP>-23 | dB |
| DIFFRSRPP_9 | -21≥ΔRSRPP>-22 | dB |
| DIFFRSRPP_10 | -20≥ΔRSRPP>-21 | dB |
| DIFFRSRPP_11 | -19≥ΔRSRPP>-20 | dB |
| DIFFRSRPP_12 | -18≥ΔRSRPP>-19 | dB |
| DIFFRSRPP_13 | -17≥ΔRSRPP>-18 | dB |
| DIFFRSRPP_14 | -16≥ΔRSRPP>-17 | dB |
| … | … | … |
| DIFFRSRPP_25 | -5≥ΔRSRPP>-6 | dB |
| DIFFRSRPP_26 | -4≥ΔRSRPP>-5 | dB |
| DIFFRSRPP_27 | -3≥ΔRSRPP>-4 | dB |
| DIFFRSRPP_28 | -2≥ΔRSRPP>-3 | dB |
| DIFFRSRPP_29 | -1≥ΔRSRPP>-2 | dB |
| DIFFRSRPP_30 | 0≥ΔRSRPP>-1 | dB |
| DIFFRSRPP_31 | 1≥ΔRSRPP>0 | dB |
| DIFFRSRPP_32 | 2≥ΔRSRPP>1 | dB |
| DIFFRSRPP_33 | 3≥ΔRSRPP>2 | dB |
| DIFFRSRPP_34 | 4≥ΔRSRPP>3 | dB |
| DIFFRSRPP_35 | 5≥ΔRSRPP>4 | dB |
| DIFFRSRPP_36 | 6≥ΔRSRPP>5 | dB |
| … | … | … |
| DIFFRSRPP_47 | 17≥ΔRSRPP>16 | dB |
| DIFFRSRPP_48 | 18≥ΔRSRPP>17 | dB |
| DIFFRSRPP_49 | 19≥ΔRSRPP>18 | dB |
| DIFFRSRPP_50 | 20≥ΔRSRPP>19 | dB |
| DIFFRSRPP_51 | 21≥ΔRSRPP>20 | dB |
| DIFFRSRPP_52 | 22≥ΔRSRPP>21 | dB |
| DIFFRSRPP_53 | 23≥ΔRSRPP>-22 | dB |
| DIFFRSRPP_54 | 24≥ΔRSRPP>23 | dB |
| DIFFRSRPP_55 | 25≥ΔRSRPP>24 | dB |
| DIFFRSRPP_56 | 26≥ΔRSRPP>25 | dB |
| DIFFRSRPP_57 | 27≥ΔRSRPP>26 | dB |
| DIFFRSRPP_58 | 28≥ΔRSRPP>27 | dB |
| DIFFRSRPP_59 | 29≥ΔRSRPP>28 | dB |
| DIFFRSRPP_60 | 30≥ΔRSRPP>29 | dB |
| DIFFRSRPP_61 | ΔRSRPP>30 | dB |

<End of Change 1>