3GPP TSG-RAN WG4 Meeting #108	R4-2312825

Toulouse, France, 21 – 25 August, 2023

Title: 	Reply LS on measurements without gap

Source: 	Huawei, HiSilicon

Agenda item:	8.9.1

Document for:	Approval

Introduction

RAN4 received LS from RAN2 [1] related to LTE inter-RAT measurement to NR without MG. In this paper we will provide reply LS for [1].

Discussion

In [1] RAN2 asked RAN4 whether interruption is allowed according to Rel-16 RAN4 specifications when UE reports “no gap” in Rel-16 field for LTE inter-RAT measurement to NR.

RAN2 has discussed the corresponding CR for the RAN4 LS on measurements without gap [1]. However, RAN2 would like to get clarification on the meaning of the legacy Rel-16 “no gap” indication. Rel-18 has defined new code points where UE may support no gap with interruption or no gap without interruption, but it is not clear how these are related to the legacy behaviour. Therefore, RAN2 would like to ask RAN4 whether interruption is allowed according to Rel-16 RAN4 specifications when UE reports “no gap” in Rel-16 field for LTE inter-RAT measurement to NR?

We understand there is no need to clarify the question for R16.

- There is no requirement in Rel-16 RAN4 specifications for inter-RAT NR measurement with NFG.

- The WI NR_MG_enh2-Core is for R18, and the requirements only apply to R18 UE

In R16 36.133, there are following requirements related to inter-RAT measurement without gaps. However, those requirements are already there in R15 or earlier release of 36.133, and the inter-RAT measurements considered are to 2G/3G. Also, UE capability for inter-RAT NR measurement without gaps are signaled by interRAT-NeedForGapsNR.

Inter-frequency and inter-RAT measurement requirements within this clause rely on the UE being configured with one measurement gap pattern unless the UE has signaled that it is capable according to the capability interFreqNeedForGaps or interRATNeedForGaps of conducting such measurements without gaps and without interruption.

Technically, it is not possible to regulate the reporting behavior for R16 UE which may be already in the field, so whether a R16 UE would cause interruption for LTE inter-RAT measurement to NR when reporting “no gap” should be left to UE implementation.

Based on above, we suggest RAN4 not to make clarification on whether interruption is allowed for R16.

Proposal: RAN4 not to make clarification on whether interruption is allowed when UE reports “no gap” in Rel-16 field for LTE inter-RAT measurement to NR.

Conclusions

In this paper we provided our views on reply LS for [1].

Proposal: RAN4 not to make clarification on whether interruption is allowed when UE reports “no gap” in Rel-16 field for LTE inter-RAT measurement to NR.

Reference

- R2-2306919, Reply LS on measurements without gap, RAN2

Annex: draft Reply LS on measurements without gap

3GPP TSG-RAN WG4 Meeting #107 R4-22xxxxx Incheon, KR, 22 – 26 May, 2023 Title: Reply LS on measurements without gap Response to: R2-2306919 Release: Release 18 Work Item: NR_MG_enh2-Core Source: RAN4 To: RAN2 Cc: Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 thanks RAN2 for the LS R2-2306919. RAN4 discussed the question in R2-2306919, and concluded that RAN4 will not make clarification on whether interruption is allowed when UE reports “no gap” in Rel-16 field for LTE inter-RAT measurement to NR. RAN4 would like to further note that - There is no requirement in R16 RAN4 specifications for inter-RAT NR measurement with interRAT-NeedForGapsNR. - The WI NR_MG_enh2-Core is for R18, and the requirements only apply to R18 UE. RAN4 respectfully asks RAN2 to take the above information into account. 2. Actions: To RAN1: RAN4 respectfully asks RAN2 to take the above information into account. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #108-bis Oct 09 – Oct 13, 2023 Xiamen, China RAN WG4 Meeting #109 Nov 13 – Nov 17, 2023 Chicago, US