3GPP TSG-RAN WG4 Meeting #108	R4-2312835

Toulouse, France, 21 – 25 August, 2023

Title: 	Reply LS on LPHAP

Source: 	Huawei, HiSilicon

Agenda item:	8.22.3.1

Document for:	Approval

Introduction

RRM requirements for LPHAP are discussed in RAN4#107, and outcomes are captured in WF [1].

RAN4 received LS from RAN2 [2] related to eDRX cycle which requires RAN4 confirmation. In this paper we will provide reply LS for [2].

Discussion

In [2] the following information is provided related to the supported eDRX cycles.

During the discussion on the eDRX cycle for eRedCap in RRC_INACTIVE, it was already agreed in eRedCap WI that the long eDRX cycle values are the same as idle eDRX (i.e., 2 hyperframes, 4 hyperframes, 8 hyperframes, 16 hyperframes, 32 hyperframes, 64 hyperframes, 128 hyperframes, 256 hyperframes, 512 hyperframes, 1024 hyperframes). To RAN1/RAN4: ACTION: RAN2 respectfully asks RAN1/RAN4 to confirm whether the eRedCap agreed eDRX cycle lengths are sufficient for positioning in RRC_INACTIVE.

Long eDRX cycles are introduced to allow UE power saving, i.e. UE can perform PRS measurement or SRS transmission with an interval aligned with the service requirements of LPHAP use case 6 (positioning interval of 15-30 seconds), while paging monitoring can be performed with a much larger interval.

We believe RAN1 is more proper to comment from power consumption point of view, e.g. whether existing eDRX cycles are sufficient to reach a battery life of 6-12 months with positioning interval of 15-30 seconds.

From RAN4 perspective, we do not see any issue with the existing eDRX cycles, and we suggest RAN4 to confirm this to RAN2.

Proposal: RAN4 to confirm that the eRedCap agreed eDRX cycle lengths are sufficient for positioning in RRC_INACTIVE.

Conclusions

In this paper we provided our views on reply LS for [2].

Proposal: RAN4 to confirm that the eRedCap agreed eDRX cycle lengths are sufficient for positioning in RRC_INACTIVE.

Reference

- R4-2310074, WF on NR Positioning – LPHAP, Huawei, HiSilicon

- R2-2306841, LS on LPHAP, RAN2

Annex: draft Reply LS on LPHAP

3GPP TSG-RAN WG4 Meeting #107 R4-22xxxxx Incheon, KR, 22 – 26 May, 2023 Title: Reply LS on LPHAP Response to: R2-2306841 Release: Release 18 Work Item: NR_pos_enh2 Source: RAN4 To: RAN2 Cc: RAN1, RAN3 Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 thanks RAN2 for the LS R2-2306841. RAN4 discussed the eDRX cycle length, and concluded that the eRedCap agreed eDRX cycle lengths are sufficient for positioning in RRC_INACTIVE, from RAN4 perspective. RAN4 respectfully asks RAN2 to take the above information into account. 2. Actions: To RAN1: RAN4 respectfully asks RAN2 to take the above information into account. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #108-bis Oct 09 – Oct 13, 2023 Xiamen, China RAN WG4 Meeting #109 Nov 13 – Nov 17, 2023 Chicago, US