3GPP TSG-RAN WG4 Meeting #108	R4-2312836

Toulouse, France, 21 – 25 August, 2023

Title: 	Discussion on RRM requirements for SL positioning

Source: 	Huawei, HiSilicon

Agenda item:	8.22.3.2

Document for:	Approval

Introduction

RRM requirements for SL positioning are discussed in RAN4#107, and outcomes are captured in WF [1]. Based on [1] the following issues need to be further discussed.

- Measurement period requirement

- Report mapping

In this paper we will provide our views on RRM requirements for SL positioning.

Discussion

Measurement period requirement

Issue 1-1-1: Measurement types to be defined for SL positioning measurement requirements: Agreements: - RAN4 to start with defining measurement period requirements for SL-PRS based RSTD measurement.

We support to define measurement period requirements for all measurement types.

For Uu positioning, there is no core requirements for UL-RTOA or AOA/ZOA because they are measured by gNB and it was agreed to not define core requirements for gNB measurements. For SL positioning, SL-PRS RTOA and SL-PRS AOA/ZOA are also measured by UE, and we do not see any issue in defining measurement period for them.

Proposal 1: RAN4 to define measurement period requirements for all measurement types.

Issue 1-1-3: Parameters in the measurement period requirements: GTW Agreements - The measurements period requirements equation is FFS - Number of samples for positioning measurements is FFS - Option 1: 1 - Option 2: 2 - Option 3: 3 - Option 4: 4 - The number of Rx beams is equal to 1 in FR1.

We support to define SL PRS measurement period requirements using 1 sample as baseline.

- For many use cases of SL positioning latency is an important requirement.

- Symbol for AGC is reserved in SL frame structure, so a dedicated sample for AGC is not needed.

- Es/Iot side condition for SL PRS measurement is better because anchor and target UEs are not supposed to be far apart. As observed in our simulation results, 1 sample measurement can give reasonable performance in most cases under -6dB Es/Iot.

- Based on the below conclusion from RAN1, transmission of SL PRS is not periodic as in NR positioning. Instead, it is always associated with PSCCH which is quite dynamic depending on the resource grant from gNB/eNB for scheme 1 or the sensing outcome for scheme 2. Although filtering among resource samples is still possible in certain cases, defining requirements based on such assumption can be quite complex.

Conclusion For a dedicated or shared resource pool, at least the following characteristics are NOT included as part of characteristics of a SL PRS resource: - Periodicity, number of instances/repetitions of SL PRS

All above factors make 1 sample measurement suited for SL PRS measurement. We are open to further discussing whether to support multiple samples in defining requirements.

Proposal 2: Measurement based on single sample is used as baseline for SL PRS measurement period requirements.

In last meeting, some companies propose to account for SL PRS Processing Capability in the measurement period requirements. For Uu PRS measurement requirements, processing capability {N,T} is considered in the requirements. Also, we notice that in RAN1 feature list, processing capability is also defined for SL PRS (41-1-1). Therefore, it is reasonable to consider it in the RAN4 requirements once the details are finalized in RAN1. For now, we suggest to wait for RAN1 conclusion before further discussion in RAN4.

Proposal 3: RAN4 waits for RAN1 conclusion on whether and how to account for SL PRS Processing Capability in the measurement period requirements.

Issue 1-1-4: Impact of SL-DRX Agreements: - RAN4 to start with defining the measurement period requirements based on non-DRX. - FFS whether the defined requirements apply for non-DRX as well as SL DRX case.

As discussed above, SL PRS transmission is always associated with PSCCH, i.e. Rx UE only measures SL PRS when it decodes the associated PSCCH. DRX will impact PSCCH reception and also SL PRS transmission, but depending on how non-DRX requirements are defined, they may or may not apply to DRX. We suggest RAN4 to first focus on finalizing non-DRX requirements before discussing DRX requirements.

Proposal 4: RAN4 to discuss requirements for DRX after requirements for non-DRX are stable.

Issue 1-1-5: Impact of other channels/signals Agreements: - RAN4 to discuss possible impacts of sharing/collision between SL PRS measurement and other SL measurements/signals on measurement period requirements based on RAN1 conclusions on this topic.

Issue 1-1-7: Impact of Uu link connection Agreements: - RAN4 to discuss the impact of Uu link mobility on SL positioning measurements, e.g., due to handover, RLF or RRC re-establishment in the Uu connection to gNB.

Based on RAN1 agreements, SL PRS are TDM-ed with other SL channels and signals, so in most cases SL PRS measurements will not collide with other SL measurements or SL control/data. There could be some exceptions, e.g. when UE searches for new SyncRef UE or measure PSBCH RSRP of detected SyncRef UE, it may cause dropping of SL Rx/Tx in certain cases as defined in cl. 12.4. We believe SL PRS measurement requirements should not apply in such cases.

Similarly, some Uu operation may cause interruption to SL Rx/Tx as defined in cl. 12.7. SL PRS measurement requirements should not apply in case SL PRS Rx/Tx is interrupted due to those Uu operation. In last meeting some companies propose to consider impact of Uu mobility. Since impact of Uu mobility is not considered in existing SL interruption requirements, we understand there is no need to defined new requirement for it only related to SL PRS measurement.

Proposal 5: SL PRS measurement requirements apply provided that reception of the slots containing SL PRS is not dropped due to other SL procedure or interrupted due to Uu operation.

Issue 1-1-6: Impact of network coverage change Agreements: - RAN4 to discuss the impact of a change in coverage status (e.g., going between in-coverage, out-of-coverage, partial coverage) on the measurement period requirements and UE behaviour for SL positioning.

So far, it is not fully clear to us how coverage status change would impact SL PRS measurement. Depending on how basic requirements are defined, they may or may not apply when there is coverage status change during the measurement period, and maybe some applicability condition is needed. We suggest RAN4 to first focus on finalizing basic requirements before discussing requirements for transition.

Proposal 6: RAN4 to discuss the impact coverage status change after requirements for non-transition case are stable.

Issue 1-2-3: The scenarios on synchronization reference resource change Agreements: - FFS whether to differentiate the possible scenarios with the synchronization reference source change. - Scenario A: Impact on an SL measurement due to the synch change in the same SL UE performing the measurement: - Receiving UE for unidirectional measurements - Receiving and transmitting UE for bidirectional measurements - Scenario B: Impact on an SL measurement due to the synch change in another SL UE (tx or ) involved in the same measurement: - Transmitting UE for unidirectional or bidirectional measurements - Scenario C: Impact on SL measurements at multiple SL UEs which are sharing the same common synchronization reference source, due to the change of the common synchronization source, - Scenario D: Impact on an SL positioning result due to the change in another SL UE involved in the same positioning session but not necessarily the same SL measurement (multi-RTT scenario). Issue 1-2-4: UE behavior and the impact on requirements when synchronization reference source change occurs Way forward: - Proposal 1: - The measurement period requirements are not impacted due to synchronization reference source change. - Proposal 2: - At least SL PRS timing measurements are impacted by the synchronization reference source. - Proposal 3: - When the synchronization reference source changes during the on-going SL measurement or right before the SL measurement: - Alt 1: Delaying/postponing the start of the synch change until the SL measurement is complete - Alt 2: Dropping or restarting the impacted SL measurement - Proposal 4: - RAN4 to discuss the impact of transition (sync source change) after requirements for non-transition case are stable.

In our view, synchronization reference source change is similar to HO in Uu positioning. We suggest RAN4 to focus on the scenario where synchronization reference source change occurs at the UE performing the measurement, i.e. the target UE. For synchronization reference source change at the anchor UE, we believe RAN1 will address the scenario e.g. with the following agreements.

Agreement Support at least the following mechanism to mitigate the impact of synchronization errors between anchor UEs for SL-TDoA based measurement - Exchange of synchronization information of anchor UEs between a UE and LMF or another UE. - FFS detailed synchronization information. E.g: synchronization source, relative time difference (RTD), synchronization quality information - FFS other mechanisms

In Uu positioning, when HO occurs during the measurement,

- For RSTD and RSRP(P), UE will continue the measurement but measurement period can be longer

- For Rx-Tx, UE will restart the measurement

On SL, for unidirectional measurements (including RSTD, RSRP(P), RTOA, AoA/ZoA), it is unclear if synchronization reference source change would impact the measurement. For example, if the measurement is single shot, then there would be no impact. For SL RSTD the two SL PRS resources from two anchor UEs may be measured at different slots and there can be synchronization reference source change in between, but we understand this is similar to Uu RSTD measurement with HO.

For bidirectional measurement (Rx-Tx), RAN1 agreed that the actual SL-PRS transmission time may be used. There may be an impact if synchronization reference source change occurs between the Rx and Tx. In such a case, the accuracy of RTT positioning may be impacted by the synchronization error, and it can be difficult to define accuracy requirement for Rx-Tx. It is reasonable to allow UE to restart the measurement in this case.

Agreement For definition of SL-PRS based Rx-Tx measurement, the actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement if the UE optionally reports the Tx time information, otherwise use the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu. - FFS: details of the Tx time information - FFS: whether additionally the network or LMF can request the UE to report the Tx time information - Note: the value of Rx-Tx measurement is within [-0.5 0.5] ms

Proposal 7: UE is allowed to restart the Rx-Tx measurement if synchronization reference source change occurs between the Rx and Tx.

Issue 1-1-8: initiation/cease of SL PRS tx Agreements: - RAN4 to discuss whether and how to define the requirements for initiation/cease of SL PRS transmissions for positioning.

RAN4 has defined requirements for initiation/cease of SL SSB transmission. The requirements are mainly for the evaluation of the serving cell RSRP or the PSBCH-RSRP of the selected SyncRef UE. This is based on the procedure defined in cl. 5.8.5.2 of 38.331.

For SL PRS, RAN2 has not defined similar condition for SL PRS transmission based RSRP. As such, we see no need to define requirements for initiation/cease of SL PRS transmissions as it is purely functional.

Proposal 8: RAN4 to deprioritize defining requirements for initiation/cease of SL PRS transmissions.

Report mapping

Issue 1-3-2: Report mapping Way forward: - Option 1: - For SL-PRS RSRP, SL-PRS RSRPP, SL-PRS AOA/ZOA, SL-PRS RSTD, SL-PRS RTOA, the report mapping requirements including reporting range, resolution step can be same as Rel-16/Rel-17 report mapping requirement. - Option 2: - The measurement ranges for SL positioning measurements (except for SL-PRS AOA/ZOA) can be different compared to those specified for Uu positioning measurements. RAN4 to discuss the relevant range for each SL positioning measurement. - Option 3: - Reuse the UL AoA measurement report mapping in 38.133 for SL AoA measurements.

Based on RAN1 conclusions on the measurement definition for SL positioning, we understand the existing report mapping of Uu positioning measurements for all the corresponding SL positioning measurements, both the range and the granularity for the reporting.

Proposal 9: Reuse the existing report mapping of Uu positioning measurements for the corresponding SL positioning measurements.

RAN4 needs to inform the decision on report mapping to RAN2 as it will impact the signalling. We provide a draft LS in the Annex.

Conclusions

In this paper we provided our views on RRM requirements for SL positioning.

Proposal 1: RAN4 to define measurement period requirements for all measurement types.

Proposal 2: Measurement based on single sample is used as baseline for SL PRS measurement period requirements.

Proposal 3: RAN4 waits for RAN1 conclusion on whether and how to account for SL PRS Processing Capability in the measurement period requirements.

Proposal 4: RAN4 to discuss requirements for DRX after requirements for non-DRX are stable.

Proposal 5: SL PRS measurement requirements apply provided that reception of the slots containing SL PRS is not dropped due to other SL procedure or interrupted due to Uu operation.

Proposal 6: RAN4 to discuss the impact coverage status change after requirements for non-transition case are stable.

Proposal 7: UE is allowed to restart the Rx-Tx measurement if synchronization reference source change occurs between the Rx and Tx.

Proposal 8: RAN4 to deprioritize defining requirements for initiation/cease of SL PRS transmissions.

Proposal 9: Reuse the existing report mapping of Uu positioning measurements for the corresponding SL positioning measurements.

Reference

- R4-2310163, WF on NR Positioning – SL positioning and CPP, CATT

Annex: draft LS on report mapping for SL positioning measurements

3GPP TSG-RAN WG4 Meeting #107 R4-22xxxxx Incheon, KR, 22 – 26 May, 2023 Title: LS on report mapping for SL positioning measurements Response to: Release: Release 18 Work Item: NR_pos_enh2 Source: RAN4 To: RAN2 Cc: RAN1 Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 discussed the report mapping for SL positioning measurement, and agreed that existing report mapping of Uu positioning measurements are reused for the corresponding SL positioning measurements. RAN4 respectfully asks RAN2 to take the above information into account. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account when defining signalling support for SL positioning. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #108-bis Oct 09 – Oct 13, 2023 Xiamen, China RAN WG4 Meeting #109 Nov 13 – Nov 17, 2023 Chicago, US