3GPP TSG-RAN WG4 Meeting #108	R4-2312837

Toulouse, France, 21 – 25 August, 2023

Title: 	Simulation results for SL positioning

Source: 	Huawei, HiSilicon

Agenda item:	8.22.3.2

Document for:	Discussion

Introduction

RRM requirements for SL positioning are discussed in RAN4#107, and outcomes are captured in WF [1]. Besides, the simulation assumption for evaluating the link level performance of SL PRS measurement is approved in [2].

In this paper we will provide our initial simulation results for SL positioning.

Discussion

In our simulation, we assume the 3 anchor UEs are transmitting SL-RPS to the target UE in the same set of the symbols, and the SL-PRS are multiplexed with different comb offsets. PSCCH decoding is not modeled, and an initial timing uncertainty of CP/2 is applied to SL-PRS.

SL RSTD

Our simulation results for SL RSTD are shown in Table 1-3 for AWGN, TDL-A and TDL-B channels.

Table 1: SL RSTD error for AWGN

| Es/Iot (dB) | SCS (kHz) | RB num | Sym num | Sample num |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 |
| (-3,-6,-6) | 15 | 52 | 4 | 101 | 71 | 62 | 51 |
|  |  |  | 2 | 143 | 101 | 86 | 71 |
|  |  | 104 | 4 | 51 | 36 | 32 | 26 |
|  |  |  | 2 | 71 | 51 | 43 | 36 |
|  | 30 | 24 | 4 | 109 | 77 | 66 | 54 |
|  |  |  | 2 | 154 | 109 | 94 | 77 |
|  |  | 48 | 4 | 54 | 39 | 34 | 28 |
|  |  |  | 2 | 77 | 54 | 47 | 39 |
|  |  | 96 | 4 | 28 | 21 | 17 | 15 |
|  |  |  | 2 | 39 | 28 | 24 | 21 |
|  | 60 | 24 | 4 | 54 | 39 | 34 | 28 |
|  |  |  | 2 | 77 | 54 | 47 | 39 |
| (0,-6,-6) | 15 | 52 | 4 | 97 | 68 | 59 | 49 |
|  |  |  | 2 | 137 | 97 | 83 | 68 |
|  |  | 104 | 4 | 49 | 34 | 31 | 25 |
|  |  |  | 2 | 68 | 49 | 41 | 34 |
|  | 30 | 24 | 4 | 104 | 74 | 63 | 52 |
|  |  |  | 2 | 148 | 104 | 90 | 74 |
|  |  | 48 | 4 | 52 | 38 | 32 | 27 |
|  |  |  | 2 | 74 | 52 | 45 | 38 |
|  |  | 96 | 4 | 27 | 20 | 16 | 14 |
|  |  |  | 2 | 38 | 27 | 23 | 20 |
|  | 60 | 24 | 4 | 52 | 38 | 32 | 27 |
|  |  |  | 2 | 74 | 52 | 45 | 38 |

Table 2: SL RSTD error for TDL-A

| Es/Iot (dB) | SCS (kHz) | RB num | Sym num | Sample num |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 |
| (-3,-6,-6) | 15 | 52 | 4 | 224 | 213 | 192 | 172 |
|  |  |  | 2 | 241 | 229 | 206 | 185 |
|  |  | 104 | 4 | 131 | 124 | 112 | 101 |
|  |  |  | 2 | 128 | 122 | 109 | 98 |
|  | 30 | 24 | 4 | 313 | 297 | 268 | 241 |
|  |  |  | 2 | 999 | 326 | 293 | 264 |
|  |  | 48 | 4 | 168 | 160 | 144 | 129 |
|  |  |  | 2 | 215 | 204 | 184 | 165 |
|  |  | 96 | 4 | 77 | 73 | 66 | 59 |
|  |  |  | 2 | 68 | 65 | 58 | 52 |
|  | 60 | 24 | 4 | 223 | 212 | 191 | 172 |
|  |  |  | 2 | 999 | 277 | 249 | 224 |
| (0,-6,-6) | 15 | 52 | 4 | 220 | 209 | 188 | 169 |
|  |  |  | 2 | 237 | 225 | 203 | 182 |
|  |  | 104 | 4 | 133 | 126 | 114 | 102 |
|  |  |  | 2 | 135 | 128 | 115 | 104 |
|  | 30 | 24 | 4 | 301 | 286 | 257 | 232 |
|  |  |  | 2 | 999 | 299 | 269 | 242 |
|  |  | 48 | 4 | 165 | 157 | 141 | 127 |
|  |  |  | 2 | 180 | 171 | 154 | 139 |
|  |  | 96 | 4 | 74 | 70 | 63 | 57 |
|  |  |  | 2 | 80 | 76 | 68 | 62 |
|  | 60 | 24 | 4 | 213 | 202 | 182 | 164 |
|  |  |  | 2 | 999 | 230 | 207 | 186 |

Table 3: SL RSTD error for TDL-B

| Es/Iot (dB) | SCS (kHz) | RB num | Sym num | Sample num |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 |
| (-3,-6,-6) | 15 | 52 | 4 | 391 | 371 | 353 | 335 |
|  |  |  | 2 | 512 | 486 | 462 | 439 |
|  |  | 104 | 4 | 143 | 136 | 129 | 123 |
|  |  |  | 2 | 138 | 131 | 125 | 118 |
|  | 30 | 24 | 4 | 611 | 580 | 551 | 524 |
|  |  |  | 2 | 999 | 630 | 599 | 569 |
|  |  | 48 | 4 | 163 | 155 | 147 | 140 |
|  |  |  | 2 | 201 | 191 | 181 | 172 |
|  |  | 96 | 4 | 152 | 144 | 137 | 130 |
|  |  |  | 2 | 156 | 148 | 141 | 134 |
|  | 60 | 24 | 4 | 275 | 261 | 248 | 236 |
|  |  |  | 2 | 999 | 305 | 290 | 275 |
| (0,-6,-6) | 15 | 52 | 4 | 379 | 360 | 342 | 325 |
|  |  |  | 2 | 499 | 474 | 450 | 428 |
|  |  | 104 | 4 | 144 | 137 | 130 | 123 |
|  |  |  | 2 | 140 | 133 | 126 | 120 |
|  | 30 | 24 | 4 | 570 | 542 | 514 | 489 |
|  |  |  | 2 | 999 | 596 | 566 | 538 |
|  |  | 48 | 4 | 164 | 156 | 148 | 141 |
|  |  |  | 2 | 194 | 184 | 175 | 166 |
|  |  | 96 | 4 | 153 | 145 | 138 | 131 |
|  |  |  | 2 | 159 | 151 | 143 | 136 |
|  | 60 | 24 | 4 | 233 | 221 | 210 | 200 |
|  |  |  | 2 | 999 | 250 | 238 | 226 |

SL Rx-Tx

Our simulation results for SL Rx-Tx are shown in Table 4-6 for AWGN, TDL-A and TDL-B channels.

Table 4: SL Rx-Tx error for AWGN

| Es/Iot (dB) | SCS (kHz) | RB num | Sym num | Sample num |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 |
| -6 | 15 | 52 | 4 | 81 | 57 | 50 | 41 |
|  |  |  | 2 | 114 | 81 | 69 | 57 |
|  |  | 104 | 4 | 41 | 29 | 26 | 21 |
|  |  |  | 2 | 57 | 41 | 35 | 29 |
|  | 30 | 24 | 4 | 87 | 62 | 53 | 44 |
|  |  |  | 2 | 123 | 87 | 75 | 62 |
|  |  | 48 | 4 | 44 | 32 | 27 | 23 |
|  |  |  | 2 | 62 | 44 | 38 | 32 |
|  |  | 96 | 4 | 23 | 17 | 14 | 12 |
|  |  |  | 2 | 32 | 23 | 20 | 17 |
|  | 60 | 24 | 4 | 44 | 32 | 27 | 23 |
|  |  |  | 2 | 62 | 44 | 38 | 32 |
| -3 | 15 | 52 | 4 | 65 | 46 | 40 | 32 |
|  |  |  | 2 | 91 | 65 | 55 | 46 |
|  |  | 104 | 4 | 32 | 23 | 20 | 17 |
|  |  |  | 2 | 46 | 32 | 28 | 23 |
|  | 30 | 24 | 4 | 70 | 49 | 42 | 35 |
|  |  |  | 2 | 98 | 70 | 60 | 49 |
|  |  | 48 | 4 | 35 | 25 | 22 | 18 |
|  |  |  | 2 | 49 | 35 | 30 | 25 |
|  |  | 96 | 4 | 18 | 13 | 11 | 10 |
|  |  |  | 2 | 25 | 18 | 16 | 13 |
|  | 60 | 24 | 4 | 35 | 25 | 22 | 18 |
|  |  |  | 2 | 49 | 35 | 30 | 25 |
| 0 | 15 | 52 | 4 | 54 | 38 | 33 | 27 |
|  |  |  | 2 | 76 | 54 | 46 | 38 |
|  |  | 104 | 4 | 27 | 19 | 17 | 14 |
|  |  |  | 2 | 38 | 27 | 23 | 19 |
|  | 30 | 24 | 4 | 58 | 41 | 35 | 29 |
|  |  |  | 2 | 82 | 58 | 50 | 41 |
|  |  | 48 | 4 | 29 | 21 | 18 | 15 |
|  |  |  | 2 | 41 | 29 | 25 | 21 |
|  |  | 96 | 4 | 15 | 11 | 9 | 8 |
|  |  |  | 2 | 21 | 15 | 13 | 11 |
|  | 60 | 24 | 4 | 29 | 21 | 18 | 15 |
|  |  |  | 2 | 41 | 29 | 25 | 21 |

Table 5: SL Rx-Tx error for TDL-A

| Es/Iot (dB) | SCS (kHz) | RB num | Sym num | Sample num |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 |
| -6 | 15 | 52 | 4 | 129 | 120 | 116 | 109 |
|  |  |  | 2 | 142 | 124 | 117 | 113 |
|  |  | 104 | 4 | 87 | 82 | 80 | 77 |
|  |  |  | 2 | 93 | 87 | 87 | 86 |
|  | 30 | 24 | 4 | 158 | 120 | 110 | 105 |
|  |  |  | 2 | 999 | 189 | 114 | 104 |
|  |  | 48 | 4 | 94 | 88 | 84 | 79 |
|  |  |  | 2 | 105 | 92 | 85 | 82 |
|  |  | 96 | 4 | 50 | 46 | 44 | 43 |
|  |  |  | 2 | 52 | 50 | 48 | 48 |
|  | 60 | 24 | 4 | 114 | 91 | 86 | 84 |
|  |  |  | 2 | 999 | 145 | 87 | 80 |
| -3 | 15 | 52 | 4 | 119 | 108 | 103 | 99 |
|  |  |  | 2 | 125 | 117 | 110 | 105 |
|  |  | 104 | 4 | 72 | 69 | 68 | 66 |
|  |  |  | 2 | 84 | 81 | 79 | 77 |
|  | 30 | 24 | 4 | 128 | 115 | 108 | 104 |
|  |  |  | 2 | 148 | 117 | 110 | 105 |
|  |  | 48 | 4 | 85 | 83 | 77 | 72 |
|  |  |  | 2 | 94 | 89 | 82 | 81 |
|  |  | 96 | 4 | 47 | 44 | 42 | 41 |
|  |  |  | 2 | 50 | 48 | 46 | 45 |
|  | 60 | 24 | 4 | 95 | 91 | 86 | 83 |
|  |  |  | 2 | 118 | 90 | 83 | 81 |
| 0 | 15 | 52 | 4 | 116 | 107 | 104 | 98 |
|  |  |  | 2 | 123 | 116 | 111 | 105 |
|  |  | 104 | 4 | 72 | 69 | 67 | 66 |
|  |  |  | 2 | 83 | 80 | 79 | 76 |
|  | 30 | 24 | 4 | 127 | 114 | 108 | 102 |
|  |  |  | 2 | 129 | 115 | 109 | 104 |
|  |  | 48 | 4 | 85 | 79 | 76 | 69 |
|  |  |  | 2 | 93 | 87 | 80 | 76 |
|  |  | 96 | 4 | 47 | 44 | 42 | 41 |
|  |  |  | 2 | 50 | 48 | 45 | 46 |
|  | 60 | 24 | 4 | 93 | 90 | 85 | 81 |
|  |  |  | 2 | 99 | 91 | 86 | 83 |

Table 6: SL Rx-Tx error for TDL-B

| Es/Iot (dB) | SCS (kHz) | RB num | Sym num | Sample num |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 |
| -6 | 15 | 52 | 4 | 235 | 221 | 195 | 186 |
|  |  |  | 2 | 295 | 247 | 217 | 199 |
|  |  | 104 | 4 | 108 | 93 | 87 | 80 |
|  |  |  | 2 | 114 | 103 | 99 | 96 |
|  | 30 | 24 | 4 | 396 | 241 | 219 | 208 |
|  |  |  | 2 | 999 | 301 | 219 | 206 |
|  |  | 48 | 4 | 128 | 116 | 111 | 110 |
|  |  |  | 2 | 148 | 133 | 156 | 159 |
|  |  | 96 | 4 | 107 | 91 | 82 | 78 |
|  |  |  | 2 | 113 | 101 | 96 | 90 |
|  | 60 | 24 | 4 | 166 | 121 | 121 | 128 |
|  |  |  | 2 | 999 | 296 | 130 | 129 |
| -3 | 15 | 52 | 4 | 198 | 197 | 185 | 168 |
|  |  |  | 2 | 227 | 221 | 204 | 187 |
|  |  | 104 | 4 | 102 | 88 | 82 | 76 |
|  |  |  | 2 | 109 | 95 | 91 | 87 |
|  | 30 | 24 | 4 | 248 | 221 | 203 | 195 |
|  |  |  | 2 | 350 | 244 | 224 | 206 |
|  |  | 48 | 4 | 121 | 108 | 102 | 99 |
|  |  |  | 2 | 127 | 113 | 110 | 108 |
|  |  | 96 | 4 | 106 | 90 | 82 | 78 |
|  |  |  | 2 | 108 | 95 | 89 | 83 |
|  | 60 | 24 | 4 | 119 | 108 | 105 | 100 |
|  |  |  | 2 | 146 | 116 | 120 | 117 |
| 0 | 15 | 52 | 4 | 192 | 185 | 178 | 166 |
|  |  |  | 2 | 214 | 204 | 194 | 177 |
|  |  | 104 | 4 | 101 | 88 | 82 | 76 |
|  |  |  | 2 | 108 | 95 | 90 | 87 |
|  | 30 | 24 | 4 | 235 | 211 | 198 | 187 |
|  |  |  | 2 | 260 | 229 | 209 | 200 |
|  |  | 48 | 4 | 120 | 107 | 100 | 96 |
|  |  |  | 2 | 123 | 109 | 104 | 102 |
|  |  | 96 | 4 | 107 | 90 | 82 | 77 |
|  |  |  | 2 | 108 | 95 | 89 | 82 |
|  | 60 | 24 | 4 | 114 | 102 | 98 | 95 |
|  |  |  | 2 | 119 | 108 | 107 | 104 |

SL PRS-RSRP

Our simulation results for SL PRS-RSRP are shown in Table 7-9 for AWGN, TDL-A and TDL-B channels.

Table 7: SL PRS-RSRP error for AWGN

| Es/Iot (dB) | SCS (kHz) | RB num | Sym num | Sample num |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 |
| -6 | 15 | 52 | 4 | 1.2 | 0.9 | 0.8 | 0.7 |
|  |  |  | 2 | 2.0 | 1.6 | 1.5 | 1.3 |
|  |  | 104 | 4 | 0.8 | 0.6 | 0.5 | 0.4 |
|  |  |  | 2 | 1.1 | 0.8 | 0.7 | 0.6 |
|  | 30 | 24 | 4 | 2.2 | 1.9 | 1.8 | 1.7 |
|  |  |  | 2 | 3.0 | 2.5 | 2.3 | 2.2 |
|  |  | 48 | 4 | 1.4 | 1.1 | 1.0 | 0.9 |
|  |  |  | 2 | 2.3 | 2.0 | 1.8 | 1.7 |
|  |  | 96 | 4 | 0.9 | 0.7 | 0.6 | 0.6 |
|  |  |  | 2 | 1.4 | 1.1 | 1.0 | 0.9 |
|  | 60 | 24 | 4 | 4.2 | 3.8 | 3.6 | 3.6 |
|  |  |  | 2 | 4.8 | 4.4 | 4.2 | 4.1 |
| -3 | 15 | 52 | 4 | 0.8 | 0.6 | 0.5 | 0.5 |
|  |  |  | 2 | 1.2 | 0.9 | 0.8 | 0.7 |
|  |  | 104 | 4 | 0.7 | 0.6 | 0.5 | 0.5 |
|  |  |  | 2 | 0.8 | 0.6 | 0.5 | 0.5 |
|  | 30 | 24 | 4 | 1.2 | 0.9 | 0.8 | 0.7 |
|  |  |  | 2 | 1.7 | 1.2 | 1.1 | 1.0 |
|  |  | 48 | 4 | 0.9 | 0.7 | 0.7 | 0.6 |
|  |  |  | 2 | 1.2 | 0.8 | 0.7 | 0.6 |
|  |  | 96 | 4 | 0.8 | 0.7 | 0.6 | 0.6 |
|  |  |  | 2 | 0.9 | 0.7 | 0.6 | 0.6 |
|  | 60 | 24 | 4 | 1.2 | 0.8 | 0.7 | 0.6 |
|  |  |  | 2 | 1.7 | 1.2 | 1.0 | 0.9 |
| 0 | 15 | 52 | 4 | 0.7 | 0.5 | 0.5 | 0.5 |
|  |  |  | 2 | 0.8 | 0.6 | 0.5 | 0.4 |
|  |  | 104 | 4 | 0.6 | 0.5 | 0.5 | 0.5 |
|  |  |  | 2 | 0.7 | 0.5 | 0.5 | 0.5 |
|  | 30 | 24 | 4 | 0.8 | 0.6 | 0.5 | 0.4 |
|  |  |  | 2 | 1.2 | 0.8 | 0.7 | 0.6 |
|  |  | 48 | 4 | 0.8 | 0.7 | 0.6 | 0.6 |
|  |  |  | 2 | 0.9 | 0.7 | 0.6 | 0.6 |
|  |  | 96 | 4 | 0.7 | 0.6 | 0.6 | 0.6 |
|  |  |  | 2 | 0.8 | 0.6 | 0.6 | 0.5 |
|  | 60 | 24 | 4 | 0.9 | 0.7 | 0.6 | 0.5 |
|  |  |  | 2 | 1.2 | 0.8 | 0.7 | 0.6 |

Table 8: SL PRS-RSRP error for TDL-A

| Es/Iot (dB) | SCS (kHz) | RB num | Sym num | Sample num |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 |
| -6 | 15 | 52 | 4 | 1.5 | 1.2 | 1.0 | 0.9 |
|  |  |  | 2 | 2.4 | 1.7 | 1.7 | 1.5 |
|  |  | 104 | 4 | 1.2 | 0.9 | 0.9 | 0.8 |
|  |  |  | 2 | 1.6 | 1.1 | 0.9 | 0.8 |
|  | 30 | 24 | 4 | 2.9 | 2.4 | 2.3 | 2.1 |
|  |  |  | 2 | 999 | 5.1 | 3.9 | 3.5 |
|  |  | 48 | 4 | 1.6 | 1.1 | 0.9 | 0.9 |
|  |  |  | 2 | 2.6 | 2.1 | 2.0 | 1.8 |
|  |  | 96 | 4 | 1.2 | 1.0 | 0.9 | 0.8 |
|  |  |  | 2 | 1.6 | 1.1 | 1.0 | 0.9 |
|  | 60 | 24 | 4 | 4.6 | 4.2 | 4.0 | 3.9 |
|  |  |  | 2 | 999 | 6.8 | 5.6 | 5.5 |
| -3 | 15 | 52 | 4 | 1.2 | 0.9 | 0.9 | 0.8 |
|  |  |  | 2 | 1.4 | 1.0 | 0.9 | 0.8 |
|  |  | 104 | 4 | 1.0 | 0.9 | 0.8 | 0.7 |
|  |  |  | 2 | 1.2 | 0.9 | 0.8 | 0.8 |
|  | 30 | 24 | 4 | 1.6 | 1.1 | 0.9 | 0.9 |
|  |  |  | 2 | 2.4 | 1.8 | 1.6 | 1.3 |
|  |  | 48 | 4 | 1.3 | 1.1 | 1.0 | 0.9 |
|  |  |  | 2 | 1.5 | 1.2 | 1.0 | 0.9 |
|  |  | 96 | 4 | 1.1 | 0.9 | 0.8 | 0.8 |
|  |  |  | 2 | 1.3 | 1.0 | 0.9 | 0.9 |
|  | 60 | 24 | 4 | 1.5 | 1.1 | 0.9 | 0.9 |
|  |  |  | 2 | 2.4 | 1.8 | 1.5 | 1.3 |
| 0 | 15 | 52 | 4 | 1.0 | 0.8 | 0.8 | 0.7 |
|  |  |  | 2 | 1.1 | 0.8 | 0.7 | 0.7 |
|  |  | 104 | 4 | 0.8 | 0.7 | 0.7 | 0.6 |
|  |  |  | 2 | 0.9 | 0.8 | 0.7 | 0.7 |
|  | 30 | 24 | 4 | 1.2 | 0.9 | 0.8 | 0.8 |
|  |  |  | 2 | 1.6 | 1.2 | 1.0 | 0.9 |
|  |  | 48 | 4 | 1.1 | 0.9 | 0.9 | 0.8 |
|  |  |  | 2 | 1.2 | 1.0 | 0.9 | 0.8 |
|  |  | 96 | 4 | 0.9 | 0.8 | 0.7 | 0.7 |
|  |  |  | 2 | 1.0 | 0.9 | 0.8 | 0.8 |
|  | 60 | 24 | 4 | 1.2 | 0.9 | 0.8 | 0.8 |
|  |  |  | 2 | 1.6 | 1.2 | 1.1 | 1.0 |

Table 9: SL PRS-RSRP error for TDL-B

| Es/Iot (dB) | SCS (kHz) | RB num | Sym num | Sample num |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 |
| -6 | 15 | 52 | 4 | 3.0 | 2.2 | 1.8 | 1.6 |
|  |  |  | 2 | 4.4 | 3.1 | 2.6 | 2.3 |
|  |  | 104 | 4 | 2.8 | 2.1 | 1.7 | 1.5 |
|  |  |  | 2 | 3.9 | 2.8 | 2.4 | 2.1 |
|  | 30 | 24 | 4 | 3.4 | 2.2 | 1.9 | 1.7 |
|  |  |  | 2 | 999 | 5.3 | 4.1 | 3.7 |
|  |  | 48 | 4 | 2.6 | 1.8 | 1.4 | 1.3 |
|  |  |  | 2 | 4.1 | 3.0 | 2.6 | 2.4 |
|  |  | 96 | 4 | 2.2 | 1.6 | 1.3 | 1.2 |
|  |  |  | 2 | 3.3 | 2.4 | 1.9 | 1.8 |
|  | 60 | 24 | 4 | 4.2 | 3.8 | 3.6 | 3.6 |
|  |  |  | 2 | 999 | 7.2 | 5.9 | 5.4 |
| -3 | 15 | 52 | 4 | 1.9 | 1.5 | 1.3 | 1.3 |
|  |  |  | 2 | 1.5 | 1.1 | 0.9 | 0.8 |
|  |  | 104 | 4 | 1.7 | 1.4 | 1.3 | 1.2 |
|  |  |  | 2 | 1.3 | 1.0 | 0.9 | 0.9 |
|  | 30 | 24 | 4 | 2.0 | 1.6 | 1.4 | 1.3 |
|  |  |  | 2 | 2.4 | 1.7 | 1.5 | 1.3 |
|  |  | 48 | 4 | 2.0 | 1.7 | 1.5 | 1.4 |
|  |  |  | 2 | 1.6 | 1.3 | 1.1 | 1.0 |
|  |  | 96 | 4 | 1.7 | 1.5 | 1.4 | 1.3 |
|  |  |  | 2 | 1.4 | 1.2 | 1.1 | 1.0 |
|  | 60 | 24 | 4 | 2.1 | 1.6 | 1.4 | 1.3 |
|  |  |  | 2 | 2.6 | 1.8 | 1.5 | 1.3 |
| 0 | 15 | 52 | 4 | 1.7 | 1.4 | 1.2 | 1.2 |
|  |  |  | 2 | 1.2 | 0.9 | 0.8 | 0.8 |
|  |  | 104 | 4 | 1.6 | 1.3 | 1.2 | 1.2 |
|  |  |  | 2 | 1.0 | 0.9 | 0.8 | 0.8 |
|  | 30 | 24 | 4 | 1.8 | 1.4 | 1.3 | 1.2 |
|  |  |  | 2 | 1.7 | 1.3 | 1.1 | 1.0 |
|  |  | 48 | 4 | 1.8 | 1.5 | 1.4 | 1.3 |
|  |  |  | 2 | 1.3 | 1.1 | 1.0 | 0.9 |
|  |  | 96 | 4 | 1.5 | 1.3 | 1.2 | 1.2 |
|  |  |  | 2 | 1.2 | 1.0 | 0.9 | 0.8 |
|  | 60 | 24 | 4 | 1.8 | 1.5 | 1.3 | 1.2 |
|  |  |  | 2 | 1.8 | 1.4 | 1.2 | 1.1 |

SL PRS-RSRPP

Our simulation results for SL PRS-RSRPP are shown in Table 10 for 2-tap channels.

Table 10: SL PRS-RSRP error for 2-tap channel

| Es/Iot (dB) | SCS (kHz) | RB num | Sym num | Sample num |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1 | 2 | 3 | 4 |
| -6 | 15 | 52 | 4 | 2.5 | 2.0 | 1.7 | 1.4 |
|  |  |  | 2 | 3.1 | 2.6 | 2.2 | 2.0 |
|  |  | 104 | 4 | 2.1 | 1.7 | 1.4 | 1.2 |
|  |  |  | 2 | 2.4 | 1.9 | 1.7 | 1.5 |
|  | 30 | 24 | 4 | 3.1 | 2.5 | 2.1 | 1.9 |
|  |  |  | 2 | 4.0 | 3.3 | 3.0 | 2.8 |
|  |  | 48 | 4 | 2.5 | 2.0 | 1.7 | 1.5 |
|  |  |  | 2 | 3.2 | 2.7 | 2.4 | 2.1 |
|  |  | 96 | 4 | 2.5 | 2.1 | 1.9 | 1.7 |
|  |  |  | 2 | 2.6 | 2.1 | 1.8 | 1.6 |
|  | 60 | 24 | 4 | 3.2 | 2.6 | 2.3 | 2.0 |
|  |  |  | 2 | 4.2 | 3.5 | 3.2 | 2.9 |
| -3 | 15 | 52 | 4 | 2.2 | 1.8 | 1.4 | 1.2 |
|  |  |  | 2 | 2.6 | 2.1 | 1.8 | 1.6 |
|  |  | 104 | 4 | 1.8 | 1.4 | 1.2 | 1.0 |
|  |  |  | 2 | 2.0 | 1.6 | 1.3 | 1.1 |
|  | 30 | 24 | 4 | 2.5 | 2.0 | 1.8 | 1.6 |
|  |  |  | 2 | 3.2 | 2.6 | 2.3 | 2.0 |
|  |  | 48 | 4 | 2.0 | 1.6 | 1.3 | 1.1 |
|  |  |  | 2 | 2.4 | 1.8 | 1.5 | 1.3 |
|  |  | 96 | 4 | 1.8 | 1.5 | 1.3 | 1.1 |
|  |  |  | 2 | 2.0 | 1.6 | 1.3 | 1.1 |
|  | 60 | 24 | 4 | 2.4 | 1.8 | 1.6 | 1.3 |
|  |  |  | 2 | 3.0 | 2.4 | 2.1 | 1.8 |
| 0 | 15 | 52 | 4 | 1.9 | 1.6 | 1.3 | 1.0 |
|  |  |  | 2 | 2.2 | 1.8 | 1.5 | 1.2 |
|  |  | 104 | 4 | 1.7 | 1.4 | 1.1 | 0.9 |
|  |  |  | 2 | 1.8 | 1.5 | 1.2 | 1.0 |
|  | 30 | 24 | 4 | 2.1 | 1.6 | 1.4 | 1.2 |
|  |  |  | 2 | 2.5 | 2.0 | 1.7 | 1.5 |
|  |  | 48 | 4 | 1.8 | 1.4 | 1.2 | 1.0 |
|  |  |  | 2 | 2.0 | 1.6 | 1.3 | 1.1 |
|  |  | 96 | 4 | 1.7 | 1.5 | 1.3 | 1.1 |
|  |  |  | 2 | 1.8 | 1.5 | 1.3 | 1.1 |
|  | 60 | 24 | 4 | 2.0 | 1.6 | 1.3 | 1.1 |
|  |  |  | 2 | 2.4 | 1.8 | 1.6 | 1.3 |

Observation 1: For SL-PRS with full staggering pattern, the performance is impacted by different factors (BW, Es/Iot, channel model, sample number, SCS) in a similar way as PRS measurement.

Observation 2: For SL-PRS with partial staggering pattern, the performance is worse than full staggering pattern, and can be quite poor for some case with small RB number and single sample.

In general, we support to define accuracy requirements based on single shot measurement from core requirements perspective. As can be seen in the tables above, for most cases, the performance with single shot measurement is reasonable, except for partial staggering with 24 RB and single sample under -6dB Es/Iot and fading channel, where the performance is very poor. In NR positioning, the accuracy for 24 RB are defined only for 4-sample, while single sample requirements apply for >= 48 RB under AWGN channel, and we suggest to follow the same principle.

Proposal 1: Define accuracy requirements for SL-PRS measurement based on single sample for >= 48 RB and AWGN channel.

Conclusions

In this paper we provided our initial simulation results for SL positioning.

Observation 1: For SL-PRS with full staggering pattern, the performance is impacted by different factors (BW, Es/Iot, channel model, sample number, SCS) in a similar way as PRS measurement.

Observation 2: For SL-PRS with partial staggering pattern, the performance is worse than full staggering pattern, and can be quite poor for some case with small RB number and single sample.

Proposal 1: Define accuracy requirements for SL-PRS measurement based on single sample for >= 48 RB and AWGN channel.

Reference

- R4-2310163, WF on NR Positioning – SL positioning and CPP, CATT

- R4-2310077, Simulation assumptions for sidelink positioning, CATT