3GPP TSG-RAN WG4 Meeting #108	R4-2312849

Toulouse, France, 21 – 25 August, 2023

Title: 	Discussion on option B-1-1 for BWP without restriction

Source: 	Huawei, HiSilicon

Agenda item:	8.10.2.2

Document for:	Discussion

Introduction

RRM requirements for option B-1-1 for BWP without restriction are discussed in RAN4#107 and the outcomes are captured in [1]. Based on [1] there is no remaining open issue for L1 or timing requirements for option B-1-1. During RAN#100, the WID is revised [2], and support of L3 intra-frequency measurements without gaps for option B-1-1 is added in a new objective.

- L3 intra-frequency measurements without gaps

In this paper we will provide our views on RRM requirements for option B-1-1 for BWP without restriction.

Discussion

L3 intra-frequency measurements without gaps

2) Specify the necessary requirements to support L3 intra-frequency measurements without gaps (RAN4) - For Option B-1-1 - Specify applicability of legacy intra-frequency L3 measurements without gap and corresponding UE capabilities, if needed. - Note: UEs capable of Option B-1-1 BWP operation can optionally support L3 intra-frequency measurements with gaps. - For Option C - Specify requirements for L3 intra-frequency measurements based on existing RedCap NCD-SSB requirements. - Note: further check in RAN #101 on whether any work is required to support L3 intra-frequency measurements without gaps for Option B-1-2

We understand new applicability for intra-frequency L3 measurements without gap is needed.

Intra-frequency measurement without MG is not supported for option B-1-1 based on current spec. Some clarification on the applicability condition for intra-frequency measurement without MG is needed. As copied below, the applicability condition for intra-frequency measurement without MG is defined in cl. 9.2.1 and it is reasonable to add supporting option B-1-1 as another applicability condition.

The UE can perform intra-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) if - the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement, or - the SSB is completely contained in the active BWP of the UE, or - the active downlink BWP is initial BWP[3]. For UE supporting nr-NeedForGapNCSG-reporting-r17 and indicating NeedForGapNCSG-InfoNR for intra-frequency measurement, - An intra-frequency SSB measurement is defined as measurement without gap if - the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoNR for the intra-frequency measurement, and - the SSB is not completely contained in the active BWP of the UE, and - the active downlink BWP is not an initial BWP [3] The delay requirements are specified in clause 9.2.5

As noted in [2], UEs capable of option B-1-1 can optionally support L3 intra-frequency measurements with gaps. We understand this means UEs capable of option B-1-1 is not mandatorily required to support intra-frequency measurements without gaps, but it is an optional capability under the capability for option B-1-1.

Proposal 1: RAN4 to introduce a new capability to indicate whether UE supports intra-frequency measurements based on CD-SSB outside active BWP without gaps and without interruptions when UE supports option B-1-1 (FG 53-1).

Proposal 2: RAN4 to add supports of option B-1-1 and the new capability in Proposal 1 as one applicability condition for intra-frequency measurement without MG.

Besides, RAN4 also needs to discuss the relation with NFG and NCSG capability. For example, if UE supports option B-1-1 and the new capability for intra-frequency measurements without gaps for a band, it is unreasonable that it reports it needs interruption/NCSG or MG for the intra-frequency measurement, and it will cause confusion to NW.

Proposal 3: RAN4 to discuss NFG/NCSG capability reporting for UE supporting option B-1-1 and the new capability in Proposal 1.

Conclusions

In this paper we provided our views on RRM requirements for option B-1-1 for BWP without restriction.

Proposal 1: RAN4 to introduce a new capability to indicate whether UE supports intra-frequency measurements based on CD-SSB outside active BWP without gaps and without interruptions when UE supports option B-1-1 (FG 53-1).

Proposal 2: RAN4 to add supports of option B-1-1 and the new capability in Proposal 1 as one applicability condition for intra-frequency measurement without MG.

Proposal 3: RAN4 to discuss NFG/NCSG capability reporting for UE supporting option B-1-1 and the new capability in Proposal 1.

Reference

- R4-2310176, WF on NR BWP operation without restriction, vivo

- RP-231486, Revised WID: Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, vivo