3GPP RAN4 Meeting #108	R4-2312883

# Toulouse, FR, August 21st – 25th, 2023

Title: 	Sidelink PSSCH/PSCCH MPR in unlicensed spectrum

Agenda Item:	8.30.2.1.2

Source: 	Qualcomm Incorporated

Document for:	Approval

# Introduction

In this paper we share PSSCH/PSCCH simulation results using the example configurations that have been proposed in email after the May RAN4 meeting.

# Power Class 5 MPR simulations

We have performed PSSCH/PSCCH simulations to determine the allowed MPR. EVM, IBE, SLU ACLR, NR spurious. The MPR value has been quantized to 0.5 dB. The table has 3 columns, one for QPSK and 16QAM, one for 64 QAM, and one for 256 QAM. A blank entry means no simulation was performed.

For convenience the MPR table for SS from 38.101-1 is shown below.

Table: Power class 5 PSSCH/PSCCH simulation results

|  | case | Precoding | BW |  | RB Setup | SCS | QPSK and 16QAM MPR | 64 QAM MPR | 256 QAM MPR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 |  | 105RB0 | 15 | 2.5 |  | 5.0 |
|  | 2 | CP-OFDM | 20 |  | 50RB0 | 30 | 2.5 |  | 5.0 |
|  | 3 | CP-OFDM | 40 |  | 216RB0 | 15 | 2.5 |  | 5.0 |
|  | 4 | CP-OFDM | 60 |  | 160RB0 | 30 | 3.0 |  | 5.0 |
|  | 5 | CP-OFDM | 80 |  | 216RB0 | 30 | 3.0 |  | 5.0 |
|  | 6 | CP-OFDM | 100 |  | 270RB0 | 30 | 3.0 |  | 5.0 |
| Interlaced Allocation Single CC | 7 | CP-OFDM | 20 |  | 1 RB0 every 10RBs | 15 | 3.0 |  |  |
|  | 8 | CP-OFDM | 20 |  | 1RB0 every 5RBs | 30 | 3.0 |  |  |
|  | 9 | CP-OFDM | 40 |  | 1RB0 every 10RBs | 15 | 3.0 |  |  |
|  | 10 | CP-OFDM | 60 |  | 1RB0 every 5RBs | 30 | 3.0 |  |  |
|  | 11 | CP-OFDM | 80 |  | 1RB0 every 5RBs | 30 | 3.0 |  |  |
|  | 12 | CP-OFDM | 100 |  | 1RB0 every 5RBs | 30 | 3.0 |  |  |
| Wide band Full operation | 13 | CP-OFDM | 40 |  | Bitmap 10 | 15 | 2.5 |  |  |
|  | 14 | CP-OFDM | 60 |  | Bitmap 100 | 30 | 2.5 |  |  |
|  | 15 | CP-OFDM | 60 |  | Bitmap 110 | 30 | 3.0 |  |  |
|  | 16 | CP-OFDM | 80 |  | Bitmap 1000 | 30 | 2.5 |  |  |
|  | 17 | CP-OFDM | 80 |  | Bitmap 1100 | 30 | 3.0 |  |  |
|  | 18 | CP-OFDM | 80 |  | Bitmap 1110 | 30 | 3.0 |  |  |
|  | 19 | CP-OFDM | 80 |  | Bitmap 0100 | 30 | 1.5 |  |  |
|  | 20 | CP-OFDM | 80 |  | Bitmap 0110 | 30 | 1.5 |  |  |
|  | 21 | CP-OFDM | 100 |  | Bitmap 10000 | 30 | 2.5 |  |  |
|  | 22 | CP-OFDM | 100 |  | Bitmap 11000 | 30 | 3.0 |  |  |
|  | 23 | CP-OFDM | 100 |  | Bitmap 11100 | 30 | 3.0 |  |  |
|  | 24 | CP-OFDM | 100 |  | Bitmap 11110 | 30 | 3.0 |  |  |
|  | 25 | CP-OFDM | 100 |  | Bitmap 01000 | 30 | 1.5 |  |  |
|  | 26 | CP-OFDM | 100 |  | Bitmap 01100 | 30 | 1.5 |  |  |
|  | 27 | CP-OFDM | 100 |  | Bitmap 01110 | 30 | 1.5 |  |  |
|  | 28 | CP-OFDM | 100 |  | Bitmap 00100 | 30 | 1.5 |  |  |
| Wide band Interlaced operation | 29 | CP-OFDM | 40 |  | Bitmap 10 | 15 | 3 |  |  |
|  | 30 | CP-OFDM | 60 |  | Bitmap 100 | 30 | 3 |  |  |
|  | 31 | CP-OFDM | 60 |  | Bitmap 110 | 30 | 3 |  |  |
|  | 32 | CP-OFDM | 80 |  | Bitmap 1000 | 30 | 3 |  |  |
|  | 33 | CP-OFDM | 80 |  | Bitmap 1100 | 30 | 3 |  |  |
|  | 34 | CP-OFDM | 80 |  | Bitmap 1110 | 30 | 3 |  |  |
|  | 35 | CP-OFDM | 80 |  | Bitmap 0100 | 30 | 1.5 |  |  |
|  | 36 | CP-OFDM | 80 |  | Bitmap 0110 | 30 | 1.5 |  |  |
|  | 37 | CP-OFDM | 100 |  | Bitmap 10000 | 30 | 3 |  |  |
|  | 38 | CP-OFDM | 100 |  | Bitmap 11000 | 30 | 3 |  |  |
|  | 39 | CP-OFDM | 100 |  | Bitmap 11100 | 30 | 3 |  |  |
|  | 40 | CP-OFDM | 100 |  | Bitmap 11110 | 30 | 3 |  |  |
|  | 41 | CP-OFDM | 100 |  | Bitmap 01000 | 30 | 1.5 |  |  |
|  | 42 | CP-OFDM | 100 |  | Bitmap 01100 | 30 | 1.5 |  |  |
|  | 43 | CP-OFDM | 100 |  | Bitmap 01110 | 30 | 1.5 |  |  |
|  | 44 | CP-OFDM | 100 |  | Bitmap 00100 | 30 | 1.5 |  |  |

Proposal: The power class 5 PSSCH/PSCCH simulation results in the table should be considered in the discussion of MPR requirements.

# Conclusions

Proposal: The power class 5 PSSCH/PSCCH simulation results in the table should be considered in the discussion of MPR requirements.

# References

- 3GPP TS 38.101-1 v18.2.0