3GPP TSG-RAN WG4 Meeting # 108				                                         R4-2312899

Toulouse, France, August 21 – August 25, 2023

Agenda Item:	8.16.3

Source:	Xiaomi

Title:	Channel model validation results for Band n28

Document for:	information

1	Introduction

In the Rel-18 MIMO OTA WID [1], bands n1, n5, n8, n28, n77 are listed as first priority for FR1 MIMO OTA performance requirements definition.

In this paper, we provide channel model validation results for band n28. Remaining of the channel model validation results for other bands will be submitted at following RAN4 meetings.

2	Discussion

The channel model validation results are provided in Figs. 1-3 and Table 2. The measurement setup, test procedures, and instrument settings are according to TS 38.151 V17.2.0. The measurement setup is composed by GTS5000 16-probe MIMO OTA chamber and Spirent Vertex channel emulator.

The following parameters of CDL-C UMi channel model have been validated:

Table 1. The validated CDL-C UMi channel model parameters

| Frequency | Band | Parameter |
| --- | --- | --- |
| 722MHz | n28 | Temporal correlation |
|  |  | PDP |
|  |  | Spatial correlation |
|  |  | Cross-polarization |
|  |  | Power Validation |

Fig. 1 Temporal correlation measurement results and reference curves for CDL-C UMi, fc ≤ 2.5 GHz

Fig. 2 PDP measurement results and reference values for CDL-C UMi, fc ≤ 2.5 GHz

Fig. 3 Spatial correlation measurement results and reference curves for CDL-C UMi, fc ≤ 2.5 GHz

Table 2. Cross-polarization verification results for CDL-C UMi, fc ≤ 2.5 GHz

| Frequency | Beam | Reference | Measurement result | Pass/fail limit |
| --- | --- | --- | --- | --- |
| fc = 722 MHz | Beam 1 | Input 1+2: V/H = 0 dB | -0.09 dB | ±1dB |

Table 3. Power verification results for CDL-C UMi, fc =780.5MHz

| Frequency | Integrated Channel Span | Measurement result | Pass/fail limit |
| --- | --- | --- | --- |
| fc = 780.5 MHz | 10MHz | 1.3 dB | ±1.5dB |

Observation 1: The above parameters of CDL-C UMi have been validated: TCF, SCF, PDP, Cross-polarization at 722MHz and power validation at 780.5MHz.

3	Conclusion

In this paper, we provide channel model validation results for band n28.

Observation 1: The parameters of CDL-C UMi have been validated: TCF, SCF, PDP, Cross-polarization at 722MHz and power validation at 780.5MHz.

References

- R4-222668, “R18 New WID: Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) test methodology and requirements for NR UEs”, CAICT, OPPO, Keysight Technologies, 3GPP RAN#97e, Sep. 2022.