3GPP RAN4 Meeting #108	R4-2312904

# Toulouse, FR, August 21st – 25th, 2023

Title: 	n47 Sidelink CA

Agenda Item:	8.30.2.3

Source: 	Qualcomm Incorporated

Document for:	Agreement

# Introduction

Below we show a recap of CA WFs from the last two RAN4 meetings. Below we discuss some of the FFS aspects.

WF from 106-bis-e # Topic 1: General aspects for SL CA ## Sub-topic 1-1: Spec suffix < GTW Agreement>: Capture the requirement for SL intra-band CA under suffix E in RAN4 specification. ## Sub-topic 1-2: Notation for NR SL CA < GTW Agreement>: Notation for NR SL CA is SL_n47X - X can be bandwidth class like B or C ## Sub-topic 1-3: Notation for NR SL CA < GTW Agreement>: All applicable SCS for ITS band n47 is applicable for NR sidelink CA operation: 15kHz, 30kHz and 60kHz # Topic 2: Bandwidth class and aggregated channel bandwidth for SL CA ## Sub-topic 2-1: Channel bandwidth class and aggregated channel BW for sidelink CA < GTW Agreement>: Reuse NR CA bandwidth classes A and B ## Sub-topic 2-2: The maximum aggregated CBW for sidelink CA < GTW Agreement>: The maximum aggregated CBW for sidelink intra-band contiguous CA for n47 is - 70MHz (30MHz+40MHz) ## Sub-topic 2-3: Configurations for sidelink CA < GTW Agreement>: Use Option 4 as baseline - Option 4: NR V2X intra-band con-current operating configuration; NR Band; Interface; Channel bandwidth (MHz) (NOTE 1); Maximum aggregated bandwidth (MHz); Bandwidth combination set. SL_n47B; n47; PC5; 10, 20, 30, 40; 70; 0. n47; 10, 20, 30. NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 in TS38.101-1.. # Topic 3: Tx RF Requirements for SL CA ## Sub-topic 3-1: UE maximum output power for SL CA <Way forward >: For the UE maximum output power of SL(V2X) intra-band contiguous CA, consider PC3 in Rel-18 - FFS: PC2. ## Sub-topic 3-2: MPR/A-MPR simulation assumptions for SL CA <Way forward >: For basic parameters, reuse the simulation assumptions in TR38.785 (Rel-17 enhanced NR sidelink). Other constraints for PSCCH/PSSCH/PSFCH/S-SSB can be assumed based on current RAN1’s agreement Center frequency; 5.9GHz. Bandwidth; per CC: 10/20/30/40MHz aggregated CBW: 20+40MHz/ 30+40MHz. Maximum output power for aggregated CBW; 23dBm. Numerology; 15 kHz/30kHz/60kHz. Modulation per CC; QPSK/16QAM/64QAM/256QAM. Waveform; CP-OFDM. Carrier leakage; 25dBc. IQ image; 25dBc. CIM3; FFS: 45dBc or, 45dBc and 60dBc. PA calibration; PA calibrated to deliver [31dBc] ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case.. ## Sub-topic 3-3: Configured transmitted power for SL intra-band contiguous CA < GTW Agreement>: Specify PEMAX,CA for SL CA - PEMAX,CA is not applied to S-SSB ## Sub-topic 3-4: Methodology on Tx requirements for SL intra-band contiguous CA <Way forward >: Open issue needs further discussion - Option 1: Reusing the existing requirements of NR intra-band contiguous CA - Option 2: LTE intra-band contiguous CA requirements are considered as baseline - Option 3: For further study # Topic 4: Rx RF Requirements for SL CA < GTW Agreement>: Specify reference sensitivity by considering the aggregated channel bandwidth for SL intra-band contiguous CA

From WF in RAN4#107 # Topic 1: General aspect for SL CA ## Sub-topic 1-1: The supported scenario for SL CA < GTW Agreement>: - SL-MIMO and Tx diversity of NR SL intra-band contiguous CA can be considered as 2nd priority - Consider 2Tx antenna architecture in Rel-18 ## Sub-topic 1-2: The spec structure for SL CA The way to capture system parameters and requirements for SL CA, as examples: 5.2E Operating band for V2X 5.2E.1 V2X operating bands 5.2E.2 V2X operating bands for con-current operation 5.2E.3 Operating bands for intra-band contiguous Sidelink CA 6.2E.2 UE maximum output power reduction for V2X 6.2E.2.1 General 6.2E.2.2 MPR for Power class 2 and Power class 3 V2X UE 6.2E.2.3 MPR for Power class 2 and Power class 3 V2X con-current operation 6.2E.2.4 MPR for Power class 1 UE in Band n14 6.2E.2.5 MPR for Power class 3 Sidelink CA operation < Way forward>: The system parameter and the requirements for SL CA are to be captured in new sub-clause(s) under Suffix E. # Topic 2: Requirements for SL intra-band contiguous CA ## Sub-topic 2-1: System parameter #### Issue 2-1-1: The SCS for SL CA. < GTW Agreement>: - Capture the restriction on the same SCS among SL CA carriers in section TS 38.101-1 section 4.2 - All the requirements for intra-band contiguous CA apply under the assumption of the same subcarrier spacing for SL SA. Issue 2-1-2: Channel BW and Channel raster for SL CA < GTW Agreement>: - Intra-band CA configurations and bandwidth combination sets defined for sidelink as follows Sidelink CA configuration / Bandwidth combination set. Sidelink CA configuration; Sidelink CA configuration for TX; Component carriers in order of increasing carrier frequency; Maximum aggregated bandwidth [MHz]; Bandwidth combination set. Channel bandwidths for carrier [MHz]; Channel bandwidths for carrier [MHz]; Channel bandwidths for carrier [MHz]; Channel bandwidths for carrier [MHz]; Channel bandwidths for carrier [MHz]. SL_n47B; SL_n47B; [10]; [20,30]; 70; 0. [20]; [20,30]. 30; [30,]40. The specific channel bandwidths for SL_n47B should be considered according to operators’ demand. < Way forward>: Channel spacing for SL CA RAN4 consider the channel spacing for SL CA - The channel spacing defined for intra-band contiguous carrier aggregation in 5.4A.1 is the start point. ## Sub-topic 2-2: Tx requirements for SL CA < GTW Agreement>: UE maximum output power for SL CA - Only support PC3 in Rel-18 < Way Forward>: FFS in the next meeting on methodology of Tx requirements for SL intra-band contiguous CA Candidate options: - Option 1: The requirements for NR intra-band contiguous CA as baseline. - Option 2: The requirements for LTE intra-band contiguous CA as baseline. - Option 3: Study the Tx requirements case by case < Way Forward>: Transmit ON/OFF time mask for SL con-current operation with CA - RAN4 can consider the switching between SL CA and LTE-V in Rel-18 - FFS: The optimization for symbol-level switching time similar to that of Tx switching for SL CA and uplink transmission. < Way Forward>: PSFCH for SL CA - Non-contiguous RB allocation of PSFCH for intra-band contiguous SL CA and the restriction on the number of PSFCH is for further study. ## Sub-topic 2-3: Rx requirements for SL CA <Way Forward>: Reference sensitivity - FFS in the next meeting: For SL_n47B, the reference sensitivity defined for single carrier operation in n47 as shown in Table 7.3E.2-1 shall apply for each component carrier with two CCs are activated at the same time

# Multiple PSFCH

Our understanding is RAN1 is still discussing multiple PSFCH per carrier. For RAN4 we should begin our work with a single PSFCH in each carrier. RAN4 companies are also free to bring in studies with multiple PSFCH per carrier.

Proposal: For RAN4 we should begin our work with a single PSFCH in each carrier. RAN4 companies are also free to bring in studies with multiple PSFCH per carrier.

# CIM3 assumption

CIM3 of -60 is achievable in UE design, and also has been used over the years as a RAN4 assumption. We should stick to this. -45 dBc is too loose.

Proposal: RAN4 to use -60 dBc assumption for n47 SL CA analysis.

# Power class

RAN4 should focus the work on PC3 in order to complete in rel18 and exclude PC2.

Proposal: RAN4 should focus the work on PC3 in order to complete in rel18 and exclude PC2.

# Conclusions

Proposal: For RAN4 we should begin our work with a single PSFCH in each carrier. RAN4 companies are also free to bring in studies with multiple PSFCH per carrier.

Proposal: RAN4 to use -60 dBc assumption for n47 SL CA analysis.

Proposal: RAN4 should focus the work on PC3 in order to complete in rel18 and exclude PC2.

# References

- R4-2306637, WF on SL CA

- R4-2310314, WF on NR_SL_enh2_UERF_part3