3GPP TSG-RAN WG4 Meeting #108									                          R4- 2312905

Toulouse, France, Aug. 21 – 25, 2023

Agenda item:			8.13.2.2

Source:	Ericsson

Title:	Discussion on ATG UE RF Tx requirements

Document for:	Discussion

# Introduction

This paper discusses the remaining open issues of ATG UE RF Tx requirements.

# Discussion

In the last meeting, several agreements were reached for ATG UE RF Tx requirements. However, there are still some open issues list below. This paper delves deeper into these issues and proposals are provided.

## Minimum output power for ATG

WF: FFS the assumption of minimum distance between ATG BS and ATG UE to derive the minimum output power.

To estimate the minimum output power for ATG UE, one can consider the scenario where there is the minimum pathloss towards the BS and the maximum antenna gain.

In this case, the minimum altitude considered for ATG is 3 km, which sets the minimum possible distance between the aircraft to an ATG BS at 3 km. In this case, the BS antenna would be pointing straight up towards the UE and the UE antenna would be pointing straight down. Although this scenario may not be very likely, it does represent the theoretical minimum pathloss that could occur. Also, we go beyond the assumptions for the co-existence simulations that some margin is embedded into the minimum power. This is the lowest ever needed in any conceivable scenario. Assuming a minimum SNR target of 0 dB, the estimated minimum UE power in this scenario is overly pessimistic, since a higher SNR target is likely to be used in reality, and the UE power could also be higher.

With the above assumption, an omnidirectional UE at 2 GHz experiences a coupling loss of 84 dB.

Coupling loss at 2 GHz = 108 dB (pathloss) – 24 dB (BS antenna gain).

With a 20 MHz bandwidth, the noise floor of the BS is -96 dBm, leading to a required UE power of -12 dBm.

In the case of 4GHz UE with a 20 dB gain, the coupling loss can be calculated as 73 dB, which is derived as below:

Coupling loss at 4 GHz = 114 dB (pathloss) – 24 dB (BS antenna gain) – 17 dB (UE antenna gain).

With a 100 MHz bandwidth, the BS noise floor is -89 dBm which requires a UE output power of -16 dBm. Based on these calculations, a minimum output power of -20 dBm should be sufficient. However, it is worth noting that this analysis relies on an unlikely scenario in which an aircraft is directly above a BS at a minimum altitude. If a more realistic scenario would be considered, then an even higher UE minimum output power could be considered.

The requirement for minimum output power may not be significantly impacted by the scaling of bandwidth, as the above estimations showed that it has already much more relaxed compared to the current requirement specified for the UE in TS 38.101-1[18]. Therefore, our proposal for the minimum output power is:

- The minimum output power for ATG UE is -25 dBm for 2 GHz and -20 dBm for 4 GHz.

## Configured transmitted power

It is still important for the network to be able to configure a Pmax in case there are regulatory limits. It is unsure of the applicable regulation at present, it may be beneficial to have the functionality/capability to limit the power to below the UEs indicated maximum if required.

- It is proposed to define the Pcmax following the definition in TS 38101-1

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The minimum output power for ATG UE is -25 dBm for 2 GHz and -20 dBm for 4 GHz.

Proposal 2	It is proposed to define the Pcmax following the definition in TS 38101-1

# References

[1] R4-2310400 WF on UE RF requirements for ATG