3GPP TSG-RAN WG4 Meeting #108	R4-2312915

Toulouse, France, August 21 – August 25, 2023

Title:	Discussion of test procedure

Source:	OPPO

Agenda item:	8.2.2

Document for:	Approval

- Introduction

At least two versions of the test procedure for multi-RX DL 2 AoA spherical coverage measurement were discussed during the past several meetings. The Figure 2-1 of the contribution R4-2309435 [1] was agreed as the starting point for further discussion on the test procedure [2].

This contribution further discusses the test procedure for multi-RX DL 2 AoA spherical coverage measurement and updates the test procedure diagram accordingly.

- Discussion

In the session of core requirement discussion in the last RAN4 #107 meeting, the requirement metric for 2 AoA reception was agreed as below [3].

Based on the agreed requirement metric, the test procedure diagram in the contribution R4-2309435 needs the following modifications.

- The requirement metric is calculated based on a specific angular separation. Therefore, the first loop in the procedure is AoA separations, and then the second loop is measurement grid points.

- The UE orientation may be determined by UE declaration, rather than sweeping all the three possible UE orientations. Therefore, the UE orientation is not included in the loop of test procedure.

- The connecting sequence of AoA1 and AoA2 is not needed. Practically, the throughput almost does not change between “connect AoA1 then AoA2” and “connect AoA2 then AoA1”.

Besides, according to the proposal in another contribution that “Only test AoA+ pair at each test point on the sphere”, AoA- pair is a redundant measurement step for multi-RX DL 2 AoA simultaneous reception measurement. Therefore, the test procedure is updated without the test step of AoA- pair measurement.

Further updates of the above diagram may be needed based on the core requirement outcomes, e.g. how many polarization combinations required, how many AoA separations required, how to apply area weights, etc.

Proposal 1: Update the test procedure diagram for multi-RX DL 2 AoA spherical coverage measurement with the above flow-chart figure. And further updates may be needed based on the core requirement outcomes.

- Conclusions

This contribution further discusses the test procedure for multi-RX DL 2 AoA spherical coverage measurement and updates the test procedure diagram accordingly. And provide the following proposal.

Proposal 1: Update the test procedure diagram for multi-RX DL 2 AoA spherical coverage measurement with the above flow-chart figure. And further updates may be needed based on the core requirement outcomes.

- References

[1] R4-2309435, Test procedure for Multi-Rx spherical coverage, Rohde & Schwarz

[2] R4-2309811, WF for FR2 OTA test SI, Qualcomm Incorporated

[3] R4-2310491, WF on UE RF requirements for FR2_multiRx_UERF, Apple