3GPP TSG-RAN WG4 Meeting #108	R4-2312916

Toulouse, France, August 21 – August 25, 2023

Title:	For measurement grid analysis

Source:	OPPO

Agenda item:	8.2.5

Document for:	Approval

- Introduction

Measurement grids would impact on the measurement time and measurement uncertainty. In last RAN4 meeting #107, the methodology of analyzing multi-RX DL 2 AoA spherical coverage was discussed, and the following agreements were achieved [1].

The differences of the simulation results between fine grids and coarse grids help to perform MU analysis and further determine proper measurement grid for multi-RX DL 2 AoA spherical coverage.

This contribution presents our simulations and analysis on measurement grid with different step size for multi-RX DL 2 AoA spherical coverage.

- Discussion

The simulations of multi-RX DL 2 AoA spherical coverage are performed with different step sizes. The simulation assumptions are the same with those simulations performed in the core requirement definition session. Including, the antenna module implementations are Top-Left implementation and Left-Right implementation, as below.

The following parts of the UE are modelled for beam pattern simulation.

| Components | Materials |
| --- | --- |
| Front/Back cover | Glass |
| Side cover | Lossy Metal 1 |
| LCD | Lossy Metal 1 |
| Battery | Lossy Metal 1 |
| PCB | Lossy Metal 2 |

Note: Lossy Metal 1 and Lossy Metal 2 are different metal materials.

The typical antenna patterns with 0-degree phase shift on Left side / Right side / Top side based on above simulation modelling are illustrated as below.

Based on the above simulation assumptions and antenna patterns, four types of step sizes are simulated, i.e. 2-degree step size, 5-degree step size, 15-degree step size and 30-degree step size. The performance metrics of spherical coverage pass rate are compared with those of 2-degree step size. Various of AoA separations are also simulated for analysis.

From the above simulation results, the following observations can be made.

Observation 1: For fine measurement grid, the performance metrics derived under 5-degree step size have little difference with those results under 2-degree step size, i.e. less than 1%.

Observation 2: 30-degree step size causes obvious deviation, and the biggest deviation occurs at 180 AoA offset for Top-Left antenna module implementation, i.e. 6.1%.

Observation 3: 15-degree step size produces deviations from 0% to 2.4%.

Based on the above observations, the measurement step size for multi-RX DL 2 AoA spherical coverage are proposed.

Proposal 1: Take 15-degree step size for the measurement of multi-RX DL 2 AoA spherical coverage as the starting point.

Proposal 2: Whether 30-degree step size can be adopted for multi-RX DL 2 AoA spherical coverage measurement depends on further MU analysis and total permitted MU.

- Conclusions

This contribution presents our simulations and analysis on measurement grid with different step size for multi-RX DL 2 AoA spherical coverage.. And provide the following proposal.

Observation 1: For fine measurement grid, the performance metrics derived under 5-degree step size have little difference with those results under 2-degree step size, i.e. less than 1%.

Observation 2: 30-degree step size causes obvious deviation, and the biggest deviation occurs at 180 AoA offset for Top-Left antenna module implementation, i.e. 6.1%.

Observation 3: 15-degree step size produces deviations from 0% to 2.4%.

Proposal 1: Take 15-degree step size for the measurement of multi-RX DL 2 AoA spherical coverage as the starting point.

Proposal 2: Whether 30-degree step size can be adopted for multi-RX DL 2 AoA spherical coverage measurement depends on further MU analysis and total permitted MU.

- References

[1] R4-2309811, WF for FR2 OTA test SI, Qualcomm Incorporated