3GPP TSG-RAN WG4 Meeting #108	R4-2312918

Toulouse, France, August 21 – August 25, 2023

Title:	On requirement metric for 2 AoA reception

Source:	OPPO

Agenda item:	8.7.2.2

Document for:	Approval

- Introduction

The requirement metric for 2 AoA reception was discussed during the past several RAN4 meetings. In the last meeting RAN4#107, the agreement [1] was reached on the requirement metric as below.

This contribution presents our analysis and proposals on the requirement metric for 2 AoA reception.

- Discussion

In the RAN4#106 meeting, companies bring up that the position of AoA2 with respect to AoA1 may introduce testing bias. And it is agreed that both +AoA offset and -AoA offset for each test point shall be considered as the starting point. In the RAN4#106bis-e meeting, the approach that the result at each test point is constructed based on two AoA pairs containing that test point, named as AoA+ pair and AoA- pair is taken as baseline. AoA+ pair represents the two AoAs of AoA1 and AoA1+AoA separation, while AoA- pair represents the two AoAs of AoA1 and AoA1-AoA separation. And in the last RAN4 meeting #107, AoA+ pair and AoA- pair are used in the 2-AoA reception requirement metric definition.

On the other hand, based on the optimized perceived DL direction coverage, the following agreements are reached in the RAN4#106 meeting [2].

The above agreements can draw a conclusion that AoA1 and AoA2 are always on the same meridian according to the DUT’s coordination system, or on the meridian with 180-degree separation (the additional probe rotates over the south pole).

There is another agreement in the RAN4 #106 meeting that AoA offset/separation value should be an integer multiple of the step size of the constant size measurement test grid.

With the understanding of the above agreements, the AoA+ pair and AoA- pair are simulated on any meridian (together with the meridian with 180-degree separation) with the condition of AoA offset = 30 degree, as below. It is found that every AoA pair is actually measurement twice, i.e. one with the label of AoA+ and the other with the label of AoA-.

In general, under any AoA offset/separation, around the entire sphere, every AoA pair will be measured twice under AoA+/AoA- approach. The deduced process is detailed as below.

Set AoA offset = Δ

For θ=θ1,

AoA1θ1=θ1,φ1,

AoA2+θ1=θ1+∆,φ1

AoA2-θ1=θ1-∆,φ1

So,

AoA+ pair θ1=θ1,φ1,θ1+∆,φ1

AoA- pair θ1=θ1,φ1,θ1-∆,φ1

Considering that AoA offset/separation value is an integer multiple of the step size of the constant size measurement test grid, there will be a θ2=θ1+∆.

For θ=θ2

AoA2θ2=θ2,φ1=θ1+∆,φ1,

AoA2+θ2=θ2+∆,φ1=θ1+2∆,φ1

AoA2-θ2=θ2-∆,φ1=θ1,φ1

So,

AoA+ pair θ2=θ1+∆,φ1,θ1+2∆,φ1

AoA- pair θ2=θ1+∆,φ1,θ1,φ1

Clearly that,

AoA+ pair θ1=AoA- pair θ2                  with θ2=θ1+∆

Thus, any AoA- pair equals to one AoA+ pair.

Observation 1: Under any AoA offset/separation, around the entire sphere, every AoA pair will be measured twice under AoA+/AoA- approach.

With the above observation, and further considering the challenging test time consumption, it is proposed that only test AoA+ pair at each test point on the sphere. The benefits of the proposal are not only simplifying the requirement metric, but also shortening the test process and test time. Besides, a controversial issue is solved effectively that there is no need to further discuss the “combine method” of AoA+ and AoA-.

Proposal 1: Only test AoA+ pair at each test point on the sphere.

Proposal 2: The requirement metric of 2-AoA reception will be updated consequently.

“For a specific angular separation between 2 TRPs and a specific UE orientation under standardized DL power level which is equal between 2 TRPs, the result at each test point is based on AoA+ pair. Overall result (probability to support 2TRP DL) is by averaging regional results.”

- Conclusions

This paper presents our analysis and proposals on the requirement metric for 2-AoA reception. And provide the following proposal.

Proposal: Only test AoA+ pair at each test point on the sphere.

Proposal 2: The requirement metric of 2-AoA reception will be updated consequently.

“For a specific angular separation between 2 TRPs and a specific UE orientation under standardized DL power level which is equal between 2 TRPs, the result at each test point is based on AoA+ pair. Overall result (probability to support 2TRP DL) is by averaging regional results.”

- References

[1] R4-2310491, WF on UE RF requirements for FR2_multiRx_UERF, Apple

[2] R4-2303708, WF on NR FR2 multi-Rx chain DL reception, Samsung, Apple