3GPP TSG-RAN WG4 Meeting #108	R4-2312937

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.872 | CR | 0003 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TR 38.872 to introduce some potential RF impacts for for CA_n5-n28-n105 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_sub1GHz_combo_enh |  |  |  |  |  | Date: |  |  | 2023-07-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce some potential RF impacts for for CA_n5-n28-n105 |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce some potential RF impacts for for CA_n5-n28-n105. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The outcomes for CA_n5-n28-n105 for this SI can’t be captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TR 38.872>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-221464, “Revised SI: Study on enhancement for 700/800/900MHz band combinations for NR”, RAN#96

[3]	TS 38.101-1

[4]	TS 38.101-3

[5]	RP-230789, “ Study on enhancement for sub-1Ghz NR band combinations”. RAN 99

[6]	3GPP TR 38 892 , APT 600 MHz NR band.

[7]	R4-2308581, Discussion on RF architecture and potential MSD issues for CA_n5-n105, Huawei, HiSilicon, Spark NZ

[8]	R4-2309299, Input on CA_n5-n105 cross-band MSD and DeltaT/R, Skyworks Solutions Inc.

[9]	R4-2309356, Considerations on CA_n5-n105, Qualcomm France

[10]	R4-2309358, Considerations on CA_n26-n28, Qualcomm France

[11]	R4-2306468, WF on CA_n5-n28 and CA_n8-n20-n28, Huawei, HiSilicon

[12]	R4-2308583, Discussion on RF architecture and potential MSD issues for CA_n26-n28, Huawei, HiSilicon

[13]	R4-2307477, Input on CA_n26-n28 requirements, Skyworks Solutions Inc.

[14] 	R4-2306470, WF on RF architecture and critical issues for CA_n28-n105

[15] 	R4-230xxxx, Discussion on RF architecture and potential MSD issues for CA_n5-n28-n105

<<Next of Change>>

5.7	CA_n5-n28-n105

5.7.1	UE RF architecture assumption

5.7.1.1	General

The UE RF architectures with 2 antenna, 3 antenna, 4 antenna can be assumed to analyze and study for CA_n5-n28-n105.The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands. UE RF architecture with three antennas for CA_n5-n28-n105 is used as baseline for evaluation. The UE RF architectures with two or four antennas are not precluded in the study item. In addition, these RF architectures doesn’t preclude the single UL transmission on either band n28 or n105 and Tx switching between band n28 and n105 is also not precluded.

5.7.1.2	UE RF architecture with two antennas

UE RF architecture with two antennas for CA_n5-n28-n105 is shown below.

Figure 5.7.1.2-1 two-antenna RF architecture for CA_n5-n28-n105

The antenna relative bandwidth ratio for two-antenna RF architecture is shown below.

Table 5.7.1.2-1: the antenna relative bandwidth ratio for two-antenna RF architecture

| Antenna | The covered frequency range | Relative bandwidth ratio |
| --- | --- | --- |
| Main TRx antenna | 612~894MHz | 37% |
| Diverstiy TRx antenna | 612~894MHz | 37% |

For two-antenna UE RF architecture, although only two low band antennas are kept due to the compact size of smartphone, UE has to face the difficulties of wide bandwidth low-band antenna and complex multiplexer.

5.7.1.3	UE RF architecture with three antennas

For three-antenna RF architecture, three types (a) (b) and (c) are provided below for CA_n5-n28-n105.

Figure 5.7.1.3-1 three-antenna RF architecture (c) for CA_n5-n28-n105

For a three-antenna RF architecture in Figure 5.7.1.3-1, the UE has also to implement a quadplexer (n5UL/n5DL/n105DL/n105UL). In addition, wide bandwidth antenna is another challenge for this type of UE RF architecture to cover frequency range from 612MHz to 894MHz. The antenna relative bandwidth ratio for three-antenna RF architecture (a) is shown Table 5.7.1.3-1.

Table 5.7.1.3-1 the antenna relative bandwidth ratio for three-antenna RF architecture (a)

| Antenna | The covered frequency range | Relative bandwidth ratio |
| --- | --- | --- |
| Main TRx antenna 1 | 612~894MHz | 37% |
| Main TRx antenna 2 | 703~803MHz | 13% |
| Diverstiy TRx antenna | 612~894MHz | 37% |

Figure 5.7.1.3-2 three-antenna RF architecture (b) for CA_n5-n28-n105

For the three-antenna RF architecture Figure 5.7.1.3-2, the challenge is to implement a quadplexer (n5UL/n5DL/n105DL/n28DL), but wide bandwidth antenna in main TRx 2 can be tuned at the UL frequency range of band n5, which may help ease the design of low band antenna. The antenna relative bandwidth ratio for three-antenna RF architecture (b) is shown in 5.7.1.3-2.

Table 5.7.1.3-2 the antenna relative bandwidth ratio for three-antenna RF architecture (b)

| Antenna | The covered frequency range | Relative bandwidth ratio |
| --- | --- | --- |
| Main TRx antenna 1 | 703~803MHz (band n28 is active) 612~703MHz (band n105 is active) | 13% 13.8% |
| Main TRx antenna 2 | 612~894MHz | 37% |
| Diverstiy TRx antenna | 612~894MHz | 37% |

Figure 5.7.1.3-3 three-antenna RF architecture (c) for CA_n5-n28-n105

For three-antenna RF architecture Fig 5.7.1.3-3, only single UL is supported for CA_n5-n28-n105. Two Rx filter groups are considered and the antenna efficiency can be compromised for pure Rx wide bandwidth antenna. This kind of implementation can be achieved more easily. The antenna relative bandwidth ratio for three-antenna RF architecture (c) is shown in table 5.7.1.3-3.

Table 5.7.1.3-3 the antenna relative bandwidth ratio for three-antenna RF architecture (c)

| Antenna | The covered frequency range | Relative bandwidth ratio |
| --- | --- | --- |
| Main TRx antenna 1 | 824~894MHz (band n5 is active) 703~803MHz (band n28 is active) 612~703MHz (band n105 is active) | 8% 13% 13.8% |
| Main TRx antenna 2 | 612~894MHz | 37% |
| Diverstiy TRx antenna | 612~894MHz | 37% |

For three-antenna UE RF architecture, one more low-band antenna can help ease the design of wide bandwidth antenna and avoid to implement the most complicated multiplexer comparing to two-antenna UE RF architecture.

5.7.1.4	UE RF architecture with four antennas

UE RF architecture with four antennas for CA_n5-n28-n105 is shown below.

Figure 5.7.1.4-1 four-antenna RF architecture for CA_n5-n28-n105

The antenna relative bandwidth ratio for four-antenna RF architecture is shown below.

Table 5.7.1.4-1: the antenna relative bandwidth ratio for two-antenna RF architecture

| Antenna | The covered frequency range | Relative bandwidth ratio |
| --- | --- | --- |
| Main TRx antenna 1 | 612~703MHz (band n105 is active) | 13.8% |
| Main TRx antenna 2 | 824~894MHz (band n5 is active) | 8% |
| Main TRx antenna 3 | 703~803MHz (band n28 is active) | 13% |
| Diverstiy TRx antenna | 612~894MHz | 37% |

For four-antenna UE RF architecture, even if UE can avoid implementing wide bandwidth low-band antenna and complex multiplexer, UE pays the price of space and antenna performance.

5.7.2	Operating bands for CA

Table 5.7.2-1: CA band combination of band n5+n28+n105

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n105 | 612 MHz | – | 652 MHz | 663 MHz | – | 703 MHz | FDD |

5.7.3	Channel bandwidths per operating band for CA

Table 5.7.3-1: Supported bandwidths per CA band combination of band n8+n20+n28

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n28A-n105A | CA_n5A-n105A CA_n5A-n28A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

5.7.4	∆TIB,c and ∆RIB,c values

For CA_n5-n28-n105, the ∆TIB,c and ∆RIB,c values are derived from the existing CA_n5-n28 and CA_n28-n105. The following ∆TIB,c and ∆RIB,c values for CA_n5-n28-n105 can be further considered.

Table 5.7.4-1:ΔTIB,c and ΔRIB,c for CA_n5-n28-n105

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] | ΔRIB,c [dB] |
| --- | --- | --- | --- |
| CA_n5A-n28A-n105A | n5 | 0.7 | 0.2 |
|  | n28 | Max(0.7, the value from CA_n28-n105) | Max(0.2, the value from CA_n28-n105) |
|  | n105 | the value from CA_n28-n105 | the value from CA_n28-n105 |

5.7.5	UE co-existence studies for UL inter-band CA

Table 5.7.5-1 lists Band n5 + Band n105 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n28.

Table 5.7.5-1: UL CA_n5-n105 IMD product

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 703 | 824 | 849 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 121 | 186 | 1487 | 1552 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 477 | 582 | 945 | 1035 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2150 | 2255 | 2311 | 2401 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1140 | 1285 | 1769 | 1884 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2813 | 2958 | 3135 | 3250 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 372 | 242 | 2974 | 3104 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2733 | 2593 | 1988 | 1803 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1221 | 1066 | 461 | 291 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3959 | 4099 | 3476 | 3661 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3798 | 3953 | 3637 | 3807 |

Table 5.7.5-2 lists Band n5 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n105.

Table 5.7.5-2: UL CA_n5-n28 IMD product

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 824 | 849 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 76 | 146 | 1527 | 1597 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 557 | 672 | 900 | 995 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2230 | 2345 | 2351 | 2446 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1260 | 1420 | 1724 | 1844 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2933 | 3093 | 3175 | 3295 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 292 | 152 | 3054 | 3194 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2693 | 2548 | 2168 | 1963 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1141 | 976 | 596 | 411 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3999 | 4144 | 3636 | 3841 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3878 | 4043 | 3757 | 3942 |

There is no IMD interference which fall into Rx frequencies of third DL band n28 when CA_n5-n105 is the UL configuration.

3rd order IMD may also fall into Rx frequencies of third DL band n105 when CA_n5-n28 is the UL configuration.

5.7.6	REFSENS evaluation

The MSD test configurations were evaluated as below for band n105 MSD due to IMD3 of Tx band n5 + band n28 referring to contribution [15].

Table 5.7.6-1: REFSENS exceptions due to IMD interference for CA_n5-n28-n105

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n28-n105 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n105 | 686 | 5 | 25 | 635 | 22.9 | FDD | IMD3 |

Generally, there are some network deployment methods to avoid the IMD product hitting the DL carrier directly. For example, if the UL carrier in band n28 is configured in 703~718MHz, no matter which UL frequency carrier in band n5 is configured, the IMD3 products will not hit the DL carrier in band n105 directly.

<<End of Change>>