3GPP TSG-RAN WG4 Meeting # 108                               R4-2312961

Toulouse, France, August 21 – August 25, 2023

Source: 	Huawei, HiSilicon

Title: 	Simulation result for ATG Synchronized scenarios based on latest template

Agenda Item:	8.13.1.4

Document for:	Approval

Introduction

In the RAN4#106bis meeting, we provided our simulation results in contribution [1]. For ATG synchronized scenarios, 45dB ACLR / 46dB ACS for ATG BS can be reused to guarantee coexistence between ATG system and TN system.. And 30dB ACLR / 33dB ACS can be reused for ATG UE to guarantee coexistence between ATG system and TN system.

In addition, we had the way forward [2] on how to capture the simulation results for ATG co-existence evaluation.

Agreement: - Companies are encouraged to provide simulation results on top of following template to captured into TR - One table per scenario - Both table and plot are suggested to collect simulation results. - following show the template plot - X axis is ATG BS/UE ACLR/ACS (as appropriate) to make it easier to directly read off from the plots that meets the 5% throughput loss requirement. - Y axis is the throughput loss the ATG BS/UE ACLR/ACS value - Following table 1 show the template table Table 1: template table to collect simulation results Company; ATG/ TN BS antenna model; Performance Metric; Throughput Loss (%) assuming the same ACLR and ACS as TN. Maximum distance between ATG BS and ATG UE. 100 km; 300 km. [Company 1]; Non-Subarray; 5% in the whole network. Average of all users in the whole network. 5% of users within the cell with largest throughput loss for the case of TN UL victim note 1. Average of all users within the cell with largest throughput loss for the case of TN UL victim note 1. Subarray; 5% in the whole network. Average of all users in the whole network. 5% of users within the cell with largest throughput loss for the case of TN UL victim note 1. Average of all users within the cell with largest throughput loss for the case of TN UL victim note 1. Note 1: only applicable for scenario 2 and 10.

In this paper, we’d like to share our simulation results based on the latest template.

Discussion

Referring to TR 38.876 [3], we have the following synchronized scenarios.

Table 1 synchronized scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 3.5 GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |

Simulation results

All the simulation assumed that maximum distance between ATG BS and ATG UE is 100km.

Scenario 1 (4GHz)

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG BS ACLR 45 dB | Throughput Loss (%) at ATG BS ACLR 44 dB | Throughput Loss (%) at ATG BS ACLR 43 dB | Throughput Loss (%) at ATG BS ACLR 42 dB | Throughput Loss (%) at ATG BS ACLR 41 dB |
| --- | --- | --- | --- | --- | --- | --- |
| Non-Subarray | 5% in the whole network | 0 | 0.05532 | 0.30621 | 0.49989 | 0.51064 |
|  | Average of all users in the whole network | 0 | 0.01759 | 0.03568 | 0.05672 | 0.05946 |

Scenario 2 (4GHz)

| c | Performance Metric | Throughput Loss (%) at ATG UE ACLR 30 dB | Throughput Loss (%) at ATG UE ACLR 29 dB | Throughput Loss (%) at ATG UE ACLR 28 dB | Throughput Loss (%) at ATG UE ACLR 27 dB | Throughput Loss (%) at ATG UE ACLR 26 dB |
| --- | --- | --- | --- | --- | --- | --- |
| Non-Subarray | 5% in the whole network | 0 | 0 | 0 | 0 | 0 |
|  | Average of all users in the whole network | 0 | 0 | 0 | 0 | 0 |

Scenario 3 (4GHz)

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG UE ACS 33 dB | Throughput Loss (%) at ATG UE ACS 32 dB | Throughput Loss (%) at ATG UE ACS 31 dB | Throughput Loss (%) at ATG UE ACS 30 dB | Throughput Loss (%) at ATG UE ACS 29 dB |
| --- | --- | --- | --- | --- | --- | --- |
| Non-Subarray | 5% in the whole network | 0.1777 | 0.30314 | 0.48804 | 0.51178 | 0.95954 |
|  | Average of all users in the whole network | 0.2344 | 0.24167 | 0.34044 | 0.38139 | 0.58237 |

Scenario 4 (4GHz)

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG BS ACS 46 dB | Throughput Loss (%) at ATG BS ACS 45 dB | Throughput Loss (%) at ATG BS ACS 44 dB | Throughput Loss (%) at ATG BS ACS 43 dB | Throughput Loss (%) at ATG BS ACS 42 dB |
| --- | --- | --- | --- | --- | --- | --- |
| Non-Subarray | 5% in the whole network | 0.22464 | 0.23587 | 0.24527 | 0.27195 | 0.29602 |
|  | Average of all users in the whole network | 0.001773 | 0.038085 | 0.038359 | 0.042005 | 0.045876 |

Scenario 9 (2GHz)

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG BS ACLR 45 dB | Throughput Loss (%) at ATG BS ACLR 44 dB | Throughput Loss (%) at ATG BS ACLR 43 dB | Throughput Loss (%) at ATG BS ACLR 42 dB | Throughput Loss (%) at ATG BS ACLR 41 dB |
| --- | --- | --- | --- | --- | --- | --- |
| Non-Subarray | 5% in the whole network | 0.21915 | 0.22515 | 0.24044 | 0.26619 | 0.28541 |
|  | Average of all users in the whole network | 0.011378 | 0.011813 | 0.011871 | 0.011901 | 0.012177 |

Scenario 10 (2GHz)

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG UE ACLR 30 dB | Throughput Loss (%) at ATG UE ACLR 29 dB | Throughput Loss (%) at ATG UE ACLR 28 dB | Throughput Loss (%) at ATG UE ACLR 27 dB | Throughput Loss (%) at ATG UE ACLR 26 dB |
| --- | --- | --- | --- | --- | --- | --- |
| Non-Subarray | 5% in the whole network | 0 | 0 | 0 | 0 | 0 |
|  | Average of all users in the whole network | 0.024655 | 0.024748 | 0.024962 | 0.025592 | 0.025972 |

Scenario 11 (2GHz)

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG UE ACS 33 dB | Throughput Loss (%) at ATG UE ACS 32 dB | Throughput Loss (%) at ATG UE ACS 31 dB | Throughput Loss (%) at ATG UE ACS 30 dB | Throughput Loss (%) at ATG UE ACS 29 dB |
| --- | --- | --- | --- | --- | --- | --- |
| Non-Subarray | 5% in the whole network | 1.3613 | 1.5249 | 1.9196 | 2.0489 | 3.6495 |
|  | Average of all users in the whole network | 1.0258 | 1.2708 | 1.4767 | 1.4855 | 2.3524 |

Scenario 12 (2GHz)

| ATG/ TN BS antenna model | Performance Metric | Throughput Loss (%) at ATG BS ACS 46 dB | Throughput Loss (%) at ATG BS ACS 45 dB | Throughput Loss (%) at ATG BS ACS 44 dB | Throughput Loss (%) at ATG BS ACS 43 dB | Throughput Loss (%) at ATG BS ACS 42 dB |
| --- | --- | --- | --- | --- | --- | --- |
| Non-Subarray | 5% in the whole network | 1.7028 | 1.75304 | 1.8123 | 1.8942 | 1.9173 |
|  | Average of all users in the whole network | 0.0012431 | 0.00174585 | 0.00199447 | 0.0020431 | 0.00229447 |

Based on the simulation results of synchronized scenarios, the ATG ACLR and ACS requirements are proposed in the following table.

Table 2 Agreed ATG ACLR and ACS values based on synchronized scenarios

| ATG |  | Values |
| --- | --- | --- |
| BS | ACLR | 45 |
|  | ACS | 46 |
| UE | ACLR | 30 |
|  | ACS | 33 |

Summary

Proposal 1: the ATG ACLR and ACS requirements are proposed in the following table.

Table 2 Agreed ATG ACLR and ACS values based on synchronized scenarios

| ATG |  | Values |
| --- | --- | --- |
| BS | ACLR | 45 |
|  | ACS | 46 |
| UE | ACLR | 30 |
|  | ACS | 33 |

- References

[1] R4-2305372, Discussion on 2GHz and 4GHz simulation result for ATG scenario, Huawei, HiSilicon

[2] R4-2310401, WF on ATG co-existence evaluation, CMCC

[3] TR 38.876, Study on Air-to-ground network for NR, v0.4.0