3GPP TSG-RAN WG4 Meeting # 108                               R4-2312962

Toulouse, France, August 21 – August 25, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on ATG UE Tx requirements

Agenda Item:	8.13.2.2

Document for:	Approval

Introduction

In the RAN4#106bis and RAN4#107 meetings, two way forward [1] [2] were approved with some progress, but the details of configured transmitted power and minimum output power need more discussion. In this paper, we’d like to provide our views on two RF requirements.

Discussion on Configured transmitted power

In the RAN4#107 meeting, RAN4 has already concluded the configured output power for ATG UE need to be introduced.

<Agreement>: Introduce the configured output power for ATG UE.

In addition, the reason why to specify such requirements was recorded into the topic summary [3].

• For co-channel interference management reason, network should be able to configure the UE transmit power. Option 1 is preferred. • It’s proposed to define the following Pcmax,f,c. Otherwise, network signalling Pemax can’t restrain the output power of ATG UE for compliance with regulation. • It is still important for the network to be able to configure a Pmax in case there are regulatory limits.

Since RAN4 has agreed not to specify MPR/AMPR requirements and ATG UE need to indicate the maximum output power at maximum modulation order and full PRB configurations, the Pcmax for ATG UE can be expressed as below. Otherwise, if we don’t specify the configured output power of ATG UE, network signalling Pemax can’t restrain the output power of ATG UE for compliance with regulation.

Proposal 1: it’s proposed to define the following Pcmax,f,c. Otherwise, network signalling Pemax can’t restrain the output power of ATG UE for compliance with regulation.

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c, [PMaxOutputPower]}

PCMAX_H,f,c = PEMAX,c

where

PEMAX,c is the value given by [either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE], whichever is applicable according to TS 38.331[7];

[PMaxOutputPower] is the maximum UE output power at maximum modulation order and full PRB configurations which is indicated by ATG UE;

Discussion on Minimum output power

In the RAN4#106bis meeting, we have the following way forward for minimum output power requirements.

Sub-topic 1-3 Minimum output power for ATG Candidate options: - Option 2: Considering some margins, the minimum output power for ATG could be written as below with 25Db relaxation: Channel bandwidth; (MHz); 5,10,15,20; 25,30,35,40,45,50; 60,70,80,90,100. REF_SCS; (kHz); 15; 30. Minimum output power; (dBm); -40+25; -40+25+10log10 (BWChannel /20); -40+25+10log10 (BWChannel /20). Measurement bandwidth; (MHz); MBW=REF_SCS*(12*NRB+1)/1000. NOTE: The minimum output power value is rounded to the nearest number down to one decimal point.. - Option 3: Define UE minimum output power to be -25 dBm for 2 GHz, and -20 dBm for 4 GHz. - Option 5: the minimum output power for ATG UE could be [-3dBm/100MHz] and scale with other BW. <Way forward>: FFS the assumption of minimum distance between ATG BS and ATG UE to derive the minimum output power.

According to the contribution [4, 5], companies provided some technical judgements and analysis about the minimum output power.

Minimum output power for ATG UE + antenna gain for ATG UE + antenna gain for BS – pathloss(2GHz, 3000m) >= RESENS of BS. And the assumptions for each parameter are shown below.

Antenna gain for ATG UE: 10 dB.

Antenna gain for BS: 23dB

Pathloss(2GHz, 3000m) = 32.5 +20log10(2)+20log10(3000)=108dB

REFSENS for BS 5MHz = -101.7 dBm

So Minimum output power for ATG UE is -27dBm.

Since 20km minimum distance between ATG BS and UE is assumed for co-existence study, generally, X/Y can’t be larger than 10dB.

Proposal 2: To specify Minimum output power for ATG UE with following requirements.

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -20 | -20+10log10 (BWChannel /20) | -20+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

If RAN4 can’t reach an agreement on the Minimum output power requirements, in order to make progress, the following assumptions are proposed to further derive the minimum output power requirements.

Proposal 3: If RAN4 can’t reach an agreement on the Minimum output power requirements the following assumptions are proposed to further derive the minimum output power requirements.

- ATG BS antenna gain: Referring to Table 6.2.3.1-1 of TR 38.876, we have the following assumption for ATG BS antenna model. ATG. Base Station Antenna Characteristics. Antenna pattern; TR 38.921. Element gain (dBi) (Note 2); 7.1. Horizontal/vertical 3 dB beam width of single element (degree); 90º for H 54º for V. Horizontal/vertical front-to-back ratio (dB); 30 for both H/V. Antenna polarization; Linear ±45º. Antenna array configuration (Row × Column) (Note 4); 8 × 8 elements. Number of supported polarizations, P; 2. Horizontal/Vertical radiating element spacing; 0.5 of wavelength for H, 0.9 of wavelength for V. Array Ohmic loss (dB) (Note 2); 2. Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3); 25. Base station maximum coverage angle in the horizontal plane (degrees); 120. Base station vertical coverage range (degrees) (Note 1); 25. Mechanical uptilt (degrees); 14. Based on that, we can assume BS antenna gain = 10*log10(8*8) + 7.1 = 25.2dB - ATG BS maximum output power: (For ATG UE maximum input level evaluation) 53dBm sum of two polarizations for 4GHz 46dBm sum of two polarizations for 2GHz - ATG UE antenna gain Referring to Table 6.2.3.2-1 of TR 38.876, we have the following assumption for ATG UE antenna model for 4GHz Horizontal/vertical 3 dB beam width of single element (degree); 90º for H 90º for V. Element gain (dBi); 5 dBi. Horizontal/vertical front-to-back ratio (dB); 30dBc. Antenna polarization; Linear ±90º. Antenna array configuration (Row × Column x Polarization) (Note 4); (8x2x2) or (16x1x2). Horizontal/Vertical radiating element spacing; 0.5 of wavelength for H, 0.5 of wavelength for V. UE antenna orientation; Single UE panel deployed on the abdomen of the airplane facing downwards and with the longest dimension of the array aligned with the direction of the flight route. The flight route is pointed at the BS.. 4GHz ATG UE phased array antenna gain = 10*log10(16) + 5 = 17dB 2GHz ATG UE Omni-direction antenna gain = 0dB - Pathloss model: Free space Pathloss ( fc_GHz, MinimumDistance_m ) = 32.5 +20log10(fc_GHz)+20log10(MinimumDistance_m) - The assumption of minimum distance between ATG BS and ATG UE is 3km ~ 20km. - Minimum output power for ATG UE + antenna gain for ATG UE + antenna gain for BS – pathloss(MinDistance, Fc) >= RESENS of BS

- 3 Summary

Based on the discussion, all the observations and proposals are listed below:

Proposal 1: it’s proposed to define the following Pcmax,f,c. Otherwise, network signalling Pemax can’t restrain the output power of ATG UE for compliance with regulation.

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c, [PMaxOutputPower]}

PCMAX_H,f,c = PEMAX,c

where

PEMAX,c is the value given by [either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE], whichever is applicable according to TS 38.331[7];

[PMaxOutputPower] is the maximum UE output power at maximum modulation order and full PRB configurations which is indicated by ATG UE;

Proposal 2: To specify Minimum output power for ATG UE with following requirements.

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -20 | -20+10log10 (BWChannel /20) | -20+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

Proposal 3: If RAN4 can’t reach an agreement on the Minimum output power requirements the following assumptions are proposed to further derive the minimum output power requirements.

- ATG BS antenna gain: Referring to Table 6.2.3.1-1 of TR 38.876, we have the following assumption for ATG BS antenna model. ATG. Base Station Antenna Characteristics. Antenna pattern; TR 38.921. Element gain (dBi) (Note 2); 7.1. Horizontal/vertical 3 dB beam width of single element (degree); 90º for H 54º for V. Horizontal/vertical front-to-back ratio (dB); 30 for both H/V. Antenna polarization; Linear ±45º. Antenna array configuration (Row × Column) (Note 4); 8 × 8 elements. Number of supported polarizations, P; 2. Horizontal/Vertical radiating element spacing; 0.5 of wavelength for H, 0.9 of wavelength for V. Array Ohmic loss (dB) (Note 2); 2. Conducted power (before Ohmic loss) per antenna element (dBm) (Note 3); 25. Base station maximum coverage angle in the horizontal plane (degrees); 120. Base station vertical coverage range (degrees) (Note 1); 25. Mechanical uptilt (degrees); 14. Based on that, we can assume BS antenna gain = 10*log10(8*8) + 7.1 = 25.2dB - ATG BS maximum output power: (For ATG UE maximum input level evaluation) 53dBm sum of two polarizations for 4GHz 46dBm sum of two polarizations for 2GHz - ATG UE antenna gain Referring to Table 6.2.3.2-1 of TR 38.876, we have the following assumption for ATG UE antenna model for 4GHz Horizontal/vertical 3 dB beam width of single element (degree); 90º for H 90º for V. Element gain (dBi); 5 dBi. Horizontal/vertical front-to-back ratio (dB); 30dBc. Antenna polarization; Linear ±90º. Antenna array configuration (Row × Column x Polarization) (Note 4); (8x2x2) or (16x1x2). Horizontal/Vertical radiating element spacing; 0.5 of wavelength for H, 0.5 of wavelength for V. UE antenna orientation; Single UE panel deployed on the abdomen of the airplane facing downwards and with the longest dimension of the array aligned with the direction of the flight route. The flight route is pointed at the BS.. 4GHz ATG UE phased array antenna gain = 10*log10(16) + 5 = 17dB 2GHz ATG UE Omni-direction antenna gain = 0dB - Pathloss model: Free space Pathloss ( fc_GHz, MinimumDistance_m ) = 32.5 +20log10(fc_GHz)+20log10(MinimumDistance_m) - The assumption of minimum distance between ATG BS and ATG UE is 3km ~ 20km. - Minimum output power for ATG UE + antenna gain for ATG UE + antenna gain for BS – pathloss(MinDistance, Fc) >= RESENS of BS

- References

[1] R4-2306609, WF on ATG UE RF requirements, Huawei, HiSilicon

[2] R4-2310400, WF on UE RF requirements for ATG, Huawei, HiSilicon

[3] R4-2310020, Topic summary for [107][137] NR_ATG_UERF_part2, Moderator (Huawei)

[4] R4-2218694, ATG UE Tx RF requirement, Apple

[5] R4-2219612, Discussion on ATG UE Tx requirements, Huawei, HiSilicon