3GPP TSG-RAN WG4 Meeting # 108                               R4-2312970

Toulouse, France, August 21 – August 25, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on RF requirements for CA_n28-n105

Agenda Item:	7.30.1.5

Document for:	Approval

Introduction

In the RAN4#107 meeting, RAN4 completed the study phase of CA_n28-n105 as two draft CRs [1] [2] were approved. In the WF [3], RAN4 has agreed to only specify one UL configuration for CA_n28-n105. Thus, only MSD due to cross band isolation and delta Tib and Rib should be specified. In this paper, we’d like to further discuss these RF requirements for CA_n28-n105.

Discussion on MSD due to cross band isolation

n28 UL => n105 DL

the 2nd adjacent channel interference from 30MHz band n28 UL may fall into the frequency range 643~652MHz. n28 Tx filter rejection at 612~652MHz is assumed as 30~40 dB. Three-antenna RF architecture is assumed for MSD evaluation.

We have the following evaluation for MSD due to cross band isolation for CA_n28-n105.

Table 1 MSD due to cross band isolation n28 => n105

|  | parameters | n105 main path | n105 diversity path |
| --- | --- | --- | --- |
| transmit power for n28, dBm |  | 23 | 23 |
| Tx band n28 BW(MHz, Lcrb) | 30 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n28 uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n105 receival singal at ANT port, dBm |  | 13 | 13 |
| n105 filter rejection at 703~733MHz, dB | 40 |  |  |
| signal After n105 filter, dBm |  | -31 | -31 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -112 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -108 |
| noise figure dB | 11.8 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -162.2 |  |  |
| Rx band n105 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -95.67 | -95.67 |
| ACLR2, dB | 43 |  |  |
| n28 PA leakage PSD at PA output port at 2nd adjacent channel, dBm/MHz |  | -34.77 |  |
| n28 PA noise PSD at PA output port, dBm/RxBW |  | -28.24 |  |
| n28 Tx filter rejection at 612~652MHz, dB | 40 |  |  |
| n28 PA noise power at Rx n105 ant port at 612~652MHz, dBm |  | -78.24 | -78.24 |
| Total noise level at ANT port |  | -78.16 | -78.16 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -79.16 | -79.16 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(5MHz BW), dBm | -80.17 |  |  |
| MSD | 17.03 |  |  |

Proposal 1: Based on the evaluation, the REFSENS degradation due to cross band isolation from 30MHz n28 UL to n105 DL can be specified below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | 17.0 | >ACLR |

n105 UL => n28 DL

In my contribution [4], we proposed not to specify MSD for this case as 40dB isolation between n105 Tx and n28 Rx band. However, during discussion in the RAN4#107 meeting, companies proposed that the maximum isolation between n105 Tx and n28 Rx band is about 35dB. Thus, we can compromise to consider 35dB isolation between n105 Tx and n28 Rx band. Three-antenna RF architecture is assumed for MSD evaluation.

Table 2 MSD due to cross band isolation n105 => n28

|  | parameters | n28 main path | n28 diversity path |
| --- | --- | --- | --- |
| transmit power for n105, dBm |  | 23 | 23 |
| Tx band n105 BW(MHz, Lcrb) | 20 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n105 uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n28 receival singal at ANT port, dBm |  | 13 | 13 |
| n28 filter rejection at 663~703MHz, dB | 40 |  |  |
| signal After n28 filter, dBm |  | -31 | -31 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -112 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -108 |
| noise figure dB | 10.5 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -163.5 |  |  |
| Rx band n5 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -96.97 | -96.97 |
| n105 PA noise PSD at PA output port, dBm/Hz | -120 |  |  |
| n105 PA noise PSD at PA output port, dBm/RxBW |  | -53.47 |  |
| n105 Tx filter rejection at 758~803MHz, dB | 35 |  |  |
| n105 PA noise power at Rx n105 ant port at 758~803MHz, dBm |  | -98.47 | -98.47 |
| Total noise level at ANT port |  | -94.45 | -94.45 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(5MHz BW) |  | -95.45 | -95.45 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(5MHz BW), dBm | -96.46 |  |  |
| MSD | 2.04 |  |  |

Proposal 2: to specify the MSD due to cross band isolation from band n105 UL to n28DL.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n28 | 693 | 20 | 15 | 20 (RBstart=86) | 760.5 | 5 | 2 | >ACLR2 |

Discussion on Delta Tib and Rib

Comparing to the Tib and Rib of CA_n5-n28 and CA_n20-n28, 0.7dB Tib for both n28&n105 and 0.2dB Rib for both n28&n105 can be used.

Proposal 3: The following Delta Tib and Rib values for CA_n28-n105 can be considered.

Table 1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28A-n105A | n28 | 0.7 |
|  | n105 | 0.7 |

Table 2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n28A-n105A | n28 | 0.2 |
|  | n105 | 0.2 |

- Summary

Proposal 1: Based on the evaluation, the REFSENS degradation due to cross band isolation from 30MHz n28 UL to n105 DL can be specified below.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n28 | n105 | 718 | 30 | 15 | 25 (RBstart=0) | 649.5 | 5 | 17.0 | >ACLR |

Proposal 2: to specify the MSD due to cross band isolation from band n105 UL to n28DL.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n28 | 693 | 20 | 15 | 20 (RBstart=86) | 760.5 | 5 | 2 | >ACLR2 |

Proposal 3: The following Delta Tib and Rib values for CA_n28-n105 can be considered.

Table 1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28A-n105A | n28 | 0.7 |
|  | n105 | 0.7 |

Table 2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n28A-n105A | n28 | 0.2 |
|  | n105 | 0.2 |

- References

[1] R4-2309093	Draft CR for TR 38.872: TP for CA_n28A-n105A UE RF architectures, Apple

[2] R4-2310255	draftCR for TR 38.872 on CA_n28-n105 1UL and 2UL configurations, Skyworks Solutions, Inc., Qualcomm, Huawei, Spark NZ.

[3]	R4-2306469, WF on RF architecture and critical issues for CA_n28-n105, Apple

[4]	R4-2308582, Discussion on RF architecture and potential MSD issues for CA_n28-n105, Huawei, HiSilicon, Spark NZ