3GPP TSG-RAN WG4 Meeting # 108	R4-2312974

# Toulouse, France, August 21 – August 25, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-18 NTN coexistence study assumption

Agenda Item:	8.26.2

Document for:	Approval

Introduction

In RAN4#107 meeting, a WF [1] for above 10GHz NTN-TN co-existence study was approved with a lot of agreements and some open issues for further discussion. Except for that, we still have to discuss the NTN UL power control based on the simulation assumptions [2]. In this paper, we’d like to further discuss these parts and open issues.

Discussion on NTN UE UL power control

In the previous meetings, RAN4 hasn’t discussed UL power control for NTN UE. Based on the power control principle, we assume the following CLx-ile

CLx-ile = 10*log10(Pmax) – (SNRtarget + 10*log10(kTBF) )

Currently NTN UE maximum output power is assumed as 33dBm. And the targeted SNR is 15dB. The thermal noise is about -85.1dBm if 200MHz channel bandwidth is assumed. Thus, we can get CLx-ile = 33dBm – (15 - 85.1dBm) = 103.1.

Proposal 1: To apply the following UL power control for NTN UE.

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 could be applied for NTN UE with following parameters.

Where, Pmax = 33dBm, Rmin = 60 dB, CLx-ile and γ are set as following:

-	CLx-ile = 103.1,

-	γ = 1 For uplink scenario

Discussion on SAN antenna pattern

In last meeting, the following updates about SAN antenna pattern was approved.

- Take the normalised SAN antenna pattern for Ka-Band coexistence studies and related half-power beam-width (2θ-3dB). Equation; Half-power beam-width 2θ-3dB Value; GEO constellation type @ 17GHz; LEO constellation type @ 17GHz. Fθ=2 J1uu; 2θ-3dB=2×Arcsin1.616×λπD; 0.1763 degrees; 1.7337 degrees. where: - Jix is the Bessel function of first type and ith order with argument x - is the angle in a θ;φ spherical coordinates system, - u=πDλsin⁡(θ) - D : Antenna diameter - : Wavelength

A problem is idenfied for this proposal that J1u can be negative number which can’t be considered as antenna gain. Another issue is that the new antenna pattern is different from what we used in TR 38.811. In order to align with each other as soon as possible, it’s proposed to use the antenna pattern in TR 38.811

Proposal 2: To use the antenna pattern in section 6.4.1 of TR 38.811 for Ka band SAN in order to align with each other as soon as possible.

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered.

1                for θ=0

4J1kasinθkasinθ2        for 0<θ≤90°

where:

-	J1(x) is the Bessel function of the first kind and first order with argument;

-	x, a is the radius of the antenna's circular aperture;

-	k = 2f/c is the wave number;

-	f is the frequency of operation;

-	c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

Discussion on Atmospheric loss

In order to simplify the coexistence simulation, it’s proposed to use a fixed value 2dB as Atmospheric loss.

Proposal 3: To use a fixed value 2dB as Atmospheric loss for Ka band coexistence study.

Summary

Based on the discussion above, the following observations and proposals are listed.

Proposal 1: To apply the following UL power control for NTN UE.

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 could be applied for NTN UE with following parameters.

Where, Pmax = 33dBm, Rmin = 60 dB, CLx-ile and γ are set as following:

-	CLx-ile = 103.1,

-	γ = 1 For uplink scenario

Proposal 2: To use the antenna pattern in section 6.4.1 of TR 38.811 for Ka band SAN in order to align with each other as soon as possible.

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered.

1                for θ=0

4J1kasinθkasinθ2        for 0<θ≤90°

where:

-	J1(x) is the Bessel function of the first kind and first order with argument;

-	x, a is the radius of the antenna's circular aperture;

-	k = 2f/c is the wave number;

-	f is the frequency of operation;

-	c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

Proposal 3: To use a fixed value 2dB as Atmospheric loss for Ka band coexistence study.

- References

[1] R4-2309767, WF for co-existence study of Above 10GHz NTN band, Samsung

[2] R4-2309771, Simulation assumptions for NTN co-existence study in bands above 10GHz, Samsung

[3]