3GPP TSG-RAN WG4 Meeting # 108                               R4-2312979

Toulouse, France, August 21 – August 25, 2023

Source: 	Huawei, HiSilicon, Spark NZ

Title: 	Discussion on RF architecture and potential MSD issues for CA_n5-n28-n105

Agenda Item:	8.3.2.1

Document for:	Approval

Introduction

In the RAN4#107 meeting, an updated work plan for the enhancement of sub-1GHz bands was approved [1]. This followed the revision of SID approved in RAN#100 [2] It was also agreed that RAN4 needs to complete the technical study of CA_n5A-n28A-n105A by the RAN4#108 meeting. The following configurations should be considered.

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n28A-n105A1 | CA_n5A-n28A CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

In this paper, we’d like to discuss the RF architecture and potential MSD issue due to intermodulation of CA_n5A-n28A or CA_n5A-n105A.

UE RF architecture

Referring to the previous study of CA_n8-n20-n28, two-antenna RF architecture, three-antenna RF architecture and four-antenna RF architecture can also be considered for CA_n5-n28-n105. Previous agreements in RAN4 have concluded that two up links n28 and n 105 cannot be simultaneously active due to IMD issues. Here Tx switching ( not shown in the architecture diagrams below) will be required to enable only one band in uplink mode. However, two uplinks n5-n28 and n5-n105 are allowed and a single up link in all three bands is allowed.

For the two-antenna RF architecture, it seems that it’s very difficult to achieve two or three quadplexer/pentaplexer and wide bandwidth low band antenna shown below. However, there are measurements to show quad band feasibility for the low bands and it seems within the realms of feasibility that four antennas covering 698- 960 MHz are possible. Noted that this is not the range covering n5A-n28A-n105A but the bandwidth ratio of the measurements and the former are similar.

Figure 1 two-antenna RF architecture for CA_n5-n28-n105

The antenna relative bandwidth ratio for two-antenna RF architecture is shown in table 1.

Table 1 the antenna relative bandwidth ratio for two-antenna RF architecture

| Antenna | The covered frequency range | Relative bandwidth ratio |
| --- | --- | --- |
| Main TRx antenna | 612~894MHz | 37% |
| Diverstiy TRx antenna | 612~894MHz | 37% |

For three-antenna RF architecture, we provide three types (a) (b) and (c) as below.

Figure 2 three-antenna RF architecture (solution A) for CA_n5-n28-n105 (dual transmitter antenna architecture)

For a three-antenna RF architecture in Fig 2, the UE has also to implement a quadplexer (n5UL/n5DL/n105DL/n105UL). In addition, wide bandwidth antenna is another challenge for this type of UE RF architecture to cover frequency range from 612MHz to 894MHz. The antenna relative bandwidth ratio for three-antenna RF architecture (solution A) is shown in Table 2.

Table 2 the antenna relative bandwidth ratio for three-antenna RF architecture (solution A)

| Antenna | The covered frequency range | Relative bandwidth ratio |
| --- | --- | --- |
| Main TRx antenna 1 | 612~894MHz | 37% |
| Main TRx antenna 2 | 703~803MHz | 13% |
| Diverstiy TRx antenna | 612~894MHz | 37% |

Figure 3 three-antenna RF architecture (solution B) for CA_n5-n28-n105 (“no common diplexer” based RF front-end antenna)

For the three-antenna RF architecture Fig 3, the challenge is to implement a quadplexer (n5UL/n5DL/n105DL/n28DL), but wide bandwidth antenna in main TRx 2 can be tuned at the UL frequency range of band n5, which may help ease the design of low band antenna. The antenna relative bandwidth ratio for three-antenna RF architecture (solution B) is shown in table 3.

Table 3 the antenna relative bandwidth ratio for three-antenna RF architecture (solution B)

| Antenna | The covered frequency range | Relative bandwidth ratio |
| --- | --- | --- |
| Main TRx antenna 1 | 703~803MHz (band n28 is active) 612~703MHz (band n105 is active) | 13% 13.8% |
| Main TRx antenna 2 | 612~894MHz | 37% |
| Diverstiy TRx antenna | 612~894MHz | 37% |

Figure 4 three-antenna RF architecture (solution C) for CA_n5-n28-n105

For three-antenna RF architecture Fig 4, only single UL is supported for CA_n5-n28-n105. Two Rx filter groups are considered and the antenna efficiency can be compromised for pure Rx wide bandwidth antenna. This kind of implementation can be achieved more easily. The antenna relative bandwidth ratio for three-antenna RF architecture (solution C) is shown in table 4.

Table 4 the antenna relative bandwidth ratio for three-antenna RF architecture (solution C)

| Antenna | The covered frequency range | Relative bandwidth ratio |
| --- | --- | --- |
| Main TRx antenna 1 | 824~894MHz (band n5 is active) 703~803MHz (band n28 is active) 612~703MHz (band n105 is active) | 8% 13% 13.8% |
| Main TRx antenna 2 | 612~894MHz | 37% |
| Diverstiy TRx antenna | 612~894MHz | 37% |

In addition, four-antenna RF architecture is also provided as below. For this implementation, there is no need to achieve complex multiplexer or wide bandwidth antenna. But the implementation for four low band antennas are very challenge for smart phone as the size of smartphone is very limited. The antenna relative bandwidth ratio for four-antenna RF architecture is shown in table 5.

Figure 5 four-antenna RF architecture for CA_n5-n28-n105

Table 5 the antenna relative bandwidth ratio for four-antenna RF architecture

| Antenna | The covered frequency range | Relative bandwidth ratio |
| --- | --- | --- |
| Main TRx antenna 1 | 612~703MHz (band n105 is active) | 13.8% |
| Main TRx antenna 2 | 824~894MHz (band n5 is active) | 8% |
| Main TRx antenna 3 | 703~803MHz (band n28 is active) | 13% |
| Diverstiy TRx antenna | 612~894MHz | 37% |

Observation 1: from UE implementation perspective, two-antenna RF architecture, three-antenna RF architecture and four-antenna RF architecture can be considered for CA_n5-n28-n105. But the implementation or the number of low band antennas have different advantages or disadvantages.

1) For two-antenna UE RF architecture, although only two low band antennas are kept due to the compact size of smartphone, UE has to face the difficulties of wide bandwidth low-band antenna and complex multiplexer.

2) For three-antenna UE RF architecture, one more low-band antenna can help ease the design of wide bandwidth antenna and avoid to implement the most complex multiplexer comparing to two-antenna UE RF architecture.

3) For four-antenna UE RF architecture, even if UE can avoid implementing wide bandwidth low-band antenna and complex multiplexer, UE pays the price of space and antenna performance.

Proposal 1: it’s recommended to implement three-antenna UE RF architecture for CA_n5-n28-n105 as this implementation can balance the complexity of filter/antenna within the form factor of a UE.

Discussion on Delta Tib and Rib

Referring to the endorsed CR [3], RAN4 has agreed to specify Delta Tib and Rib for CA_n5-n28 as below.

Table 6 ΔTIB,c and ΔRIB,c for CA_n5-n28

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] | ΔRIB,c [dB] |
| --- | --- | --- | --- |
| CA_n5A-n28A | n5 | 0.7 | 0.2 |
|  | n28 | 0.7 | 0.2 |

Referring to the draft CR [4] for CA_n28-n105, RAN4 has approved the following candidate values for further discussion.

For CA_n28-n105 the 2 antenna implementation should drive the TIB,c and RIB,c values, three companies provided input that is summarized in the two tbles below. Table 5.5.3.4-1: ΔTIB,c due to NR CA (two bands) Inter-band CA combination; ΔTIB,c for NR bands (dB)9. Component band in order of bands in configuration10 Proposed values by company A / B / C. CA_n28-n105; 1.0 / 0.7 / 0.8; 1.0 / 0.7 / 0.8. NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3.. Table 5.5.3.4-2: ΔRIB,c due to NR CA (two bands) Inter-band CA combination; ΔRIB,c for NR bands (dB)8. Component band in order of bands in configuration9 Proposed values by company A / B / C. CA_n28-n105; 1.0 / 0.4 / 0.2; 1.0 / 0.4 / 0.2. NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77..

Thus, it’s better to derive ΔTIB,c and ΔRIB,c for CA_n5-n28-n105 based on the band combination CA_n5-n28 and CA_n28-n105.

Proposal 2: Comparing to the Tib and Rib of CA_n5-n28 and CA_n28-n105, the following Delta Tib and Rib values for CA_n5-n28-n105 can be considered.

Table 7 ΔTIB,c and ΔRIB,c for CA_n5-n28-n105

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] | ΔRIB,c [dB] |
| --- | --- | --- | --- |
| CA_n5A-n28A-n105A | n5 | 0.7 | 0.2 |
|  | n28 | Max(0.7, the value from CA_n28-n105) | Max(0.2, the value from CA_n28-n105) |
|  | n105 | the value from CA_n28-n105 | the value from CA_n28-n105 |

Discussion on MSD due to intermodulation

UL n5-n105

Table 8 lists Band n5 + Band n105 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n28.

Table 8 UL CA_n5-n105 IMD product

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 703 | 824 | 849 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 121 | 186 | 1487 | 1552 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 477 | 582 | 945 | 1035 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2150 | 2255 | 2311 | 2401 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1140 | 1285 | 1769 | 1884 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2813 | 2958 | 3135 | 3250 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 372 | 242 | 2974 | 3104 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2733 | 2593 | 1988 | 1803 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1221 | 1066 | 461 | 291 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3959 | 4099 | 3476 | 3661 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3798 | 3953 | 3637 | 3807 |

Observation 2: there is no IMD interference which fall into Rx frequencies of third DL band n28 when CA_n5-n105 is the UL configuration.

UL n5-n28

Table 9 lists Band n5 + Band n28 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, which may fall into the Rx receiver of DL band n105.

Table 9 UL CA_n5-n28 IMD product

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 824 | 849 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 76 | 146 | 1527 | 1597 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 557 | 672 | 900 | 995 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2230 | 2345 | 2351 | 2446 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1260 | 1420 | 1724 | 1844 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2933 | 3093 | 3175 | 3295 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 292 | 152 | 3054 | 3194 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2693 | 2548 | 2168 | 1963 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1141 | 976 | 596 | 411 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3999 | 4144 | 3636 | 3841 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3878 | 4043 | 3757 | 3942 |

Observation 3: 3rd order IMD may also fall into mobile receive frequencies of third DL band n105 when CA_n5-n28 is the UL configuration.

There are network-based mitigation issues discussed later to address any potential impact of the IMD.

Three-antenna RF architecture Fig 2 and Fig 3 respectively are used to derive the MSD of band n105 when UL CA is CA_n5-n28. These architectures are referred to as dual-transmitter antenna architecture Fig 2 and no common diplexer band RF architecture Fig 3 in 3GPP reference TR 36.860 [7].

The component linearity assumptions, the component attenuation and isolation values are listed in Tables 5 and 6 respectively.

Table 5 General linearity parameters

| Component | IP3(dBm) |
| --- | --- |
| Antenna switch | 68 |
| Diplexer | 86 |
| Triplexer/Duplexer | 74 |
| PA forward mixing | 30 |
| PA reverse mixing | 28 |
| LNA | -6 |

The above values are based on common understanding in RAN4. Company’s contribution [8] is referred.

Table 6 Attenuation and isolation values

| Attenuation and Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna ISO | 10 |  |
| PA gain | 25 |  |
| PCB isolation Paout-Pain | 60 | PCB isolation (PA forward mixing) |
| n28 Tx attenuation at Band n105 Rx | 30 |  |
| n28 Tx attenuation at Band n5 Tx | 30 |  |
| n5 Tx attenuation at n105 RX | 30 |  |
| n5 Tx attenuation at n28 TX | 30 |  |
| n105 Rx attenuation at n5 TX | 40 |  |
| n105 Rx attenuation at n28 RX | 40 |  |

The above values are based on common understanding in RAN4 especially for PA gain and PCB/antenna isolation referring to TR 36.860 [7].

Table 7 summarize the MSD analysis results for DL_n5A-n28A-n105A_UL_n5A-n28A. Some main IMD interference paths were depicted in figure 6.

What Fig 2 and Fig 3 are different architectures, but from MSD calculation point of view, they are essentially the same. This is because these two architectures have the same characteristics. Namely, Tx chains of band n28 and n5 are split under two antennas and the main Rx chain of band n105 is combined with the Tx chain of band n5. Therefore, we only show one table of MSD calculation.

Table 7 MSD calculation for victim Rx of band n105 when UL_n5A-n28A

| No. | Component | IMD3(dBm/5MHz) |  |
| --- | --- | --- | --- |
|  |  | Main_@Antenna | Div_@Antenna |
| 1) | n5 PA Forward mixing interference | -95.305 |  |
| 2) | n28 PA Forward mixing interference | -69.15 |  |
| 3) | n5 PA Reserve mixing interference | -112.5 |  |
| 4) | n28 PA Reserve mixing interference | -76 |  |
|  | n5/n105 quadplexer | -210.5 |  |
|  | n28 Duplexer | -156 |  |
| 5) | n105 LNA via filters | -66 | -79 |
|  | n105 LNA via PCB coupling | -94 | -94 |
|  | Diplexer (L-H) | -133 | -142 |
|  | Antenna switch | -99 | -135.5 |
|  | From main path |  | -73.99 |
|  | Total | -63.99 | -72.77 |

Figure 6 The illustration of main IMD interference paths.

Proposal 3: It’s proposed to capture band n105 MSD due to IMD3 of Tx band n5 + band n28 as below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n28-n105 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n105 | 686 | 5 | 25 | 635 | 22.9 | FDD | IMD3 |

Generally, there are some network deployment methods to avoid the IMD product hitting the DL carrier directly. For example, if the UL carrier in band n28 is configured in 703~718MHz, no matter which UL frequency carrier in band n5 is configured, the IMD3 products will not hit the DL carrier in band n105 directly.

Observation 4: Generally, there are some network deployment methods to avoid the IMD product hitting the DL carrier directly. For example, if the UL carrier in band n28 is configured in 703~718MHz, no matter which UL frequency carrier in band n5 is configured, the IMD3 products will not hit the DL carrier in band n105 directly.

- Summary

Observation 1: from UE implementation perspective, two-antenna RF architecture, three-antenna RF architecture and four-antenna RF architecture can be considered for CA_n5-n28-n105. But the implementation or the number of low band antennas have different advantages or disadvantages.

1) For two-antenna UE RF architecture, although only two low band antennas are kept due to the compact size of smartphone, UE has to face the difficulties of wide bandwidth low-band antenna and complex multiplexer.

2) For three-antenna UE RF architecture, one more low-band antenna can help ease the design of wide bandwidth antenna and avoid to implement the most complex multiplexer comparing to two-antenna UE RF architecture.

3) For four-antenna UE RF architecture, even if UE can avoid implementing wide bandwidth low-band antenna and complex multiplexer, UE pays the price of space and antenna performance.

Proposal 1: it’s recommended to implement three-antenna UE RF architecture for CA_n5-n28-n105 as this implementation can balance the complexity of filter/antenna within the form factor of a UE.

Proposal 2: Comparing to the Tib and Rib of CA_n5-n28 and CA_n28-n105, the following Delta Tib and Rib values for CA_n5-n28-n105 can be considered.

Table 10 ΔTIB,c and ΔRIB,c for CA_n5-n28-n105

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] | ΔRIB,c [dB] |
| --- | --- | --- | --- |
| CA_n5A-n28A-n105A | n5 | 0.7 | 0.2 |
|  | n28 | Max(0.7, the value from CA_n28-n105) | Max(0.2, the value from CA_n28-n105) |
|  | n105 | the value from CA_n28-n105 | the value from CA_n28-n105 |

Observation 2: there is no IMD interference which fall into Rx frequencies of third DL band n28 when CA_n5-n105 is the UL configuration.

Observation 3: 3rd order IMD may also fall into mobile receive frequencies of third DL band n105 when CA_n5-n28 is the UL configuration.

Proposal 3: It’s proposed to capture band n105 MSD due to IMD3 of Tx band n5 + band n28 as below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n28-n105 | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n105 | 686 | 5 | 25 | 635 | 22.9 | FDD | IMD3 |

Observation 4: Generally, there are some network deployment methods to avoid the IMD product hitting the DL carrier directly. For example, if the UL carrier in band n28 is configured in 703~718MHz, no matter which UL frequency carrier in band n5 is configured, the IMD3 products will not hit the DL carrier in band n105 directly.

- References

[1] R4-2307065, Work plan for enhancement for sub-1GHz band combinations, Spark NZ Ltd

[2] RP-231141, SID revision: Study on enhancement for sub-1GHz band combinations, Huawei, HiSilicon

[3] R4-2309005, CR for 38.101-1, introduction of CA_n5-n28, Xiaomi, Huawei, HiSilicon, ZTE, CATT

[4] R4-2310255, draft CR for TR 38.872 on CA_n28-n105 1UL and 2UL configurations, Skyworks Solutions, Inc., Qualcomm, Huawei, Spark NZ.

[5] Yu-Chen Chen et al, “For LTE Low-Band MIMO Antennas for the Smartphone”, IEEE Asia Pacific Conference on Antennas and Propagation, 2016.

[6] Manoj Stanley et al, “ A Novel Reconfigurable Metal Rim Integrated Open Slot Antennas for Octa- Band Smartphone Applications “, IEEE Transactions Antennas and Propagation, Vol 10, no 7, July 2017 pp 3352- 3363

[7] TR 36.860, LTE-Advanced dual uplink inter-band Carrier Aggregation (CA), V12.0.0

[8] R4-1801626, MSD analysis for DC_3A-7A_n78A, DC_3A-20A_n78A and DC_7A-20A_n78A, Huawei, HiSilicon