3GPP TSG-RAN WG4 Meeting #108							R4-2312988

Toulouse, France, August 21 – August 25, 2023

Agenda Item:	8.30.2.1

Source: 	OPPO

Title: 	on the SL-U MRP simulation results

Document for:	Approval

- Introduction

The sidelink unlicensed MPR-A-MPR simulation assumption have been discussed and captured in the TR. Based on the simulation assumption, and also some general consideration as specified in our companion paper, we have done the simulation for SL-U MPR and A-MPR. The simulation results are shown in this paper and with the results, we propose the MPR and A-MPR for the SL-U.

- Discussion

The SL-U MPR simulation assumption is captured below and the MPR A-MPR simulation are captured based on that:

UE maximum output power reduction for NR SL-U

The following assumption can serve as a starting point for MPR simulation assumptions that are considered to derive MPR requirements for SL-U operation.

- Basic RF parameters for MPR of single carrier SL-U UE

  - Waveforms: CP-OFDM for SL-U

  - Supported CBW: 20/40/60/80/100MHz.

  - Modulation: QPSK/16-QAM/64-QAM/256-QAM

  - SCS: 15/30/60kHz

  - C-IMD: 45dBc or 60dBc

  - EVM and impairments

| Modulation | SystemEVM | PA only | Image | PA+image |
| --- | --- | --- | --- | --- |
| QPSK | 17.5% | 10% | 28dB | 10.0% |
| 16QAM | 12.5% | 8% | 28dB | 8.0% |
| 64QAM | 8% | 4% | 28dB | 5.65% |
| 256QAM | 3.5% | 1.8% | 34dB | 2.69% |

  - IBE and impairment exceptions are same as NR-U

- Reuse IBE mask from TS38.10101 Table 6.4F.2.3-1

- Image/carrier exceptions position and specific test according to interlaces.

  - ACLR: 27dBc for both PC3/PC5 UE with NR MBW

  - SEM according to 38.101-1 clause 6.5F.2.2

  - PA calibration & PA configuration:

- PC3 (both single PA and dual PAs with 20+20): 1dB MPR and DFT-s-OFDM QPSK with 100RB0 20MHz waveform with 30dB ACLR

- PC5 (single PA): 1dB MPR and DFT-s-OFDM QPSK 100RB3 20MHz waveform with 27dB ACLR

  - MPR table format can be reused the MPR format of NR-U according to each transmitted channel e.g.PSSCH/PSCCH transmission, PSFCH transmission and PSBCH transmission.

  - The MPR will be applied to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel.

  - Follow the RB interlaced allocation based on Table 6.2F.2-1 and Table 6.2F.2-2 in TS38.101-1. Also refer the interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. FFS on the inter-laced RB allocation for PSFCH, S-SSB by RAN1

For SL-U in single CC operation, the simultaneous transmission of PSCCH and PSSCH in the same sub-frame is supported and the following constraints in Table 6.1.2-1 can be assumed.

Table 6.1.2-1: SL-U operation’s MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes | Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,125,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216,220,225,228,230,240,250,252,255,260,264,270 The Allowed LCRB will be decided by the supported SCS. |
| Regarding PSCCH / PSSCH multiplexing |  |
| PSCCH size | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH | 0dB |

With our companion paper, the simulation cases are discussed and shown again as below table 1.

Below is a table for better understanding. For each case, the QPSK, 16QAM, 64QAM and 256QAM will be simulated.

Table 1 simulation cases

|  | case | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 15 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 |
|  | 3 | CP-OFDM | 20 | 24RB0 | 60 |
|  | 4 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 5 | CP-OFDM | 40 | 105RB0 | 30 |
|  | 6 | CP-OFDM | 40 | 50RB0 | 60 |
|  | 7 | CP-OFDM | 60 | 160RB0 | 30 |
|  | 8 | CP-OFDM | 60 | 80RB0 | 60 |
|  | 9 | CP-OFDM | 80 | 216RB0 | 30 |
|  | 10 | CP-OFDM | 80 | 105RB0 | 60 |
|  | 11 | CP-OFDM | 100 | 270RB0 | 30 |
|  | 12 | CP-OFDM | 100 | 135RB0 | 60 |
| Interlaced Allocation Single CC | 13 | CP-OFDM | 20 | 1 RB0 every 10RBs | 15 |
|  | 14 | CP-OFDM | 20 | 1RB0 every 5RBs | 30 |
|  | 15 | CP-OFDM | 40 | 1RB0 every 10RBs | 15 |
|  | 16 | CP-OFDM | 40 | 1RB0 every 5RBs | 30 |
|  | 17 | CP-OFDM | 60 | 1RB0 every 5RBs | 30 |
|  | 18 | CP-OFDM | 80 | 1RB0 every 5RBs | 30 |
|  | 19 | CP-OFDM | 100 | 1RB0 every 5RBs | 30 |
| Wide band operation | 20 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 21 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 22 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 23 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 24 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 25 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 26 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 27 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 28 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 29 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 30 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 31 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 32 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 33 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 34 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 35 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 36 | CP-OFDM | 100 | Bitmap 00100 | 30 |

Corresponding MPR results are shown as below for case 1 to 18:

Figure 1, Contiguous and Interlaced Single CC MPR

From NR-U requirement perspective, the NR-U MPR requirement has been set as shown below captured from TS 38.101-1 while only the CP-OFDM part has been captured.

Table 1: NR-U MPR requirement

| Pre-coding | Modulation | RB Allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) | Exception for 100MHz Full5 (dB) |

| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3.5 | ≤ 4.5 |
| --- | --- | --- | --- | --- |
|  | 16 QAM | ≤ 4.0 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 |  |

Also the full and partial are defined besides the full RB and interlaced RB allocation together with the wide-band operation bit-maps. Table 2 below captures the exception MPR mapping for wide-band operation of different bit-maps.

Table 2: Exception MPR mapping for wideband operation

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 40 | 10, 01 |
| 60 | None |
| 80 | 1100, 0011, 0100, 0010 |
| 100 | 00111, 11100, 00011, 11000 |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. NOTE 2: Void. |  |

It has also been agreed in previous meeting that the similar MPR table can be reused for SL-U and hence we would like to propose similar MPR requirement as together with table 1 and table 2 with different MPR requirement for full and partial cases with the exception.

Observation 1: For NR-U MPR requirement, full and partial are defined with exception MPR mapping of wide-band operation and SL-U MPR requirement can use similar mechanism.

For case 1 to 12, the contiguous cases are simulated. For QPSK, 16QAM, 64QAM, 256QAM, the result are all quite stable, with largest MPR value of 3.3dB, 3.9dB 5.7dB and 8.9dB. In such case, to give enough margin for PA implementation, 1dB margin is proposed and corresponding MPR requirement is as 4.5dB, 5dB, 7dB and 10dB.

For case 13 to 19, the interlaced cases are simulated. For QPSK, 16QAM, 64QAM, 256QAM, the result are all quite stable, with largest MPR value of 3.4dB, 3.4dB 4.6dB and 7.9dB. In such case, to give enough margin for PA implementation, 1dB margin is proposed and corresponding MPR requirement is as 4.5dB, 4.5dB, 6dB and 9dB.

Hence it is proposed below as table 3:

Table 3: Proposed SL-U MPR requirement for single CC

| Precoding | Modulation | Full | Partial |
| --- | --- | --- | --- |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 5 | ≤ 4.5 |
|  | 64 QAM | ≤ 7 | ≤ 6 |
|  | 256 QAM | ≤ 10 | ≤ 9 |

For the wide-band operation, for case 20 to 36, both contiguous full RB and the interlaced-RB allocation has been simulated and the results can be found below:

Figure 2 contiguous Wide-band MPR

Figure 3: InterlacedWide-band MPR

The exception bitmap is selected based on figure 2 as “010” “00100” “0110” “1110” “01110” and “11110”.  Also to consider the case of symmetric, the bitmap “1110” is similar to “0111” and “11110” is similar to “01111”.

Proposal 1: The exception bitmaps are proposed as:

Table 4: Exception bitmap mapping

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 60 | 010 |
| 80 | 1110, 0110, 0111 |
| 100 | 11110, 00100, 01110, 01111 |

For case 20 to 36, the contiguous cases are simulated. For the exception bitmaps, for QPSK, 16QAM, 64QAM, 256QAM, the result are all quite stable, with largest MPR value of 3.0dB, 3.8dB 5.3dB and 8.7dB. From our proposed table 3 above, the MPR for full RB allocation can cover the exception bitmaps cases.

Observation 2: For wide-band operation, the exception bitmaps MPR can be mapped to “full” RB allocation requirements.

Besides the exception bitmaps, for QPSK, 16QAM, 64QAM, 256QAM, the result are all quite stable, with largest MPR value of 3.5dB, 3.6dB 5.0dB and 7.8dB. From our proposed table 3 above, the MPR for partial RB allocation can cover the bitmaps besides the exception cases.

Observation 3: For wide-band operation, besides the exception bitmaps, the MPR can be mapped to “partial” RB allocation.

With that, it is proposed to use table 3 as listed above for the SL-U PSSCH/PSCCH MPR requirement with the table 4 exception table 4 mapping.

Proposal 2: The SL-U MPR requirement is proposed as below:

Table 3: Proposed SL-U MPR requirement for single CC

| Precoding | Modulation | Full | Partial |
| --- | --- | --- | --- |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 5 | ≤ 4.5 |
|  | 64 QAM | ≤ 7 | ≤ 6 |
|  | 256 QAM | ≤ 10 | ≤ 9 |

Table 4: Exception bitmap mapping

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 60 | 010 |
| 80 | 1110, 0110, 0111 |
| 100 | 11110, 00100, 01110, 01111 |

Note: For bitmaps included in table 4, Full RB allocation requirement apply.

3	Conclusions

In this contribution, we give initial discussion on the sidelink evolution and the observation and proposals are shown as below:

Observation 1: For NR-U MPR requirement, full and partial are defined with exception MPR mapping of wide-band operation and SL-U MPR requirement can use similar mechanism.

Observation 2: For wide-band operation, the exception bitmaps MPR can be mapped to “full” RB allocation requirements.

Observation 3: For wide-band operation, besides the exception bitmaps, the MPR can be mapped to “partial” RB allocation.

Proposal 1: The exception bitmaps are proposed as:

Table 4: Exception bitmap mapping

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 60 | 010 |
| 80 | 1110, 0110, 0111 |
| 100 | 11110, 00100, 01110, 01111 |

Proposal 2: The SL-U MPR requirement is proposed as below:

Table 3: Proposed SL-U MPR requirement for single CC

| Precoding | Modulation | Full | Partial |
| --- | --- | --- | --- |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 5 | ≤ 4.5 |
|  | 64 QAM | ≤ 7 | ≤ 6 |
|  | 256 QAM | ≤ 10 | ≤ 9 |

Table 4: Exception bitmap mapping

| Wideband operation channel bandwidth (MHz) | Sub-band configuration exceptions |
| --- | --- |
| 60 | 010 |
| 80 | 1110, 0110, 0111 |
| 100 | 11110, 00100, 01110, 01111 |

Note: For bitmaps included in table 4, Full RB allocation requirement apply.

4 References

[1] TR 38.786