3GPP TSG-RAN WG4 Meeting #108							R4-2312990

Toulouse, France, August 21 – August 25, 2023

Agenda Item:	8.30.2.1.2

Source: 	OPPO

Title: 	on remaining issue for sidelink TX requirement

Document for:	Approval

- Introduction

During the RAN4#107 meeting, the SL-U TX requirement has been fully discussed and a TP [1] to TR 38.786 has been agreed. Furthermore, by checking the TR 38.786, most of the TX requirements have been agreed with some left over issues and hence in this paper we try to give conclusion on such issues.

- Discussion

As stated in the introduction part, currently the unfinished sub-clauses for SL-U TX requirements are listed as below:

Table 1 remaining unfinished part for SL-U

| Sub-clause | Chapter |
| --- | --- |
| 6.1.1 | Maximum output power for SL-U |
| 6.1.2 | UE MPR for SL-U |
| 6.1.3 | UE A-MPR for SL-U |

For the UE power class, it has been discussed and agreed in the RAN4#106-bis-e meeting and the agreed WF [2] is captured as below:

Issue 3-1-1: Power class in SL-U

- Option 1: Power class 5 should be considered firstly for the sidelink operation in unlicensed spectrum. [Huawei]

- Option 2: RAN4 focus on completing PC5 before the introduction of other power classes. [OPPO]

- Option 3: Both PC3 and PC5 shall be considered for SL-U operation in unlicensed bands. [Meta]

Agreement：

- Power class 5 is agreed to be developed as first priority, and PC3 can be further considered in case sufficient progress has been made for power class 5.

Based on our understanding, considering the timeline for Rel-18 is quite limited, we still have SL-U MPR/A-MPR requirement to be discussed and furthermore, the SL CA has just started. Furthermore, as currently the RAN4 frame work, the high power UE for each band will be discussed in a new separated WID. Similarly, for NR SL PC2, it is discussed in Rel-16 however the work is finished in Rel-17.

Observation 1: The Rel-18 Sidelink enhancement WID workload is high.

Observation 2: Higher power class is usually finished in a separate new WID in RAN4, i.e. NR SL PC2.

With that, it is propose to only work on power class 5 in SL-U single CC requirement.

Proposal 1: Only work on power class 5 in SL-U single CC requirement.

For the SL-U MPR and A-MPR requirement, it is discussed in a separate document while still some general issues can be discussed here.

For the A-MPR requirement, the RAN4 specification use NS values to capture the corresponding additional MPR for different regulation requirements. There is a mapping relationship between different bands and regulations. Currently the bands for NR-U n46, n96 and n102 with there corresponding NS values mapping has been agreed as shown below:

Table 6.2F.3.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_28 | NS_29 | NS_30 | NS_31 |  |  | Reserved |
| n96 | NS_01 | NS_53 | NS_54 | NS_59 | NS_60 | NS_61 |  | Reserved |
| n102 | NS_01 | NS_58 | NS_63 |  |  |  |  | Reserved |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

Table 6.2F.3.1-1B: Mapping of extended network signaling label

| NR band | Value of extendedAdditionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| n96 | NS_66 | NS_67 |  |  |  |  |  |  |
| n102 | NS_64 | NS_65 | NS_68 | NS_69 |  |  |  |  |
| NOTE: extendedAdditionalSpectrumEmission corresponds to an information element of the name [EXTENDED_ additionalSpectrumEmission] defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

Currently in NR NTN, if the regulation requirement are the same and corresponding A-MPR can be re-used, the same NS values will be used for both NR and NR NTN. While if the requirement is not the same as satellite system is using different requirements or the A-MPR is different, then a new NS value will be used and the numbering will not follow the legacy NS values but a new value for NTN only. This can be seen as below mappings from TS 38.101-5:

Table 6.2.3.1-1A: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

Observation 3: For NR NTN, if the regulation requirement are the same and corresponding A-MPR can be re-used, the NS value can be reused from NR.

Observation 4: For NR NTN, if the requirement is not the same or the A-MPR is different, then a new NS value will be used and starting from a new sequence with letter N in the end.

With that, we believe similar approach can apply also for SL-U. if the regulation requirement are the same and corresponding A-MPR can be re-used, the NS value can be reused from NR-U for SL-U. If the requirement is not the same or the A-MPR is different, then a new NS value will be used and starting from a new sequence with letter S in the end.

Proposal 2: Similar approach of NR NTN NS value definition can apply to SL-U. The principle is as following:

-If the regulation requirement are the same and corresponding A-MPR can be re-used, the NS value can be reused from NR-U for SL-U.

- If the requirement is not the same or the A-MPR is different, then a new NS value will be used and starting from a new sequence with letter S in the end.

3	Conclusions

In this contribution, we give initial discussion on the sidelink evolution and the observation and proposals are shown as below:

Proposal 1: Only work on power class 5 in SL-U single CC requirement.

Proposal 2: Similar approach of NR NTN NS value definition can apply to SL-U. The principle is as following:

-If the regulation requirement are the same and corresponding A-MPR can be re-used, the NS value can be reused from NR-U for SL-U.

- If the requirement is not the same or the A-MPR is different, then a new NS value will be used and starting from a new sequence with letter S in the end.

4 References

[1] R4-2310308		on the SL-e TX requirement, OPPO

[2] R4-2306631		WF on SLU single CC Tx requirements, Xiaomi