3GPP TSG-RAN WG4 Meeting #108							R4-2312992

Toulouse, France, August 21 – August 25, 2023

Agenda Item:	8.30.2.1.3

Source: 	OPPO

Title: 	on remaining issue for sidelink RX requirement

Document for:	Approval

- Introduction

During the RAN4#107 meeting, the SL-U RX requirement has been fully discussed and a TP [1] to TR 38.786 has been agreed. Furthermore, by checking the TR 38.786, most of the RX requirements have been agreed with some left over issues and hence in this paper we try to give conclusion on such issues.

- Discussion

As stated in the introduction part, currently the unfinished sub-clauses for SL-U RX requirements is the REFSENS requirement and its RB configuration. Currently the TR 38.786 has captured the requirement with “[]” as shown below:

Table 7.1.1-2 NR SL-U RB configurations for REFSENS

|  |  | NR band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Band | SCS kHz | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | [135] |  |
| n96 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | [135] |  |
| n102 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | [135] |  |

With the agreed SNR and RB configuration as above, the REFNSENS requirement for NR SL-U is  shown in Table 7.1.1-3.

Table 7.1.1-3 REFSENS for SL-U

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | [-89.2 + 10log10(NRB/105)] | HD |
|  | 30 | 20, 40, 60, 80, 100 | [-89.4 + 10log10(NRB/50)] |  |
|  | 60 | 20, 40, 60, 80, 100 | [-89.6 + 10log10(NRB/24)] |  |
| n96, n102 | 15 | 20, 40 | [-88.7 + 10log10(NRB/105)] | HD |
|  | 30 | 20, 40, 60, 80, 100 | [-88.9 + 10log10(NRB/50)] |  |
|  | 60 | 20, 40, 60, 80, 100 | [-89.1 + 10log10(NRB/24)] |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the RB configuration for NR SL-U REFSENS as shown in table 7.1.1-2 |  |  |  |  |

During the last RAN4 meeting, only the PRB configuration of 60kHz 100MHz is left with “[]”. Again, we look back to the subchannel size limitation and the subchannelsize is limited to 10/12/15/20/25/50/75/100 PRBs as captured below for TS 38.331.

-

sl-SubchannelSize-r16              ENUMERATED {n10, n12, n15, n20, n25, n50, n75, n100}

In such case, the 135 RB configuration can be confirmed and the “[]” can be removed.

Proposal 1: To agree on 135 RBs for SL-U 60kHz 100MHz configuration and remove the “[]” of current version of TR 38.786.

Furthermore, for the SL-U REFSENSE requirement, there is “[]” for the detail requirement. Compare to the NR-U equation, there is 0.5dB due to different SNR and the RB numbers for 20MHz is specified for sidelink. As the RB numbers can be agreed, the REFSENS requirement can also be agreed.

Proposal 2: To agree on REFSENS requirement for SL-U with equation as below:

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | -89.2 + 10log10(NRB/105) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -89.4 + 10log10(NRB/50) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.6 + 10log10(NRB/24) |  |
| n96, n102 | 15 | 20, 40 | -88.7 + 10log10(NRB/105) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -88.9 + 10log10(NRB/50) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.1 + 10log10(NRB/24) |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the RB configuration for NR SL-U REFSENS as shown in table 7.1.1-2 |  |  |  |  |

Another issue is the four antenna port reference sensitivity allowance. It is proposed to re-use the same ΔRIB,4R as NR-U to SL-U.

Table 7.3F.2-2: Four antenna port reference sensitivity allowance ΔRIB,4R

| Operating band | ΔRIB,4R (dB) |
| --- | --- |
| n46, n96, n102 | -2.2 |

Proposal 3: It is proposed to re-use the same ΔRIB,4R as NR-U to SL-U.

In the last, the SL-U FRC issue has been raised and as captured in TR 38.786, the SL-U RX requirement should use the SL specific FRC. From our opinion, we see reusing the NR SL RMC is applicable since the fundamental function of sidelink has not been changed. The legacy NR SL FRC only supports up to 40MHz while for SL-U the CBW can be up to 100MHz. Hence the FRC for SL-U can be updated based on NR SL FRC with larger channel bandwidth.

Proposal 4: FRC for SL-U to be updated based on NR SL FRC with larger channel bandwidth.

Based on the clause A.7, it is proposed the following FRC with 60,80 and 100MHz channel bandwidth added.

For 15kHz SCS, only up to 40MHz is supported hence the table A.7.2-1 can be reused.

Table A.7.2-1: Fixed reference channel for V2X receiver requirements (SCS 15 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 53 | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Subchannel size |  | 12 | 10 | 15 | 10 | 12 |
| Allocated resource blocks |  | 24 | 50 | 105 | 160 | 216 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK |
| Transport Block Size |  | 1608 | 3624 | 7936 | 12296 | 16896 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 1 | 1 | 2 | 3 |
| Beta offset for 2nd stage SCI |  | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 5160 | 12036 | 26556 | 41076 | 55860 |
| Max. Throughput averaged over 100ms | Mbps | 0.1608 | 0.3624 | 0.7936 | 1.2296 | 1.6896 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. NOTE 3: The CBW is only applicable for PS UE in n14. |  |  |  |  |  |  |

Proposal 5: To reuse current NR SL FRC for 15kHz SCS for QPSK, 64QAM and 256QA.

For 30kHz SCS, up to 100MHz is supported hence the table A.7.2-2 needs to be updated with 60/80 and 100MHz added..

Table A.7.2-2: Fixed reference channel for V2X receiver requirements (SCS 30 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| Subchannel size |  | 12 | 10 | 15 | 15 | 10 | 12 | 15 |
| Allocated resource blocks |  | 24 | 50 | 75 | 105 | 160 | 216 | 270 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Transport Block Size |  | 1608 | 3624 | 5632 | 7936 | 12296 | 16392 | 21000 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 1 | 1 | 1 | 2 | 2 | 3 |
| Beta offset for 2nd stage SCI |  | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 5160 | 12036 | 18636 | 26556 | 41076 | 55860 | 70116 |
| Max. Throughput averaged over 100ms | Mbps | 0.3216 | 0.7248 | 1.1264 | 1.5872 | 2.4592 | 3.2784 | 4.2 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

For 60kHz SCS, up to 100MHz is supported hence the table A.7.2-2 needs to be updated with 60/80 and 100MHz added..

Table A.7.2-3: Fixed reference channel for V2X receiver requirements (SCS 60 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 | 60 | 60 | 60 |
| Subchannel size |  | 10 | 12 | 12 | 10 | 15 | 15 | 15 |
| Allocated resource blocks |  | 10 | 24 | 36 | 50 | 75 | 105 | 135 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Transport Block Size |  | 456 | 1608 | 2536 | 3624 | 5504 | 7936 | 10248 |
| Transport block CRC | Bits | 16 | 16 | 16 | 16 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Beta offset for 2nd stage SCI |  | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 7 | 7 | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 1464 | 5160 | 8328 | 12036 | 18636 | 26556 | 34476 |
| Max. Throughput averaged over 100ms | Mbps | 0.1824 | 0.6432 | 1.0144 | 1.4496 | 2.2016 | 3.1744 | 4.0992 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

Similar approaches apply to 64QAM and 256QAM.

Table A.7.3-1: Fixed reference channel for V2X receiver requirements (SCS 15 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 53 | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Subchannel size |  | 12 | 10 | 15 | 10 | 12 |
| Allocated resource blocks |  | 24 | 50 | 105 | 160 | 216 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Transport Block Size |  | 11528 | 27144 | 60456 | 92200 | 127080 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 2 | 4 | 8 | 11 | 16 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 15336 | 35964 | 79524 | 123084 | 167436 |
| Max. Throughput averaged over 100ms | Mbps | 1.1528 | 2.7144 | 6.0456 | 9.22 | 12.708 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. NOTE 3: The CBW is only applicable for PS UE in n14. |  |  |  |  |  |  |

Table A.7.3-2: Fixed reference channel for V2X receiver requirements (SCS 30 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| Subchannel size |  | 12 | 10 | 15 | 15 | 10 | 12 | 15 |
| Allocated resource blocks |  | 24 | 50 | 75 | 105 | 160 | 216 | 270 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Transport Block Size |  | 11528 | 27144 | 42016 | 60456 | 92200 | 125016 | 155776 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 2 | 4 | 5 | 8 | 11 | 15 | 19 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 |  |  |  |
| Binary Channel Bits per Slot |  | 15336 | 35964 | 55764 | 79524 | 123084 | 167436 | 210204 |
| Max. Throughput averaged over 100ms | Mbps | 2.3056 | 5.4288 | 8.4032 | 12.091 | 18.44 | 25.0032 | 31.1552 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

TableA.7.3-3: Fixed reference channel for V2X receiver requirements (SCS 60 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 | 60 | 60 | 60 |
| Subchannel size |  | 10 | 12 | 12 | 10 | 15 | 15 | 15 |
| Allocated resource blocks |  | 10 | 24 | 36 | 50 | 75 | 105 | 135 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Transport Block Size |  | 3240 | 11528 | 18960 | 27144 | 42016 | 59432 | 77896 |
| Transport block CRC | Bits | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 2 | 3 | 4 | 5 | 8 | 10 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 7 | 7 | 1 |  |  |  |
| Binary Channel Bits per Slot |  | 4248 | 15336 | 24840 | 35964 | 55764 | 79524 | 103284 |
| Max. Throughput averaged over 100ms | Mbps | 1.296 | 4.6112 | 7.584 | 10.858 | 16.8064 | 23.7728 | 31.1584 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

A.7.4	FRC for maximum input level for 256QAM

For V2X transmission over PC5, Table A.7.4-1, Table A.7.4-2 and Table A.7.4-3 are applicable for Maximum input level when the 256QAM is supported.

Table A.7.4-1: Fixed reference channel for V2X receiver requirements (SCS 15 kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 53 | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Subchannel size |  | 12 | 10 | 15 | 10 | 12 |
| Allocated resource blocks |  | 24 | 50 | 105 | 160 | 216 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |  |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Transport Block Size |  | 15880 | 36896 | 81976 | 127080 | 172176 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 2 | 5 | 10 | 16 | 21 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 3 | 3 | 3 | 3 | 3 |
| Binary Channel Bits per Slot |  | 20544 | 48000 | 106080 | 164160 | 223296 |
| Max. Throughput averaged over 100ms | Mbps | 1.588 | 3.6896 | 8.1976 | 12.708 | 17.218 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. NOTE 3: The CBW is only applicable for PS UE in n14. |  |  |  |  |  |  |

Table A.7.4-2: Fixed reference channel for V2X receiver requirements (SCS 30 kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| Subchannel size |  | 12 | 10 | 15 | 15 | 10 | 12 | 15 |
| Allocated resource blocks |  | 24 | 50 | 75 | 105 | 160 | 216 | 270 |
| MCS Index |  | 23 | 23 | 23 | 23 | 24 | 24 | 24 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |  |  |  |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Transport Block Size |  | 15880 | 36896 | 58384 | 81976 | 127080 | 172176 | 217128 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 2 | 5 | 7 | 10 | 16 | 21 | 26 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 3 | 3 | 3 | 3 |  |  |  |
| Binary Channel Bits per Slot |  | 20544 | 48000 | 74400 | 106080 | 164160 | 223296 | 280320 |
| Max. Throughput averaged over 100ms | Mbps | 3.176 | 7.3792 | 11.677 | 16.395 | 25.416 | 34.4352 | 43.4256 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

Table A.7.4-3: Fixed reference channel for V2X receiver requirements (SCS 60kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 | 60 | 60 | 60 |
| Subchannel size |  | 10 | 12 | 12 | 10 | 15 | 15 | 15 |
| Allocated resource blocks |  | 10 | 24 | 36 | 50 | 75 | 105 | 135 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |  |  |  |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Transport Block Size |  | 4480 | 15880 | 25608 | 36896 | 57376 | 81976 | 106576 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 2 | 4 | 5 | 7 | 10 | 13 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 3 | 3 | 3 | 3 |  |  |  |
| Binary Channel Bits per Slot |  | 5760 | 20544 | 33216 | 48000 | 74400 | 106080 | 137760 |
| Max. Throughput averaged over 100ms | Mbps | 1.792 | 6.352 | 10.243 | 14.758 | 22.9504 | 32.7904 | 42.6304 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

Proposal 6: It is proposed the SL-U FRC as above with same FRC below 40MHz and adding 60, 80 and 100MHz CBW FRC.

3	Conclusions

In this contribution, we give initial discussion on the sidelink evolution and the observation and proposals are shown as below:

Proposal 1: To agree on 135 RBs for SL-U 60kHz 100MHz configuration and remove the “[]” of current version of TR 38.786.

Proposal 2: To agree on REFSENS requirement for SL-U with equation as below:

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | -89.2 + 10log10(NRB/105) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -89.4 + 10log10(NRB/50) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.6 + 10log10(NRB/24) |  |
| n96, n102 | 15 | 20, 40 | -88.7 + 10log10(NRB/105) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -88.9 + 10log10(NRB/50) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.1 + 10log10(NRB/24) |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the RB configuration for NR SL-U REFSENS as shown in table 7.1.1-2 |  |  |  |  |

Proposal 3: It is proposed to re-use the same ΔRIB,4R as NR-U to SL-U.

Proposal 4: FRC for SL-U to be updated based on NR SL FRC with larger channel bandwidth.

Proposal 5: To reuse current NR SL FRC for 15kHz SCS for QPSK, 64QAM and 256QA.

Proposal 6: It is proposed the SL-U FRC as below with same FRC below 40MHz and adding 60, 80 and 100MHz CBW FRC.

Table A.7.2-1: Fixed reference channel for V2X receiver requirements (SCS 15 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 53 | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Subchannel size |  | 12 | 10 | 15 | 10 | 12 |
| Allocated resource blocks |  | 24 | 50 | 105 | 160 | 216 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK |
| Transport Block Size |  | 1608 | 3624 | 7936 | 12296 | 16896 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 1 | 1 | 2 | 3 |
| Beta offset for 2nd stage SCI |  | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 5160 | 12036 | 26556 | 41076 | 55860 |
| Max. Throughput averaged over 100ms | Mbps | 0.1608 | 0.3624 | 0.7936 | 1.2296 | 1.6896 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. NOTE 3: The CBW is only applicable for PS UE in n14. |  |  |  |  |  |  |

Table A.7.2-2: Fixed reference channel for V2X receiver requirements (SCS 30 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| Subchannel size |  | 12 | 10 | 15 | 15 | 10 | 12 | 15 |
| Allocated resource blocks |  | 24 | 50 | 75 | 105 | 160 | 216 | 270 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Transport Block Size |  | 1608 | 3624 | 5632 | 7936 | 12296 | 16392 | 21000 |
| Transport block CRC | Bits | 16 | 16 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 1 | 1 | 1 | 2 | 2 | 3 |
| Beta offset for 2nd stage SCI |  | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 5160 | 12036 | 18636 | 26556 | 41076 | 55860 | 70116 |
| Max. Throughput averaged over 100ms | Mbps | 0.3216 | 0.7248 | 1.1264 | 1.5872 | 2.4592 | 3.2784 | 4.2 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

Table A.7.2-3: Fixed reference channel for V2X receiver requirements (SCS 60 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 | 60 | 60 | 60 |
| Subchannel size |  | 10 | 12 | 12 | 10 | 15 | 15 | 15 |
| Allocated resource blocks |  | 10 | 24 | 36 | 50 | 75 | 105 | 135 |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Transport Block Size |  | 456 | 1608 | 2536 | 3624 | 5504 | 7936 | 10248 |
| Transport block CRC | Bits | 16 | 16 | 16 | 16 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| Beta offset for 2nd stage SCI |  | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 | 2.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 7 | 7 | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 1464 | 5160 | 8328 | 12036 | 18636 | 26556 | 34476 |
| Max. Throughput averaged over 100ms | Mbps | 0.1824 | 0.6432 | 1.0144 | 1.4496 | 2.2016 | 3.1744 | 4.0992 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

Table A.7.3-1: Fixed reference channel for V2X receiver requirements (SCS 15 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 53 | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Subchannel size |  | 12 | 10 | 15 | 10 | 12 |
| Allocated resource blocks |  | 24 | 50 | 105 | 160 | 216 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Transport Block Size |  | 11528 | 27144 | 60456 | 92200 | 127080 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 2 | 4 | 8 | 11 | 16 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  | 15336 | 35964 | 79524 | 123084 | 167436 |
| Max. Throughput averaged over 100ms | Mbps | 1.1528 | 2.7144 | 6.0456 | 9.22 | 12.708 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. NOTE 3: The CBW is only applicable for PS UE in n14. |  |  |  |  |  |  |

Table A.7.3-2: Fixed reference channel for V2X receiver requirements (SCS 30 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| Subchannel size |  | 12 | 10 | 15 | 15 | 10 | 12 | 15 |
| Allocated resource blocks |  | 24 | 50 | 75 | 105 | 160 | 216 | 270 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Transport Block Size |  | 11528 | 27144 | 42016 | 60456 | 92200 | 125016 | 155776 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 2 | 4 | 5 | 8 | 11 | 15 | 19 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 1 | 1 | 1 |  |  |  |
| Binary Channel Bits per Slot |  | 15336 | 35964 | 55764 | 79524 | 123084 | 167436 | 210204 |
| Max. Throughput averaged over 100ms | Mbps | 2.3056 | 5.4288 | 8.4032 | 12.091 | 18.44 | 25.0032 | 31.1552 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

TableA.7.3-3: Fixed reference channel for V2X receiver requirements (SCS 60 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 | 60 | 60 | 60 |
| Subchannel size |  | 10 | 12 | 12 | 10 | 15 | 15 | 15 |
| Allocated resource blocks |  | 10 | 24 | 36 | 50 | 75 | 105 | 135 |
| MCS Index |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |  |  |  |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Transport Block Size |  | 3240 | 11528 | 18960 | 27144 | 42016 | 59432 | 77896 |
| Transport block CRC | Bits | 16 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 2 | 3 | 4 | 5 | 8 | 10 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 7 | 7 | 7 | 1 |  |  |  |
| Binary Channel Bits per Slot |  | 4248 | 15336 | 24840 | 35964 | 55764 | 79524 | 103284 |
| Max. Throughput averaged over 100ms | Mbps | 1.296 | 4.6112 | 7.584 | 10.858 | 16.8064 | 23.7728 | 31.1584 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

Table A.7.4-1: Fixed reference channel for V2X receiver requirements (SCS 15 kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 53 | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 |
| Subchannel size |  | 12 | 10 | 15 | 10 | 12 |
| Allocated resource blocks |  | 24 | 50 | 105 | 160 | 216 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |  |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Transport Block Size |  | 15880 | 36896 | 81976 | 127080 | 172176 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 2 | 5 | 10 | 16 | 21 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 3 | 3 | 3 | 3 | 3 |
| Binary Channel Bits per Slot |  | 20544 | 48000 | 106080 | 164160 | 223296 |
| Max. Throughput averaged over 100ms | Mbps | 1.588 | 3.6896 | 8.1976 | 12.708 | 17.218 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. NOTE 3: The CBW is only applicable for PS UE in n14. |  |  |  |  |  |  |

Table A.7.4-2: Fixed reference channel for V2X receiver requirements (SCS 30 kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 | 30 | 30 | 30 |
| Subchannel size |  | 12 | 10 | 15 | 15 | 10 | 12 | 15 |
| Allocated resource blocks |  | 24 | 50 | 75 | 105 | 160 | 216 | 270 |
| MCS Index |  | 23 | 23 | 23 | 23 | 24 | 24 | 24 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |  |  |  |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Transport Block Size |  | 15880 | 36896 | 58384 | 81976 | 127080 | 172176 | 217128 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 2 | 5 | 7 | 10 | 16 | 21 | 26 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 3 | 3 | 3 | 3 |  |  |  |
| Binary Channel Bits per Slot |  | 20544 | 48000 | 74400 | 106080 | 164160 | 223296 | 280320 |
| Max. Throughput averaged over 100ms | Mbps | 3.176 | 7.3792 | 11.677 | 16.395 | 25.416 | 34.4352 | 43.4256 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

Table A.7.4-3: Fixed reference channel for V2X receiver requirements (SCS 60kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 | 60 | 80 | 100 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 | 60 | 60 | 60 |
| Subchannel size |  | 10 | 12 | 12 | 10 | 15 | 15 | 15 |
| Allocated resource blocks |  | 10 | 24 | 36 | 50 | 75 | 105 | 135 |
| MCS Index |  | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |  |  |  |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Transport Block Size |  | 4480 | 15880 | 25608 | 36896 | 57376 | 81976 | 106576 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  | 1 | 2 | 4 | 5 | 7 | 10 | 13 |
| Beta offset for 2nd stage SCI |  | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 | 6.25 |
| γ value when 2nd stage SCI rate match |  | 3 | 3 | 3 | 3 |  |  |  |
| Binary Channel Bits per Slot |  | 5760 | 20544 | 33216 | 48000 | 74400 | 106080 | 137760 |
| Max. Throughput averaged over 100ms | Mbps | 1.792 | 6.352 | 10.243 | 14.758 | 22.9504 | 32.7904 | 42.6304 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 2: γ is the number of vacant resource elements in the resource block to which the last coded symbol of the 2nd-stage SCI belongs. |  |  |  |  |  |  |  |  |

4 References

[1] R4-2310309		on the SL-e RX requirement, OPPO