3GPP TSG-RAN WG4 Meeting #108							R4-2313001

Toulouse, France, August 21 – August 25, 2023

Agenda Item:	9.5.3

Source: 	OPPO

Title: 	on IOT NTN L/S band

Document for:	Approval

- Introduction

During the RAN4#107 meeting, the IOT NTN L/S band has been discussed and a WF [1] was agreed. Some of the open issues are left and hence in this paper, we try to give further discussion on it.

- Discussion

Firstly, the REFSENS requirement is left open and captured as below:

Issue 2-2-2: For REFSENS, what should be specified for band b254?

- Proposals

  - Option 1:

Table 2.4-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 254 | -102.2 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101. |  |  |

Table 2.4-2: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B in TS 36.102 | - 108.2 |

  - Option 2: Others, please elaborate.

For REFSENS requirement of cat M1, -102.2 dBm is proposed for band 254 which is the same as current requirement for band 256. From the companion WID as NTN L/S band, it has been proposed that to use the same REFSEN requirement of band n256 to band n254 as to consider the 0.5dB insertion loss. Hence here the -102.2 dBm requirement has aligned such proposals between two WIDs.

For the REFSENS for cat NB1 and NB2, currently the requirement is band agnostic and hence the requirement can be used for band 254, too.

Proposal 1: It is proposed the following REFSENS requirement as:

Table 2.4-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 254 | -102.2 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101. |  |  |

Table 2.4-2: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B in TS 36.102 | - 108.2 |

Secondly, the out-of-band blocking is also discussed in the last RAN4 meeting. The out-of-band blocking requirement for band 256 has been modified with specific ranges and a Note is added as shown below:

I Table 7.6A.3-2: Out of-band blocking for category M1 UE

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| 255 | Finterferer (C`W) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| 2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band 256 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

This is to allow band 256 to reuse band n65 duplexer so that larger out-of-band blocking ranges are introduced. For band 254, it seems that the duplexer issue does not exist anymore. Hence the mechanism of Note 1 as captured above does not apply to band 254. Similar requirement and ranges of band 255 can be re-used for band 254.

Proposal 2: For out-of-band requirement, similar requirement of band 255 can be reused for band 254.

3	Conclusions

In this contribution, we give initial discussion on the IOT NTN and the observation and proposals are shown as below:

Proposal 1: It is proposed the following REFSENS requirement as:

Table 2.4-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 254 | -102.2 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101. |  |  |

Table 2.4-2: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B in TS 36.102 | - 108.2 |

Proposal 2: For out-of-band requirement, similar requirement of band 255 can be reused for band 254.

4 References

[1] R4-2310297		WF on IoT_NTN_FDD_LS_band