3GPP TSG-RAN WG1 Meeting #108	R4-2313014

Toulouse, France, August 21st – 25th 2023

Agenda Item:	8.19.2.2.2

Source:	Ericsson

Title:	TP to TR 38.858 section 10.4 Feasibility of FR2 BS aspects

Document for:	Approval

# 1	Introduction

During RAN4#106, a structure for the RAN4 chapter of the SBFD TR was agreed, along with editors for each of the sections. It was also agreed that each company can submit TP for each section, and then that the TP for each section will be merged.

At RAN4#106bis-e, it was further agreed that for FR1 WA, MR and FR2 each company would capture results in a company specific section.

In this TP, we present a proposal for the section 10.5 containing analysis for the FR2 BS section.

# 3	Text Proposal

### 10.5.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

### 10.5.1.1	Summary table for self-interference analysis

Editor's note: This section captures the summary table which is based on self-interference analysis framework.

### 10.5.1.2	Feasibility study on self-interference

Editor's note: This section captures the feasibility study on self-interference based on individual companies’ analysis.

### 10.5.1.2.1	Ericsson

Editor's note: Individual company may provide the analysis assumption/configuration used for the corresponding analysis summarized in 10.5.1.1. Additionally, the views on the preference/views on component technology and corresponding trade-off can be provided and analysed.

Table 10.5.1.2.1-1 presents an overview of the self-interference mitigation potential for FR2-1 (28GHz) for three output power levels. It can be seen that mitigation of self-interference with less than 1dB receiver desensitization appears feasible for 30dBm. With 35dBm, the desensitization is around 1.5dB from self-interference. There may be some potential to further refine the performance. For 40dBm output power, mitigation of self-interference becomes significantly more challenging.

Table 10.5.1.2.1-1 FR2-1 Self-interference analysis

| FR2-1 |  |  |  | Ericsson |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | 40 dBm TRP | 35 dBm TRP | 30 dBm TRP |
| BS TX Power dBPTX = ① dBm |  |  |  | 40 dBm | 35 dBm | 30 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc | 28 dBc | 28 dBc |
|  |  | Frequency isolation techniques used |  | Digital Filtering, CFR |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 dBc | 80 dBc | 80 dBc |
|  |  | Spatial isolation techniques used |  | A combination of spatial isolation, chokes, absorption, mushroom EBG. |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 dBc | 10 dBc | 10 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Up to 5dB EIRP loss, depending on beam direction |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -78 dBm | -83 dBm | -88 dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | 0 dBc | 0 dBc | 0 dBc |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -50 dBm | -55 dBm | -60 dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  |  |  |  |
|  |  | Frequency isolation techniques |  |  |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -35 dBm | -35 dBm | -35 dBm |
|  |  |  | Rx IM3 contribution (dBm) | -80 dBm | -95 dBm | -110 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Reciprocal phase noise mixing will add noise at around -95dBm. | Reciprocal phase noise mixing will add noise at around -100dBm | Reciprocal phase noise mixing will add noise at around -105dBm |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -80 dBm | -94 dBm | -104 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 10 dBc | 10 dBc | 10 dBc |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 119 dBc | 125,5dBc | 127 dBc |
| Noise floor ⑩dBm |  |  |  | -87 dBm/CBW | -87 dBm/CBW | -87 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93 dBm | -93 dBm | -93 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 133 dBc | 128 dBc | 123 dBc |
| SBFD configuration |  |  |  | 75-50-75 |  |  |
| Guardband assumption (if exist) |  |  |  | 3 RB |  |  |
| bandwidth over which suppression is achieved |  |  |  | Several GHz |  |  |
| Others |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |

TX – RX isolation

A number of techniques exist to improve the TX-RX panel isolation including chokes, absorption, mushroom EBG etc. A detailed electromagnetic simulation of these techniques is presented in R4-2219633, which demonstrates that the isolation between a TX panel and RX sub-array is around 80dB. Unlike for FR1, the self-isolation does not seem to vary significantly with beam direction. Figure 10.5.1.2.1-1 shows EM simulation results and a visualization of the EM isolation.

| - boresight | - +15 deg. toward RX | - -15 deg. away from RX |
| --- | --- | --- |

| - boresight | - +15 deg. toward RX | - -15 deg. away from RX |
| --- | --- | --- |

Figure 10.5.1.2.1-1 EM simulation results for FR2 self-isolaiton (top) co-polarized (middle) cross polarized elements (bottom) visualization of the EM propagation

TX beam nulling

The transmit panel has a large number of transmit elements and hence a high number of degrees of freedom to perform beamforming. Beam Nulling can be used in the transmit panel to reduce the power at the receive panel. It is not clear that beam nulling has the same impact on both the transmitted signal and the transmitter leakage, however for simplicity this has been assumed.

Interference cancellation

Analogue and digital interference cancellation have not been considered due to the complexity and losses of building interconnectors for FR2, the high bandwidth and large amount of digital processing required.

Digital processing

Digital RX combining taking into account interference covariance have the potential to mitigate interference in the receiver. The gains from digital combining depend on the BS environment considering other interference sources such as other sectors, gNB and UEs.

Phase noise reciprocal mixing

Reciprocal mixing of phase noise causes a more significant amount of noise in the receiver in FR2-1 compared to FR2-1. However, the analysis suggests that also for FR2-1, reciprocal mixing of phase noise will not cause a significant degradation.

----------------------   Next section -----------------------------

### 10.5.2.1	Summary table for co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the summary table which is based on co-channel inter-sub-band co-site inter-sector interference analysis framework.

### 10.5.2.2	Feasibility study on co-channel inter-sub-band co-site inter-sector interference

Editor's note: This section captures the feasibility study on co-channel inter-sub-band co-site inter-sector interference based on individual companies’ analysis.

### 10.5.2.2.1	Ericsson

Editor's note: Individual company may provide the analysis assumption/configuration used for the corresponding analysis summarized in 10.3.2.1. Additionally, the views on the preference/views on component technology and corresponding trade-off can be provided and analysed.

Table 10.5.2.2.1-1 presents an analysis of the inter-sector interference effects for FR2-1 BS. The analysis suggests that inter-sector interference can lead to substantial degradations. Apart from with the most optimal beam directions settings and 30dBm TRP, degradations due to inter-sector interference are substantial. Although TX beam nulling may be considered to reduce inter-sector interference, in excess of 30dB-60dB suppression (depending on output power) would be needed to mitigate interference for all beam directions.

For 30dBm TRP, RX processing or improvements in the TX interference suppression may improve the inter-sector performance. However, the degradation will not be reduced to well below 1dB (taking into account also degradation from self-interference will occur) for all beam directions.

Thus co-sectorization is likely to pose technical challenges for SBFD deployments in FR2.

Table 10.5.2.2.1-1 FR2-1 Inter-sector interference analysis

| FR2-1 |  |  |  | Ericsson |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  |  |  |  |  | 30dBm TRP |  | 35 dBm |  | 40 dBm |  |
| BS TX Power dBPTX = ① dBm |  |  |  |  |  |  |  | 30 dBm |  | 35 dBm |  | 40 dBm |  |
| Number of co-site co-channel sectors considered |  |  |  |  |  |  |  | 2 |  | 2 |  | 2 |  |
| Component capability and parameters |  | Frequency isolation at TX |  | Frequency isolation capability dBαCSSI-frequency, TX = ② dBc |  |  |  | 28 dBc |  | 28 dBc |  | 28 dBc |  |
|  |  |  |  | Frequency isolation techniques used |  |  |  | Digital Filtering, CFR |  | Digital Filtering, CFR |  | Digital Filtering, CFR |  |
|  | Spatial isolation | Co-channel Co-site Inter-sector Spatial isolation capability dBαCSSI-spatial = ③ dBc |  | 75-98 dBc (depending on beam steering directions) |  |  |
|  |  | Co-channel Co-site Inter-sector Spatial isolation techniques used |  | Typical site deployment with 400mm between sectors |  |  |
|  |  | TX Beam nulling /isolation of inter-sector interference in TX sub-band dBαCSSI-beam = ④ dBc |  |  |  |  |  | 0 dB |  | 0 dB |  | 0 dB |  |
|  |  | DL EIRP impact due to beam nulling in TX sub-band (considering all nulling for self- and inter-sector interference) |  |  |  |  |  | 0 dB |  | 0 dB |  | 0 dB |  |
|  |  | Interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPCSSI_leakage due to inter-sector interference (Note 1) |  |  |  |  |  | -93 dBm to -70 dBm (depending on beam steering directions) |  | -88 to -65 dBm (depending on beam steering directions) |  | -83 to -60dBm (depending on beam steering directions) |  |
|  |  | Interference signal in gNB TX subband, measured at the input of LNA (Note 1) due to inter-sector interference |  |  |  |  |  | -65 dBm to -42 dBm(depending on beam steering directions) |  | -60 to -37 dBm (depending on beam steering directions) |  | -55 to -32dBm (depending on beam steering directions) |  |
|  |  | Blocker Suppression at RX |  | Frequency isolation capability dBαCSSI-frequency, RX= ⑥ dBc |  |  |  | 0 dB |  | 0 dB |  | 0 dB |  |
|  |  |  |  | Frequency isolation techniques |  |  |  | 0 dB |  | 0 dB |  | 0 dB |  |
|  |  |  |  | RX IMD |  | Rx IIP3 capability (dBm) |  | -35 dBm |  | -35 dBm |  | -35 dBm |  |
|  |  |  |  |  |  | Rx IM3 contribution (dBm) |  | -109 to -56 dBm |  | -101.5 to -41 dBm |  | -91 to -26 dBm |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | RX phase noise is -105 dBm | RX phase noise is -100 dBm | RX phase noise is -95 dBm |
|  |  | Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized due to co-site inter-sector co-channel interference only dBPCSSI_Selectivity(Note 1, 2) |  |  |  |  |  | -103.5 to -56 dBm |  | -97.5 to -41 dBm |  | -89.5 to -26 dBm |  |
|  |  | RX Beam nulling /isolation in RX sub-band dBαCSSI-beam = ⑨ dBc |  |  |  |  |  | 0 dB |  | 0 dB |  | 0 dB |  |
|  |  | RX sensitivity degradation caused by RX beam nulling |  |  |  |  |  | 0 dB |  | 0 dB |  | 0 dB |  |
|  |  | Digital processing interference supression capability |  |  |  |  |  | 0 dB |  | 0 dB |  | 0 dB |  |
| Total interference in RX SB (dBm) (Note 2) |  |  |  |  |  |  |  | -93 to -56 dBm |  | -87.5 to -41 dBm |  | -82 to -26 dBm |  |
| Noise floor ⑩dBm |  |  |  |  |  |  |  | -87 dBm |  | -87 dBm |  | -87 dBm |  |
| Calculated Desensitization (dB) |  |  |  |  |  |  |  | 1 to 31 dB (depending on beam direction) |  | 2.8 to 46 dBm (Depending on beam direction) |  | 6 to 61 dB (Depending on beam direction) |  |
| SBFD configuration |  |  |  | 75-50-75 |  |  |
| Guardband assumption (if exist) |  |  |  | 3 RB |  |  |
| bandwidth over which suppression is achieved |  |  |  | Several GHz |  |  |
| Others |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. Note 4: The abbreviation CSSI refers to co-site co-channel inter-sector interference in this table |  |  |  |  |  |  |

Inter-sector isolation

Isolation between sectors has been simulated using electromagnetic simulations in R4-2301885 with an assumption of 400mm sector separation. The isolation varies to some degree with separation, but not to an extent that would change the overall results. For most practical site deployments, addition of materials between sectors is not likely to be feasible (and may reduce network performance). Figure 10.5.2.2.1-1 depicts the EM simulation setup.

Figure 10.5.2.2.1-1 Simulation setup for FR2 multi-sector EM modelling

Figure 10.5.2.2.1-2 depicts the achievable isolation. The left hand plot shows the isolation with azimuth steering and elevation in boresight and the right hand plot with elevation steering and azimuth on boresight. The insolation between sectors is highly dependent on the beam direction. Although an “average” isolation can be given, this would mask the fact that for certain beam directions isolation is good and for others it is not good. Since the beam direction depends on the physical positions of users, advanced co-ordination of beam directions may not be possible if other constraints such as capacity and latency are to be optimized.

| - Azimuth beam steering with elevation at boresight | - Downtilt elevation beam steering with azimuth at boresight |
| --- | --- |

Figure 10.5.2.2.1-2 FR2 antenna with 400 mm edge to edge coupling magnitude

Beam nulling

There may be some potential for beam nulling to mitigate interference between sectors. To achieve complete isolation between sectors, more than 30dB beam nulling would be needed.