3GPP TSG-RAN WG4 Meeting #108 	R4-2313055

Toulouse, France meeting, August 21st – 25th, 2023

Agenda Item: 8.10.2.1

Source: MediaTek inc.

Title: Discussion on BWP operation without BW restrictions – Option A

Document for: Discussion and decision

# Introduction

RAN plenary discussed the topic BWP operation without restrictions in Meeting RAN#100 and defined a new WI with objectives captured below [1]:

## Objective ### Objective Core part WI ### The work item includes following objectives: ### 1) Specify support for BM/RLM/BFD: - For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4).

Therefore, in this contribution paper, we provide analysis and discussion on the completeness of Option A (CSI-RS).

# Discussion on RRM requirements impact/workload in RAN4

RAN4 has discussed the completeness of Option A (CSI-RS) for several meetings and in the previous meeting R4#106-bis-e RAN4 reached the following agreements [2]:

# Topic #2: Impact of Option A #### Issue 2-1: Any clarification on FR2 CSI-RS based RLM/BFD/BM requirements for Option A <Agreement >: - Do not introduce further enhancements for CSI-RS based RLM/BFD/BM requirements for Option A #### Issue 2-2: Any clarification on existing timing requirements when CD-SSB is outside active BWP - Option 1: (Ericsson, Apple, Nokia, Huawei, CMCC, CATT, MediaTek) - No clarifications on existing timing requirements are needed. - Option 2: (vivo) - It is clarified in the spec that existing timing requirements for non-RedCap UE are applicable regardless of whether SSB is within active BWP or not. - FFS from which release the clarification is made. - Option 3: (Ericsson) - The condition to configure gaps to meet the existing UE transmission timing error requirements in clause 7.1 of TS 38.133, when the UE is performing BM/RLM/BFD based on option A, is NOT needed. - A possible compromise is to clarify in clause 7.1.2 of TS 38.133, that the availability of the SSB at the UE is for the purpose of acquiring the timing of the reference cell

According to previous RAN4 agreement and the guidance from the WID, it is clear that no requirement change is needed Option A. The only issue is whether any clarification is needed in TS38.133. In our view, the current Section 7.1.2 is already general enough to cover both cases when SSB is within and outside the active BWP. Therefore, we do not see a need for further clarification for Option A.

Observation 1: Current Section 7.1.2 is general enough to cover both cases when SSB is within and outside the active BWP.

- No further clarification on existing timing requirements in TS38.133 is needed for Option A.

# Summary

In this contribution, we have the following proposal:

Proposal 1: No further clarification on existing timing requirements in TS38.133 is needed for Option A.

# References

- RP-231486, “New WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Vodafone, vivo, Taipei, Taiwan, June 2023.

- R4-2310176, “WF on NR BWP operation without restriction”, vivo, Incheon, KR, May 2023.