3GPP TSG-RAN WG4 Meeting #108 	R4-2313056

Toulouse, France meeting, August 21st – 25th, 2023

Agenda Item: 8.10.2.2

Source: MediaTek inc.

Title: Discussion on BWP operation without BW restrictions – Option B-1-1

Document for: Discussion and decision

# Introduction

RAN plenary discussed the topic BWP operation without restrictions in Meeting RAN#100 and defined a new WI with objectives captured below [1]:

## Objective ### Objective Core part WI ### The work item includes following objectives for non-Redcap UE: 1) Specify support for BM/RLM/BFD: - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) 2) Specify the necessary requirements to support L3 intra-frequency measurements without gaps (RAN4) - For Option B-1-1 - Specify applicability of legacy intra-frequency L3 measurements without gap and corresponding UE capabilities, if needed. - Note: UEs capable of Option B-1-1 BWP operation can optionally support L3 intra-frequency measurements with gaps. The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

In previous meetings, RAN4 already made agreements on requirements for RLM/BFD/BM measurements. In this contribution paper, we provide discussion on how to specify support of L3 intra-frequency measurements based on SSB outside the active BWP without gaps and without interruptions.

# Discussion on RRM requirements

RAN4 has discussed the requirements applicability for Option B-1-1 in the previous meeting R4#107 RAN4 reached the following way forward [2]:

#### Issue 3-3: Clarification on existing timing requirements for supporting Option B-1-1 - Follow conclusion of issue 2-2. #### Issue 3-7: Whether to define additional requirements for supporting option B-1-1 <Agreement >: - Do not introduce new UE and gNB behaviour for option B-1-1 in RAN4 spec. It is up to UE and gNB implementation how option B-1-1 is supported. - Do not introduce a new section for RRM requirements for option B-1-1.

Regarding L3 intra-frequency measurements:

The existing RAN4 requirements specify that the UE can use measurement gaps or interruption to perform L3 intra-frequency measurements outside the active BWP. However, in the previous RAN plenary #100, a new objective was added to the revised WI to allow the UE to perform L3 intra-frequency measurements based SSB outside the active BWP. To our understanding, the existing requirements for L3 intra-frequency measurements can be reused and hence RAN4 needs to update the applicability.

- For UE capable of supporting Option B-1-1 capability, the requirements in section 9.2.5 for Intra-frequency measurements without measurement gaps are applicable with CD-SSB outside the active BWP with no changes required.

# Summary

In this contribution, we have the following proposal:

Proposal 1: the requirements in section 9.2.5 for Intrafrequency measurements without measurement gaps are applicable with CD-SSB outside the active BWP with no changes required.

# References

- RP-231486, “New WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Vodafone, vivo, Taipei, Taiwan, June 2023.

- R4-2310176, “WF on NR BWP operation without restriction”, vivo, Incheon, KR, May 2023.