3GPP TSG-RAN WG4 Meeting #108 	R4-2313057

Toulouse, France meeting, August 21st – 25th, 2023

Agenda Item: 8.10.2.3

Source: MediaTek inc.

Title: Discussion on BWP operation without BW restrictions – Option C

Document for: Discussion and decision

# Introduction

RAN plenary discussed the topic BWP operation without restrictions in Meeting RAN#99 and defined a new WI with objectives captured below [1]:

## Objective ### Objective Core part WI ### The work item includes following objectives for non-Redcap UE: 1) Specify support for BM/RLM/BFD: - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) 2) Specify the necessary requirements to support L3 intra-frequency measurements without gaps (RAN4) - For Option C - Specify requirements for L3 intra-frequency measurements based on existing RedCap NCD-SSB requirements. The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

Therefore, in this contribution paper, we provide analysis and discussion on the completeness of Option C (NCD-SSB) for L3 intra-frequency measurements.

# Discussion on RRM requirements

RAN4 has discussed the requirements applicability for Option C in the previous meeting R4#107 RAN4 reached the following way forward:

#### Issue 4-3: Clarification on existing timing requirements for supporting Option C <Agreement >: - The current SSB-based timing requirement can be also applied to non-RedCap UE with NCD-CCB support. - FFS whether/what additional clarification are needed.

Regarding L3 intra-frequency measurements:

The existing RAN4 requirements specify that the UE is not required to perform BM/RLM/BFD outside the active BWP. However, in the previous RAN4 meeting we agree that ‘the existing requirements for SSB-based RLM/BFD/BM for non-RedCap UE are applicable to option C’. Then, RAN4 discussed the requirements for L3 intra-frequency measurements. In addition, the existing RAN4 requirements specify that the UE is not allowed to use NCD-SSB to perform L3 intra-frequency measurements within the active BWP. Then, in the previous RAN plenary #100, a new objective was added to the revised WI to allow the UE to perform L3 intra-frequency measurements based SSB outside the active BWP, i.e. using NCD-SSB. Such scenario was discussed back in Rel-17 RedCap WI and it was agreed that existing UE requirements based on CD-SSB can be reused for 2Rx RedCap UE with NCD-SSB. Now, for the case of active BWP without CD-SSB but with the NCD-SSB is contained within the active BWP, the existing requirements in RAN4 for L3 intra-frequency measurements can be directly applied with no changes.

- For UE capable of supporting Option C capability (NCD-SSB), the requirements in section 9.2.5 for Intra-frequency measurements without measurement gaps are applicable with ND-SSB within the active BWP with no changes required to the existing requirements.

# Summary

In this contribution, we have the following proposal:

Proposal 1: For UE capable of supporting Option C capability (NCD-SSB), the requirements in section 9.2.5 for Intra-frequency measurements without measurement gaps are applicable with ND-SSB within the active BWP with no changes required to the existing requirements.

# References

- RP-231486, “New WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Vodafone, vivo, Taipei, Taiwan, June 2023.

- R4-2310176, “WF on NR BWP operation without restriction”, vivo, Incheon, KR, May 2023.