3GPP TSG-RAN WG4 Meeting #108 	R4-2313062

Toulouse, France meeting, August 21st – 25th, 2023

Agenda Item: 5.2.3

Source: MediaTek inc.

Title: NR_redcap-Core Discussion on Stationary requirements

Document for: Discussion and decision

# Introduction

For Rel-17 RedCap WI, in RAN4#102-e meeting, RAN4 has reached the following agreement [1]:

GTW agreement: UE is allowed to meet the requirements that are the most relaxed out of Rel-16 and Rel-17 requirements.

Then, RAN4 further discussed this issue specifically for the requirements of multiple relaxation of Rel-16 low mobility and not-at-cell-edge criteria, and Rel-17 stationary criteria and the following agreement was reached [2]:

Issue 2-1-1-1: Requirements for scenario 9 if scenario 9 is allowed - Proposals - Option 1: UE follows the most relaxed requirements between Rel-16 low mobility and not-at-cell-edge criteria, and Rel-17 stationary criteria (vivo Ericsson) Agreement (1st round): Option 1 is agreed

# Discussion multiple relaxation scenarios

For the scenario of UE fulfilling Rel-16 low mobility and not-at-cell edge criterion and Rel-17 stationary criteria, we observe the following:

- For the case: Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

  - The requirements for Rel-16 low mobility and not-at-cell edge criterion are more relaxed compared to stationary, which is 1 hour.

- For the case: Srxlev > SnonIntraSearchP or Squal > SnonIntraSearchQ:

  - The requirements for stationary criterion are more relaxed compared to low mobility and not-at-cell edge criterion, which is K2=240.

- RAN4 shall update the requirements in clauses ‘4.2B.2.10.8’ and ‘4.2B.2.11.8’ to reflect that for the case of 
(i) Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:  requirements to follow Rel-16 low mobility and not-at-cell edge criterion, which is 1 hour; while 
(ii) Srxlev > SnonIntraSearchP or Squal > SnonIntraSearchQ:  requirements to follow Rel-17 stationary criterion, which is K2=240.

# Summary

In this contribution, we have the following proposal:

Proposal 1: RAN4 shall update the requirements in clauses ‘4.2B.2.10.8’ and ‘4.2B.2.11.8’ to reflect that for the case of 
(i) Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:  requirements to follow Rel-16 low mobility and not-at-cell edge criterion, which is 1 hour; while 
(ii) Srxlev > SnonIntraSearchP or Squal > SnonIntraSearchQ:  requirements to follow Rel-17 stationary criterion, which is K2=240.

# References

- R4-2207105, “WF on eDRX and RRM measurement relaxations requirements for Redcap UE”, vivo, Electronic Meeting, March 2022.

- R4-2210596, “WF on eDRX and RRM measurement relaxations requirements for Redcap UE”, vivo, Electronic Meeting, May 2022.