3GPP TSG-RAN WG4 Meeting # 108 	R4-2313063

Toulouse, France, August 21st – 25th, 2023

Agenda Item: 10.2.3

Source: MediaTek Inc.

Title: NR_redcap-Core Discussion on incoming LS of Pre-MG for RedCap UE

Document for: Discussion

# Introduction

RAN4 was copied in an LS from RAN3 to RAN2 [1], in which, RAN3 is discussing the applicability of using Pre-configured measurement gaps (Pre-MG) for Rel-17 RedCap. Hence, in this contribution we provide our view on this LS.

# Discussion

During the core requirements discussion in Rel-17 RedCap, RAN4 discussed which features to include with RedCap then an LS was sent to RAN2 with summary of these supported features [2]:

RAN4 would like to thank RAN2 for the incoming LS in R2-2109218. RAN4 has discussed the RAN2 agreements on UE capabilities from a RRM perspective and further discussed the Rel-16 UE features and their applicability for RedCap in Rel-17. Based on the discussions, following conclusion is reached: Rel-15/Rel-16 features in TS 38.133; RedCap RRM requirements applicability in R17. Dual connectivity and carrier aggregation; Not applicable. 2-step RA; Applicable. NR measurements with autonomous gaps; Applicable. Rel-17 features in TS 38.133. SDT; Applicable. Note: RAN4 will not define any RRM requirements for RedCap UE for other release 16/release 17 features which are not listed above in release 17..

From RAN4 RRM point of view, it is clear from the LS that Pre-MG feature is not supported for Rel-17 RedCap UE in terms of RRM requirements. Therefore, RAN4 shall inform RAN3 that Rel-17 Pre-MG is not supported in RAN4 RRM requirements for Rel-17 RedCap. In addition, Rel-18 eRedCap doesn’t include the work on Pre-MG for RedCap UE [3]. Hence, RAN4 shall inform RAN3 that Pre-MG for RedCap is out of Rel-18 RedCap WI scope.

- RAN4 shall send an LS response to RAN3 to inform them that Rel-17 Pre-MG is not supported in RAN4 RRM requirements for Rel-17 RedCap, also, Pre-MG for Rel-18 eRedCap is out of Rel-18 RedCap WI scope.

# Summary

In this contribution, discussion on inter-RAT measurement without gaps is provided and we have the following proposals:

Proposal 1: RAN4 shall send an LS response to RAN3 to inform them that Rel-17 Pre-MG is not supported in RAN4 RRM requirements for Rel-17 RedCap, also, Pre-MG for Rel-18 eRedCap is out of Rel-18 RedCap WI scope.

# References

- R4-2311008, “LS on applicability of pre-configured measurement gaps for RedCap UE”, Qualcomm, May 2023, Incheon, Korea.

- R4-2206977, “Reply LS on UE capabilities for RedCap from RRM perspective”, Ericsson, March 2022, e-meeting.

- RP-223544, “Revised WID on Enhanced support of reduced capability NR devices”, Ericsson, e-meeting, Dec 2022.